3GPP TSG-SA3 Meeting #122																			S3-252158

Fukuoka, Japan, 19 – 23 April 2025

# Source:	Qualcomm Incorporated, Ericsson, Lenovo

# Title:	Discussion on issues in temporary ID based approaches and way forward proposal

# Document for:	Discussion

Agenda Item:	4.26

# 1	Decision/action requested

This contribution discusses issues in temporary ID based approaches and proposes a way forward proposal.

# 2	References

[1]	3GPP TR 33.713

# 3	Rationale

In KI#3 of TR 33.713 [1], it was concluded that a mechanism to protect AIoT device ID using temporary ID shall be supported. According to the solutions and the discussions from the last meeting, temporary ID-based protection mechanisms can be categorized into two approaches:

- Network-assigned temporary ID

- Derived temporary ID using device credential

This paper identifies issues in both network-assigned temporary ID and derived temporary ID using device credential, and evaluates how these issues can be addressed. Based on this analysis, we propose a way forward for a temporary ID-based ID protection mechanism.

## 3.1.	Network-assigned temporary ID

In this approach, a network entity (e.g., AIoTF) assigns a temporary ID for an AIoT device using the command procedure after inventory. This mechanism is similar to the GUTI allocation after Core Network (CN) paging in 5G. However, there are several challenges to support this mechanism in the AIoT system:

1. De-synchronization: Temporary IDs can be de-synchronized between the AIoT device and the network in various scenarios. For instance, the command response message containing the acknowledgement of a new temporary ID assignment can be lost during transmission.

Observation 1: Temporary ID can be de-synchronized between the AIoT device and the network.

2. Reliability: According to the Reply LS (S3-243813), RAN1 feedback indicated that 1) writing may not always be possible at all times, and 2) frequent or recurring writing to non-volatile memory (NVM) should be avoided. Their feedback suggests that the AIoT device may not be able to store a temporary ID due to insufficient energy, impacting the reliability of the operation/procedure.

Observation 2: AIoT devices may not reliably store temporary IDs in NVM every time they received them. There is a risk of de-synchronization if the AIoT device fails to store the temporary ID.

3. Mandatory command procedure: The command procedure always needs to be performed to support this mechanism, meaning privacy protection based on this mechanism cannot support the inventory-only procedure defined the clause 6.2.2 of TS 23.369.

Observation 3: The command procedure always needs to be performed, meaning the mechanism cannot support the inventory-only procedure described in TS 23.369.

4. Additional messaging overhead: For group inventory, the AIoTF needs to perform the command procedure with all responded AIoT devices to allocate a new temp ID, which would incur messaging overhead. This can be exploited by an attacker to overload AIoTF by sending group inventory requests repeatedly.

Observation 4: group inventory can further overload AIoTF to perform command procedures with all responded AIoT devices to the inventory request.

To address de-synchronization issue, potential solutions were discussed in SA3 #121: the network either uses the latest acknowledged temporary ID or uses permanent device ID for inventory request. However, this requires the AIoT device to store and accept previous temporary ID in addition to the latest one, or accept the permanent device ID. This compromises the desired privacy of the temporary ID.

Observation 5: Potential solutions to address de-synchronization issues compromise the desired privacy of the temporary ID.

In summary, network-assigned temporary ID solutions have issues with de-synchronization that cannot be resolved in a satisfiable manner. Eventually, de-synchronization can always lead to a state where the device becomes unreachable just because the privacy solution is not sufficiently stable. This is not a desired property for a device to be sold on a competitive market.

## 3.2.	Derived temporary ID using device credential

In this approach, a temporary ID is generated and verified independently by the AIoT device and network based on shared device credential.

The procedure for individual inventory is as follows:

1. AIoTF generates a temporary ID (T-ID) for an AIoT device, where T-ID = F(pre-shared device key, AIoT device ID, Noncenetwork).

2. AIoTF sends an Inventory Request with the generated T-ID and Noncenetwork.

3.	AIoT device checks if the received T-ID is matched with the expected one. If successful, the AIoT device generates a new T-ID’, where T-ID’ = F(pre-shared device key, AIoT device ID, Noncenetwork, Noncedevice).

4.	The AIoT device sends an Inventory Response with T-ID’, Noncenetwork, and Noncedevice. Note Noncenetwork may not be needed if there is already a correlation between the Inventory Request and the Inventory Response.

5.	The AIoTF checks if the received T-ID’ is matched with the expected one.

The procedure for group inventory is as follows:

- AIoTF sends an Inventory Request with network nonce.

- The AIoT device generates a T-ID, where T-ID = F(pre-shared device key, AIoT device ID, Noncenetwork).

- The AIoT device sends an Inventory Response with the T-ID and Noncenetwork.

- The AIoTF checks if the received T-ID is matched with the expected one.

This approach does not require the AIoT device to store/maintain any information for the inventory procedure (i.e., stateless). Thus, de-synchronization issue doesn’t exist with this mechanism.

Observation 5: No de-synchronization issue exists due to stateless operation.

This approach has the following challenges:

1. Temporary ID collision: There is a potential risk of temporary ID collision. However, this depends on the length of the temporary ID.

Observation 6: Depending on the length of the temporary ID, there is a potential risk of ID collision.

Potential temporary ID collision can be addressed by defining a sufficiently long temporary ID or using an additional inventory with a different Nonce in case of collision.

Observation 7: Potential temporary ID collision can be addressed by defining a sufficiently long temporary ID or generating a new temporary ID with a new network nonce.

The procedures for the derived temporary ID using device credential require that the AIoT device computes an expected T-ID for each paging/inventory request it receives. The question has been raised whether the AIoT device has enough energy for these frequent computations. However, energy consumption is not an issue in this case, since the AIoT device anyway does not use stored energy but instead backscattering of the energy of carrier wave. Since receiving a paging/inventory request only happens when a carrier wave is made available, there is also enough power to compute the expected T-ID. If there is not enough power for computing the expected T-ID, the AIoT device is not close enough to the source of the carrier wave.

Observation 8: Energy consumption for computation of the expected T-ID is not an issue, since the energy is received from the nearby reader.

Another relevant question is whether an attacker could capture an old temporary ID and replay it. If the AIoT device is in the vicinity, it would respond to this replayed temporary ID. However, such an attack requires an active attacker. Hence, capturing an old temporary ID is much less valuable for an attacker than e.g. capturing a long-lived GUTI that can be exploited also in passive attacks. A privacy solution that works in practice and protects against passive attacks and makes active attacks much more difficult, is much preferable to not having a (working) privacy solution.

In addition, such an active attack is possible on regular 5G, so it is not introducing a threat that does not exist on 5G. The attack on regular 5G works replaying a previously used Authentication Vector (AV) and the attacked UE is recognised as the one that responds with a synchronisation failure as opposed to a MAC failure.

Observation 9: Replay attacks against derived temporary ID based solutions require an active attacker that can link old temporary IDs with specific AIoT devices.

## 3.3.	Summary and way forward proposal

As discussed in clause 3.1 and 3.2, the network-assigned temporary ID approach has inherent issues that cannot be fixed considering the AIoT device capability and use cases. On the other hand, the derived temporary ID approach has a temporary ID collision issue, but it can be mitigated. Therefore, it is proposed to use the derived temporary ID approach.

Proposal 1: It is proposed to use the derived temporary ID approach.

# 4	Detailed proposal

It is proposed that SA3 agrees with the derived temporary ID approach.