3GPP TSG-SA3 Meeting #122	S3-252161

Fukuoka, Japan, 19 - 23 May 2025

Source:	Qualcomm Incorporated

Title:	Proposed addition to general conclusion related to credential storage

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.8.0

Work Item:	FS_AIOT_Sec

Comments

This contribution proposes updates in general conclusion regarding the AIoT device credential storge on the network side. Particularly, it is proposed that AIoT device credential is stored in ADM for public network. In case of SNPN, AIoT device credential is stored in either ADM or credential holder. For network layer security, AIoTF retrieves the security key from the entity that stores the AIoT device credential.

Proposed Changes

* * * First Change * * * *

## 7.0	General conclusion

The study has reached conclusion on the following aspects, to be taken into account for the normative phase:

1.	For the protection of any new interfaces, CN or backhaul, existing mechanisms (i.e., NDS/IP from TS 33.310 [12] and SBA security from TS 33.501 [5]) are to be reused to the extent possible.

2.	The protection of Ambient IoT Services between the AIoT device and the network can be provided by mechanisms at the network layer, the application layer or a combination of both.

Editor’s Note: Further conclusions on how to accommodate application layer mechanisms or whether that would be left to deployments, out of scope, etc. are ffs.

3.	The network layer security solution to be specified includes the following security capabilities: authentication (one-way and mutual), protection of the AIoT data (confidentiality, integrity and replay protection) and identifier privacy over the air interface.

NOTE: The exact security capabilities to be supported by a device depend on the functionalities supported by the device. Which device functionality mandates which security capabilities is to be clarified during the normative work. Whether every device supports every possible device functionality or not is out of SA3 scope.

4.	For network layer security, the AIoTF acts as the security termination point on the network side.

Editor’s Note: Further conclusions are FFS.

5.  A dedicated network for AIoT service shall be needed if the existing authentication framework (e.g., 5G-AKA, EAP-AKA’, other EAP methods for SNPN) is not reused. To secure the operator's core network, security isolation mechanism between the AIoT service domain and operator domain shall be needed (e.g., operators may choose to deploy a security gateway).

NOTE: If multiple domains exist in the deployment of the architecture, the above policy applies.

6.	AIoT device credential is stored in ADM for public network. In case of SNPN, AIoT device credential is stored in either ADM or credential holder. For network layer security, AIoTF retrieves the security key from the entity that stores the AIoT device credential.

* * * End of Changes * * * *