# 3GPP TSG-SA3 Meeting #122																			S3-252179

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Qualcomm Incorporated

Title:	Pseudo-CR on 3GPP Cryptographic Inventory QUIC Details

Document for:	Approval

Agenda item:	5.20

Spec:	3GPP TR 33.938

Version:	0.2.0

Work Item:	FS_CryptoInv

Comments

This document provides suggested edits to clause 4.4.7 QUIC and MPQUIC.

The RFC 9001 clause 5.3 says “QUIC can use any of the cipher suites defined in [TLS13] with the exception of TLS_AES_128_CCM_8_SHA256.”, so the last sentence in 4.4.7 shouldn’t talk about the TLS clause 4.4.2 as TLS is profiled differently in 3GPP than in RFC 9001.

* * * First Change * * * *

### 4.4.7	QUIC and MPQUIC

The QUIC and MPQUIC are used in 5G system in standalone mode for the following:

- QUIC is used for the security of connection between UPF and AS proxy (see Clause 18 of TS 33.501 [4]).

- MPQUIC steering functionality is used for ATSSS (see Clause AA of TS 33.501 [4]).

For the QUIC establishment, the RFC 9001 [9] mandates the use of TLS with the exception of TLS_AES_128_CCM_8_SHA256.

* * * End of Changes * * * *