3GPP TSG-SA3 Meeting #122	S3-252192

Fukuoka, Japan 19 - 23 May 2025

# Source:	OPPO

# Title:	New Study on Security Aspects of Lower Layers

# Document for:	Discussion and Approval

# Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Security Aspects of Lower Layers

Acronym: FS_L1L2_Sec

Unique identifier:

Potential target Release: Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  |  |  |
| Don't know |  |  |  | X | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1040023 | NR_Mob_Ph4_Sec | SA3 work item |
| 1022091 | NR mobility enhancements Phase 4 | RAN2 work item |

# 3	Justification

In the R19 NR_Mob_Ph4_Sec to support security aspect of Layer1/Layer 2 Triggered Mobility (LTM), SA3 concluded that in Release 19, MAC CE message will not be protected after agreeing that “even though the unprotected MAC leads to increased attack scenarios and probability of handover failure.”

In R20, SA3 should continue finding a solution to address the vulnerabilities associated to unprotected MAC and LTM procedures.

Some of the threats and vulnerabilities captured during study phase of NR_Mob_Ph4_Sec have been captured, such as parameters (e.g., PCI, NCC) being sent during the unprotected LTM procedure that can be modified and cause LTM to fail.  Other parameters should also be looked at in detail, especially given that the introduction of AI technology in RAN will have an impact to other potential vulnerabilities.

AI/ML driven RAN enhancements will lead to more RAN and UE related parameters being collected. The availabilities of more RAN/UE data along with enhanced AI capabilities from adversaries, those parameters that were once considered not privacy-sensitive may become exposed to privacy risks, especially if they are combined in ways that can be used to derive privacy information.

Just to name a few:

− LCG ID (Logical Channel Group ID) is used for grouping logically similar service types for better management of wireless resources. An attack may use LCG ID to learn about the type of service a group of UE is using, their usage habits or preferences.

− C-RNTI is used to identify a UE within a cell. Attacker can monitor and analyze C-RNTI usage and potentially link its usage with other intercepted parameters to proximate the location of the UE.  If attacker can obtain C-RNTI usage of a UE among several RAN nodes, the attacker can get more precise location information of the UE.

− UE Contention Resolution ID is used to resolve contention during random access to ensure RAN can identify a UE correctly. However, if an attacker monitoring the use of this identifier during UE-RAN message exchange, the attacker may potentially track the UE’s access to the network. Additional information available to the attacker such as network topology and cell deployment location can further increase the potential to expose the location of the UE.

− TAG ID is used to identify a group of serving cells for synchronization between UE and cells that are potential HO targets of the UE. If attacker detects frequent updates of TAG ID for a particular UE, the attacker may deduce that the UE may be in a mobility situation.  Depending on the fluctuation of timing advance given to the UE and the ability of the attacker to detect that fluctuation, the attacker may gain some precision in estimating the location of the UE within the neighboring serving cells.

− TA command is used to inform the timing advance required to synchronize with service cells. Monitoring the change of TA values may lead to UE being tracked. For example, if TA for a UE remain static for a long period of time during the daytime, the UE may be in an office of if the TA remain static for a long period of time during evening or nighttime, attacker can deduce that the UE is at home.

These above examples are just scratching the surface of what is needed to be analyzed. Combination of the above examples with other additional unprotected parameters may prove more vulnerable to privacy exposure and attacks. Therefore, in addition to address the LTM use cases studied in R19, a comprehensive study of the entire MAC CE security review is prudent.

Another angle to the AI enhancements to RAN is the likelihood of increased efficiency of RAN procedure that could bring potential vulnerabilities to the AI management life cycle. For example, the interference stage may be tampered with by modifying the MAC CE by the attacker.

Furthermore, there may be other potential vulnerabilities on the lower layers that SA3 has not yet explored, for example, attackers can spoof scheduling information or CSI reporting information (e.g., RSRP) over the PUCCH and/or during PUCCH activation/deactivation], leading to tracking of UE, waste of UE resources and possibly DoS attacks. Such vulnerabilities are also part of the increase interest in the working group to study on protection at the lower layers. It is beneficial to 3GPP for SA3 to provide inputs to the RAN WGs, so that RAN WGs can incorporate security requirements/mechanisms at the lower layer(s).

Therefore, unfinished business in R19 to support LTM needs to be finished. SA3 needs stay ahead of the adversaries and start looking at impact to the security in lower layers due to the introduction of RAN AI.

# 4	Objective

The objective of this study is to address unfinished business in the security of LTM and study the impact to security of lower layer due to both LTM and introduction of RAN AI.

WT1: Define security procedure to support inter-CU LTM as defined in NR_Mob_Ph4_Sec

WT2: Study security of lower layer architecture and protocol, including security and privacy of unprotected lower layer parameters.

WT3: Define potential security requirements and solutions to address the vulnerabilities of lower layers.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT1 | 1 |  | Yes | No |
| WT2 | 2 |  | Maybe |  |
| WT3 | 1 |  |  |  |

Total TU estimates for the study phase: 4

Total TU estimates: 4

# 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 33.xyz | Study on Security Aspects to Lower Layers | TSG#110, Dec. 2025 | TSG#111, Mar. 2026 | TBD |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

SA2 for the Architecture aspects, SA5 for the Charging and OAM aspects.

RAN for radio interface protocol architecture and procedure aspects.

# 9	Supporting Individual Members

Supporting IM name

OPPO