# 3GPP TSG-SA3 Meeting #122																			S3-252194

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Ericsson

Title:	Conclusion update for KI#3: Security and privacy aspects of IMS DC capability exposure

Document for:	Approval

Agenda item:	5.2

Spec:	3GPP TR 33.790

Version:	1.3.0

Work Item:	FS_NG_RTC_SEC_Ph2

Comments

The conclusions for KI#3: Security and privacy aspects of IMS DC capability exposure, are incomplete.

This document proposes a way forward for concluding KI#3.

* * * First Change * * * *

## 7.3		Conclusions for Key Issue #3: Security and privacy aspects of IMS DC capability exposure

It is concluded to reuse authentication and authorization procedure specified in clause 12 of TS 33.501 [11].

TS 23.228 [7] and the TS 23.502 [16], specify in sufficient detail the IMS Subscribe/Notify Framework Architecture as well as the DC session control handling.

The normative work for KI#3 is proposed to be a new clause in TS 33.328 [17]. This clause will provide a summary of the new feature, a reference to the related clauses in TS 23.228 [7] and TS 23.502 [16] a list of security requirements, a statement that existing security specification in TS 33.501 [11], clause 12 for the NEF-AF interface can be re-used for NEF-AF authentication and authorization purposes.

Moreover, for the IMS event exposure procedures it can be stated in the normative work that the principles and level of description for AF authorization during Nnef_EventExposure_Subscribe and Nudm_EventExposure_Subscribe, operations defined for 5GC in clause 4.15.3.2.3 of TS 23.502 [16] are applicable for the Subscriber specific IMS event subscriptions.

There will be no normative work for the authorization for non-subscriber specific IMS event subscriptions and IMS DC session control.

* * * End of Changes * * * *