3GPP TSG-SA3 Meeting #122	S3-252232

Fukuoka, Japan, 17-21 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | DRAFT | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authorization for finer level service API access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3_sec |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The authorization information is updated to align with TS 23.222. # TS 23.222 Clause 8.11 API invoker obtaining authorization to access service API states, ‘The API invoker sends an obtain service API authorization request to the CAPIF core function for obtaining permission to access the service API by including the API invoker information (including its own identity and optionally application service information such as application service provider and application identifier), optionally service API operation(s) and resource(s), and any information required for authentication of the API invoker.’. # TS 23.222 Clause 8.31 states, ‘The API invoker sends an obtain service API authorization request to the CAPIF core function for obtaining permission to access the service API by including the API invoker information (including its own identity and optionally application service information such as application service provider and application identifier), optionally service API operation(s) and resource(s), and any information required for authentication of the API invoker.’ # TS 23.222 Annex E (normative): Configuration data for CAPIF > Authorization information related to the list of supported applications >> Application identifier... |  |  |  |  |  |  |  |  |
| Summary of change: |  | CAPIF-8 solution on finer level of granularity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No security solution for CAPIF-8 on finer level of granularity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******* START OF CHANGES

## 6.Y	Authorization for finer level service API access

The authorization function shall support finer level authorization as specified in TS 23.222 [3]. In both RNAA and non-RNAA scenarios, finer level service API access authorization can be used to limit access to specific services and service operations.

To enable finer level service API access, the authorization request or access token request includes the requested service and service operations at the respective granularity. In case of authorization request from API invoker to the CCF, it includes optionally application identifier as in TS 23.222 [3]. For RNAA, the request may also include the resource owner ID.

If the API invoker is authorized, the CCF responds to the API invoker with an access token including finer level authorization information, or an authorization code.

******* END OF CHANGES