3GPP TSG-SA3 Meeting #122	S3-252367

# Fukuoka, Japan, 19 – 23 May 2025

Source:	OPPO

Title:	Update AIOT Sol#24

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 37.713

Version:	0.8.0

Work Item:	FS_Ambient_IoT_Sec

Comments

It is proposed to update solution#24, and remove these following Editor’s Notes.

Remove the Editor’s Note: whether there is AIoT device external ID is based on SA2’s decision, which is FFS. And corresponding description about AIoT device external ID, since SA2 has not such conclusion.

Remove the Editor's Note:	How to synchronize and reallocate the temporary ID is FFS. A corresponding clause for temporary ID synchronization is added.

Remove the Editor's Note:	 More Evaluation is FFS. Since these ENs in the solution has been solved.

Proposed Changes

* * * First Change * * * *

## 6.24	Solution #24: temporary ID based AIoT device privacy protection

### 6.24.1	Introduction

This solution addresses the security requirement of KI#3: Privacy by protecting AIoT device identifiers. The purpose of the AIoT device temporary ID is to provide an unambiguous identification of the AIoT device that does not reveal the AIoT device permanent identity. The AIoT device temporary ID is allocated by the AIoTF while the security communication between AIoT device and AIoTF has been established.

### 6.24.2	Solution details

#### 6.14.2.1	Temporary ID allocation

This solution describes the allocation and usage of the AIoT device temporary identifier, as shown in the following figure 6.24.2-1.

Figure 6.24.2-1: signalling flow of the AIoT device temporary ID allocation and usage

- The AIoT device temporary ID is allocated by AIoTF after the security communication between AIoT device and AIoTF has been established (e.g., command procedures). Hence, the AIoT device temporary ID is protected by security contexts of the AIoT device, and transferred to the AIoT device.

For the first round of paging request and response between AIoT device and AIoTF, the AIoTF utilizes the initial temporary ID which could be pre-stored in the AIoT device and the AIoTF, to page the AIoT device. Hence, the plaintext of the AIoT device’s permanent ID is not transferred in the air interface.

The AIoT device stores the new allocated Temporary ID and removes the old temporary ID allocated before. The AIoT sends ACK to the AIOTF, the AIoTF deems the new temporary allocation successful if receives ACK from the AIoT device.

- The AF sends the AIoT Service Request to the AIoTF via the NEF, including the AIoT device ID.

- The AIoTF maps the received AIoT device ID to the AIoT device temporary ID, and use the AIoT device temporary ID to indicate the specific AIoT device. The AIoTF sends the AIoT Service Request (Inventory or Command) via the reader.

After the AIoT device temporary ID allocation, the AIoT device can be paged by its temporary ID, or partial of the temporary ID to enable more efficient radio signalling procedures.

- The AIoT device sends the AIoT Service Response to the AIoTF via the reader, including the AIoT device temporary ID.

- The AIoTF maps the AIoT device temporary ID to AIoT device ID to response the AF via the NEF.

#### 6.14.2.2 	Temporary ID synchronization

To issue the case that the temporary ID is de-synchronized between AIoT device and the AIoTF, the AIoTF should use the last successful allocated temporary ID to page the AIoT device.

### 6.24.3	Evaluation

This solution fully addresses security requirement in KI#3.

This solution requires the AIoT device to store the temporary ID allocated by the Network

NOTE: This solution is not fully evaluated.

* * * End of Change * * * *