3GPP TSG-SA3 Meeting #122	S3-252370

# Fukuoka, Japan 19 – 23 May 2025

Source:	OPPO

Title:	Remove ENs in Solution #32 and #33

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.8.0

Work Item:	FS_Ambient_IoT_Sec

Comments

According to the conclusions in Clause 8.2.2 in TR 23.700-13, SA2 agreed on that ADM (Ambient IoT Data Management) is used to store the subscription data (e.g. Device ID or credentials). Hence, this pCR is to remove below Editor's Note in Solution #32 and Solution #33.

Editor’s Note: Where is the security credential storage and key derivation in Network is FFS.

Other ENs are converted to NOTEs to state that they are not addressed in the solution;

NOTE 1: Whether DL frequency and UL frequency need to be the same is not addressed in the solution.

NOTE 2: L1 measurements in this solution is contingent and subject to RAN1 work, which is not addressed in the solution.

NOTE 3: Whether the key generation for AIoT devices in close proximity result in the same key is not addressed in the solution.

Proposed Changes

* * * Start of Change * * * *

6.32	Solution #32: Authentication Using L1 Parameter

6.32.1	Introduction

This solution addresses KI #5 and KI #4. It proposes an authentication solution using L1 parameters for “inventory and command case”.

The random number generation utilizing the reciprocity and randomness of wireless channels to generate shared keys includes three main stages: channel feature extraction, channel feature quantization and key negotiation [10].

Channel feature parameters are L1 measurements e.g., the reference signal received power (RSRP), channel state information (CSI), channel phase (CP) etc. The key generation process of the physical layer key mainly quantizes the measured wireless channel information into keys, and this process does not have high and complex computationally algorithms [11].

6.32.2	Solution details

Authentication solution for Inventory and Command Case using L1 measurement to generate a random number.

Figure 6.32.2-1 authentication solotion using L1 parameters for “inventory and command case”

1. AIoT NF triggers inventory. AIoT NF could be triggerd by AF.

2. Reader sends R2D Inventory paging message.

3， AIoT device extracts L1 measurements from Inventory paging message, and uses L1 measurements e.g. L1-RSRP to generate RAND_L1, a random number.

4. AIoT device sends inventory response, including authentication request message, and device ID.

Note that RAND_L1 is not needed to be sent over the air interface, because reader can generate the same random number.

5. Reader extracts L1 measurements from the inventory response message and uses L1 measurements e.g. L1-RSRP to generate the same RAND_L1 as AIoT device.

6. Reader sends authentication request message, including device ID and RAND_L1 to AIoT NF.

7. AIoT NF sends authentication request message, including device ID and RAND_L1 to AIoT Authentication Function.

8. AIoT authentication function sends authentication vector request to ADM/ARPF for AIoT device.

9. ADM generate a random number: RAND. It calculates Kiot with K (e.g. the root key of AIoT device), RAND_L1, Device ID and RAND.

10. ADM calculates MAC with K, RAND_L1, and RAND.

11. ADM sends RAND, MAC, device ID and Kiot to AIoT Authentication Function.

12. AIoT Authentication Function sends Authentication Response including RAND, MAC, device ID and Kiot to AIoT NF.

13. AIoT NF sends Authentication Response including RAND, MAC, device ID to Reader.

14. Reader sends Authentication Response including RAND and MAC to AIoT device in the command message.

15. AIoT device calculates XMAC with RAND, RAN_L1and K. It verifies XMAC=MAC.

16. AIoT device calculates Kiot with K, RAND_L1, Device ID and RAND.

NOTE 1: Whether DL frequency and UL frequency need to be the same is not addressed in the solution.

NOTE 2: How L1 measurements in this solution is aligned with the 3GPP L1 measurements is not addressed in the present document.

NOTE 3: Whether the key generation for AIoT devices in close proximity result in the same key is not addressed in the solution.

6.32.3	Evaluation

NOTE 4: This solution is not evaluated.

* * * Start of Next Change * * * *

6.33	Solution #33: L1 Security Key Generation

6.33.1	Introduction

This solution addresses KI#4. It proposes a L1 security key generation soluton for “inventory and command case”.

Physical layer key generation utilizing the reciprocity and randomness of wireless channels to generate shared keys includes three main stages: channel feature extraction, channel feature quantization and key negotiation [10].

Channel feature parameters are L1 measurements e.g., the reference signal received power (RSRP), channel state information (CSI), channel phase (CP) etc. The key generation process of the physical layer key mainly quantizes the measured wireless channel information into keys, and this process does not have high and complex computationally algorithms [11].

6.33.2	Solution details

Figure 6.33.2-1 Security Key Derivation using L1 measurements for “inventory and command case”

1. AIoT NF triggers inventory. AIoT NF could be triggerd by AF.

2. Reader sends R2D Inventory paging message.

3. AIoT device extracts L1 measurements from Inventory paging message, and uses L1 measurements e.g. L1-RSRP to generate K_L1, a physical layer key.

4. AIoT device sends inventory response, inclusing device ID.

Note that K_L1 is not needed to be sent over the air interface, because reader can generate the same key.

5. Reader extracts L1 measurements from the inventory response message and uses L1 measurements e.g. L1-RSRP to generate the same K_L1 as AIoT device.

6. Reader sends device ID and K_L1 to AIoT NF.

7. AIoT NF sends device ID and K_L1 to ADM.

8. ADM calculates Kiot with K, K_L1, and Device ID.

9. ADM sends device ID and Kiot to AIoT NF.

10. AIoT NF sends command message which is protected by Kiot.

11. Reader sends command message protected by Kiot to AIoT device.

12. AIoT device calculates Kiot with K, K_L1, Device ID.

13. AIoT device uses Kiot to decipher or verify command message, depends on the protection methods.

NOTE 1: Whether DL frequency and UL frequency need to be the same is not addressed in the solution.

NOTE 2: How L1 measurements in this solution is aligned with the 3GPP L1 measurements is not addressed in the present document.

NOTE 3: Whether the key generation for AIoT devices in close proximity result in the same key is not addressed in the solution.

* * * End of Change * * * *