3GPP TSG-SA3 Meeting #123	S3-252551

Goteborg, Sweden, 25 – 29 August 2025

# Source:	China Mobile, AT&T, Vodafone, T-Mobile US, NTT DOCOMO, BT, Verizon, Telecom Italia, Charter Communications, Deutsche Telekom, IIT Bombay, KDDI

# Title:	pCR to TS 33.502 - General requirements for Security related Events

# Document for:	Approval

Agenda Item:	5.1.1

# 1	Decision/action requested

Agree tdoc for incorporation into the draft TS 33.502

# 2	References

[1]	3GPP TS 33.502 v0.0.1

# 3	Rationale

Based on approved WID (SP-250872), the working task needs to specify the general requirements for security events handling and collection.

In WT2, it will specify the events that need to be reported including the following.

- 	The NF collects information on the SBA layer about malformed messages it receives that deviate from the 3GPP specified messages or are considered invalid according to the protocol specification and network state.

- 	The NF collects information about events involving receiving a massive number of incoming messages on the SBA layer.

- 	The NF collects information about failed authentication and authorization attempts from inbound connections on the SBA layer.

- 	The NF collects information about potential replay attacks on the SBA layer.

- 	The NF collects information about potential abnormal SBI call flows as defined for the communication models in Annex E of TS 23.501.

It is proposed to add general security requirements on security related events.

# 4	Detailed proposal

* * * First Change * * * *

# 5	Requirements on Security related events

Editor’s Note: This clause addresses the general requirements on configuration, collection and delivery of events.

## 5.x	General requirements

The NFs in the 5G system shall support the generation of security related events.

NFs in the 5G system shall contain a logically defined environment where security related events configuration, storage and processing is securely executed.

NOTE: Whether the environment is hardware, virtualization or container based is left to implementation.

The environment's integrity shall be assured.

Only authorised access shall be granted to the environment, i.e. to data stored and used within it, and to functions executed within it.

* * * End of Changes * * * *