3GPP TSG-SA3 Meeting #123	S3-252603

# Goteborg, Sweden, 25 – 29 August 2025

Source:	ZTE

Title:	Discussion on where to calculate a temporary ID

Document for:	Endorsement

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

To protect privacy of AIoT device identifiers by using temporary IDs, several options have been endorsed in S3-252326 during last meeting with some ENs left FFS. One common EN addresses that whether AIOTF or ADM computes and checks T-ID is FFS. We believe that the temporary ID should be generated and checked in AIOTF for the following reasons:

Reason 1: Generating temporary ID in ADM makes ADM complex and might deviate from the initial ideas on ADM.

As specified in TS 23.369, “The ADM supports the following functions:

-	Management of AIoT device profile data.

-	Management of AF authorization data.”

As a data management NE in the core network, the ADM is more suitable for storing and managing the generated temporary ID instead of actively generating it.

Reason 2: Generating temporary ID in AIOTF is aligned with AIOTF functions and the 3GPP architecture.

As specified in TS 23.369, “The AIOTF supports the following functions:

-	Termination of AIoT NAS protocol with AIoT Device.

...

-	Retrieving AIoT device profile data from ADM.

-	Retrieving AF subscription data from ADM.

-	Optionally AIoT Device context management.  ...”

This means that the AIOTF directly processes device access-layer signaling. In accordance with the 3GPP architecture design principle, temporary IDs in 3GPP standards are usually generated on access NFs rather than core network data management NFs. As a function of the access layer, the AIOTF complies with the design principle.

Therefore, based on the above reasons, the temporary ID should be generated and checked in the AIOTF but not ADM.