3GPP TSG-SA3 Meeting #123	S3-252753

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Samsung

Title:	Resolving EN on credential storage

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

According to LS from SA plenary (S3-252522), credential storage requirement in Annex I.2.2 of TS 33.501 can be applied.

* * * First Change * * * *

# 4	Overview of AIOT Security aspects

## 4.1	General

Editor’s Note: This clause contains the generic security principles, assumptions.

## 4.2	Security Requirements

### 4.2.1	Requirements on the device Type 1 in Topology 1

Editor’s Note: This clause contains the security requirement on the device, including secure storage and processing of credentials.

#### 4.2.1.1	Secure storage and processing of credentials

The requirements in this clause apply only to AIoT Devices where communications are triggered by the network. The long-term credentials used for authentication shall be securely stored and processed on the AIoT device.

The long term credentials shall be protected against cloning when stored or processed.

The long term credentials shall be confidentiality and integrity protected when stored and processed.

The long term credentials shall be protected against physical and logical attacks when stored and processed.

NOTE: How credentials are stored and processed within the AIoT device is out of scope of 3GPP.

* * * Next Change * * * *

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

0. Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed.

1. ADM shall generate RANDAIOT_n.

Editor’s Note: Whether ADM or AIOTF generates RANDAIOT_n is FFS.

2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN.

Editor’s Note: The inclusion of RANDAIOT_n in Paging Request and the size of RANDAIOT_n needs RAN confirmation.

3. NG-RAN shall send the paging request message including RANDAIOT_n to the AIoT device.

Editor’s Note: Whether replay attack is possible is FFS.

4. Upon receiving the paging request message, AIoT device shall derive RESAIOT using KAIoT and RANDAIOT_n for network authenticating AIoT Device.

Editor’s Note: How RESAIOT is derived and whether it is derived from KAIoT or intermediate key is FFS.

5. AIoT device sends D2R message to the NG-RAN, including RESAIOT and RANDAIOT_d from device.

Editor’s Note: The security requirements of generating RANDAIOT_d are FFS.

Editor’s Note: Whether RANDAIOT_d is required for inventory procedure is FFS.

6.  NG-RAN sends Inventory report message to AIOTF, including the RESAIOT and RANDAIOT_d.

7. AIOTF sends device identifier and RANDAIOT_d to ADM.

Editor’s note: The impact of interaction between AIOTF and ADM is FFS. If the authentication is expected to be run more often than normal UE, (e.g., during each inventory procedure), the analysis of load of ADM is FFS.

8. ADM derives XRESAIOT using the same method as in AIoT device.

9. ADM sends XRESAIOT to AIOTF.

10. AIOTF verifies RESAIOT. If the verification is successful, the steps 12-14 in clause 6.2.2 for inventory procedure or the step 8-11of clause 6.2.3 for command procedure in TS 23.369 [2] continues.

Editor’s note: How and where to derive keys is FFS.

Editor’s note: How to perform the mutual authentication for command procedure will be specified.

* * * End of Changes * * * *