3GPP TSG-SA3 Meeting #123	S3-252858

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Xiaomi, InterDigital

Title:	Key Issue on Security Aspects of Authorization for Sensing Service Invocation Revocation and Exposure

Document for:	Endorsement

Agenda item:	6.1.8

Spec:	3GPP TS/TR <TS/TR number>

Version:	<TS version>

Work Item:	<Work Item>

Comments

This paper proposes to add a new key issue for ISAC security study on security aspects of authorization for sensing service invocation, revocation and exposure.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X]	3GPP TR 23.700-14: "Study on Integrated Sensing and Communication; Stage 2".

…

* * * Second Change * * * *

5.X	Key Issue #X: Security aspects of authorization for sensing service invocation, revocation, exposure

5.X.1	Key issue details

In TR 23.700-14 [X], architecture for sensing services is studied to enable the 3GPP network to support sensing service invocation, revocation from the service consumer and exposure to the service consumer.

On the one hand, solutions addressing the KI#2 in TR 23.700-14 [X] of authorization and revocation for particular sensing services are developed, which focus on service request authorization or revocation based on the information of the service level agreement. Multiple editor’s notes are already captured in some of the solutions in TR 23.700-14 [X] for addressing security aspects of the proposed authorization procedures.

NOTE:	Security aspects of sensing service revocation triggered by sensing service consumer is addressed in this key issue.

On the other hand, KI#5 in TR 23.700-14 [X] addresses the type of sensing result to be exposed and the method for the network to expose the sensing result to the service consumer. Security aspect of the exposure procedure also needs to be investigated.

This key issue addresses the security aspects for sensing service innovation, revocation and exposure procedures between the network and sensing service consumer.

5.X.2	Security threats

If no authentication is performed between sensing service consumer and sensing related network function, an attacker can impersonate the sensing service consumer to send arbitrary sensing service invocation requests or revocation requests to access sensing services. Or an attacker can impersonate the network function responsible for exposure to send arbitrary sensing result to the service consumer.

If the connection between sensing service consumer and sensing related network function is not protected, the attacker can tamper or inject or sniff or replay the sensing service invocation/revocation/exposure messages.

In some solutions for authorization and revocation of sensing services proposed in TR 23.700-14 [X], there is discussion about user/consumer of the sensing service and owner of the sensing data. In case a sensing object is highly classified and owned by an authoritative owner (e.g. a highly classified building owned by a government authority), the sensing services involving such object without getting permission of the owner may result in the exposure of highly classified information.

Furthermore, when analysing all the solutions for sensing service authorization and revocation proposed in TR 23.700-14 [X], a potential risk of DDoS attack or amplification on network resource is identified as follows:

-	In some solutions, the NEF is used for authorizing sensing service requests by checking the request parameters against authorization information or service profile based on SLA. In this case, all the service requests incompliant with the SLA can be rejected by the NEF so that such requests will not be forwarded into the network to other sensing related network functions, e.g. sensing function.

-	In some other solutions, the sensing related function rather than the NEF is used for authorizing the sensing service request against authorization information or service profile based on SLA. In this case, some service requests incompliant with the SLA will be unnecessarily forwarded into the network by the NEF to the sensing function. These unnecessarily forwarded requests will consume the network resource in that the NEF has to unnecessarily interact with the NRF for sensing function discovery and unnecessarily interact with the sensing function for forwarding the requests, until the non-compliant requests are rejected by the sensing function. Such design may be exploited by an attacker to launch or amplify DDoS attack to exhaust network resource.

5.X.1	Potential security requirements

The 5G system shall be able to support mutual authentication between sensing service consumer and sensing related network function for sensing service invocation/revocation/exposure.

The 5G system shall be able to support integrity protection, confidentiality protection and anti-replay protection for the sensing service invocation/revocation/exposure messages exchanged between sensing service consumer and sensing related network function.

The 5G system shall be able to support authorization by object owner while supporting authorization of sensing service invocation and revocation.

The 5G system shall be able to mitigate DDoS amplification on network resource while supporting authorization of sensing service invocation and revocation.

* * * End of Changes * * * *