3GPP TSG-SA3 Meeting #123	S3-252866

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Xiaomi

Title:	Addressing ENs of replay attack

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

For the inventory and command case, the command request is followed by the inventory request/response. In Rel-19, only one round of command message is assumed to be exchanged between the AIOTF and AIoT device in one inventory and command procedure.

To protect the information in the command message, a key is generated after successful authentication. There are two options: the key is used for multiple inventory and command procedures, or the key is refreshed during each inventory and command procedure.

Only the option of refreshing the key during each inventory and command procedure can ensure the security of Rel-19 AIoT system. The following two reasons are considered:

- Parameters that are frequently updated, such as COUNT, cannot be maintained for AIoT device (type 1), which has limited storage resources and capacity.

- Without the freshness parameter, if the key can be used multiple times, the replay attack is possible. The attacker can replay the protected command request (e.g. resend the write command of round #1 during round #2 inventory and command procedure), which can result in wrong information being processed in the AIoT device.

Therefore, it is proposed to regenerate the key during each inventory and command procedure to address the replay attack. Security requirements of replay protection on the AIoT device and AIOTF, further clarification about the procedure assumption in clause 5.3.1, and the derivation function of KAIOTF are added to address the ENs of replay attack.

* * * First Change * * * *

4.2.1.3	Requirements for command protection

The AIoT device shall support confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF.

Confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF is optional to use.

The AIoT device shall support the following ciphering algorithms:

NEA0 and 128-NEA2 as specified in Annex D of TS 33.501 [5].

The AIoT device shall support integrity protection and replay protection of AIoT NAS messages between the AIoT device and the AIOTF.

Integrity protection and replay protection of AIoT NAS messages between the AIoT device and the AIOTF is mandatory to use.

The AIoT device shall support the following integrity algorithms:

128-NIA2 as specified in Annex D of TS 33.501 [5].

Editor’s Note: For this clause, the privacy and authentication related requirements are FFS.

* * * Second Change * * * *

4.2.2.2	Requirements on Communication Protection

The AIOTF shall support confidentiality protection of AIoT NAS Command request and response between the AIoT device and the AIOTF.

The AIOTF shall support the following ciphering algorithms:

NEA0, 128-NEA2 as defined in Annex D of the TS 33.501 [5].

Confidentiality protection of AIoT NAS Command request and response between the AIoT device and the AIOTF is optional to use.

The AIOTF shall support integrity protection and replay protection of AIoT NAS Command request and response between the AIoT device and the AIOTF.

The AIOTF shall support the following integrity algorithms:

128-NIA2 as defined in Annex D of the TS 33.501 [5].

Integrity protection and replay protection of AIoT NAS Command request and response between the AIoT device and the AIoTF is mandatory to use.

The AIOTF shall support selection of confidentiality and integrity algorithms for protecting AIoT NAS Command request and response between the AIoT device and the AIOTF based on operator’s local policy.

* * * Third Change * * * *

5.3.1	General

In each inventory and command procedure, only one command message can be sent by the AIOTF. This clause describes the security procedures for the information protection in command message. The protection of information is provided as part of the AIoT NAS protocol between AIoT device and AIOTF. The AIOTF acts as the security termination point for AIoT information protection.

* * * Fourth Change * * * *

5.3.3	Input parameters to integrity algorithm

The input parameters to the integrity algorithm as described in Annex D.3 in TS 33.501[5] shall be set as follows.

The KEY input is equal to the KAIOTF key.

The DIRECTION bit is set to 0 for uplink and 1 for downlink.

The BEARER is set to all zeros.

The COUNT is set to all zeros.

Editor’s Note: input key is FFS.

* * * Fifth Change * * * *

Annex X (normative): 
Key derivation functions

X.1	KAIOTF derivation function

When deriving a KAIOTF from KAIOT for protecting the information in the command message, the following parameters shall be used to form the input S to the  KDF:

-	FC =  TBD;

-	P0 = RANDAIOT_n;

-	L0 = length of RANDAIOT_n;

-	P1 = RANDAIOT_d;

-	L1 = length of RANDAIOT_d.

The input key KEY shall be the AIoT device root key KAIOT.

* * * End of Changes * * * *