3GPP TSG-SA3 Meeting #123	S3-252867

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Xiaomi

Title:	Update to AIOT security aspects based on SA guidance

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

Editor’s note: These requirements may need to be revisited at the time of addressing the solutions.

Editor’s note: Further requirements are FFS

Editor’s note: Solution is FFS

These ENs can be addressed by the endorsed SP-250851, which provides the following guidance on the Rel-19 AIoT system.

1) 	for Rel-19, the AIoT system is defined as private network (isolated network deployment that does not interact with a public network) e.g. SNPN, and the AIoT device credentials storage follows 3GPP defined requirements, the exact mechanism is out of scope of 3GPP (similar to Annex I.2.2 of TS 33.501). This bullet means that no interconnection exists between AIoT systems and PLMNs ;

2) 	For Rel-20, if the AIoT system is defined as public network i.e. PLMN, the AIoT device credentials storage shall use UICC.

NOTE 1: Revisiting the above requirements in bullet 1 and/or 2 can be anticipated in future releases following the normal working procedures including in Rel-20.

NOTE 2: In case UICC is used, the exact form factor and whether it is removable, non-removable or integrated is out of scope of 3GPP.

Editor’s Note: Further requirements are FFS.

Editor’s Note:	Requirements on privacy are ffs.

For the device ID privacy, the security requirements and potential solutions are discussed and captured in S3-252326. By reviewing the characteristics of Rel-19 AIoT system, several observations are made:

- For Rel-19, the AIoT system are deployed in the indoor environment. Based on the performance service requirements specified in TS 22.369, the communication range for inventory is expected to be 30m – 50m indoor. For a private network in an indoor environment, it is not clear against whom the privacy of AIoT device identifier is protected. If the attacker from the outdoor cannot get the access license, it is difficult to send/receive the inventory request/response to launch the attack.

- Mobility is not supported in Rel-19 AIoT system. The attack for tracking the trajectory of AIoT device does not need to be considered.

- Frequent or recurring writing to NVM of device type 1 should be avoided, and writing data into device type 1 may not always be possible, based on the RAN1’s instruction. Therefore, the frequent refresh of the stored Temp ID may not be supported from the device capability perspective.

- The device type 1 has limited energy. If the AIoT device keeps calculating the Temp ID to determine whether it is the one being paged, the AIoT device may quickly run out of power.

Therefore, it is proposed that the privacy protection mechanisms are optional to use to resolve these ENs. Depending on the deployment environment and the service running on the AIoT device, the owner can decide whether to use the privacy protection mechanism, which is up to the implementation of private network.

* * * First Change * * * *

4	Overview of AIOT Security aspects

4.1	General

Editor’s Note: This clause contains the generic security principles, assumptions.

This clause describes the security requirements for Ambient IoT services.  For AIoT system deployed in the private network, these security requirements are satisfied.

* * * Second Change * * * *

4.2.1.1	Secure storage and processing of credentials

The requirements in this clause apply only to AIoT Devices where communications are triggered by the network. The long-term credentials used for authentication shall be securely stored and processed on the AIoT device.

The long term credentials shall be protected against cloning when stored or processed.

The long term credentials shall be confidentiality and integrity protected when stored and processed.

The long term credentials shall be protected against physical and logical attacks when stored and processed.

* * *Third Change * * * *

4.2.1.4	Requirements for identifier privacy

-	The mechanism for protecting identifier privacy is optional to use.

* * *Fourth Change * * * *

4.2.2.3	Requirements on Privacy

-	The mechanism for protecting identifier privacy is optional to use.

* * * End of Changes * * * *