| 3GPP TR 33.786 V0.3.0 (2025-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on AIML Enablement Service Security (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Security Assumptions	8

5	Key Issues	8

5.1	Key Issue #1: Authorization for AIMLE Service Security for AIML members	8

5.1.1	Key issue details	8

5.1.2	Security threats	9

5.1.3	Potential security requirements	9

5.2	Key Issue #2: Secure AIMLE ML Model Access	9

5.2.1	Key Issue details	9

5.2.2	Security threats	9

5.2.3	Potential security requirements	9

5.X	Key Issue #X: <Key Issue Name>	10

5.X.1	Key Issue details	10

5.X.2	Security threats	10

5.X.3	Potential security requirements	10

6	Solutions	10

6.1	Solution #1: Authorization for AIMLE Services	10

6.1.1	Introduction	10

6.1.2	Solution details	10

6.1.3	Evaluation	16

6.2	Solution #2: Authorization of AIMLE clients acting as FL members for access to AIMLE Service Security	17

6.2.1	Introduction	17

6.2.2	Solution details	17

6.2.2.1	The procedure for AIMLE clients’ authorization	17

6.2.3	Evaluation	18

6.3	Solution #3: Re-using existing mechanisms	19

6.3.1	Introduction	19

6.3.2	Solution details	19

6.3.3	Evaluation	19

6.4	Solution #4: Authorization for Secure AIMLE based ML Model Access	19

6.4.1	Introduction	19

6.4.2	Solution details	19

6.4.3	Evaluation	21

6.5	Solution #5: FL member authorization for AIMLE services	21

6.5.1	Introduction	21

6.5.2	Solution details	22

6.5.3	Evaluation	22

6.Y	Solution #Y: <Solution Name>	22

6.Y.1	Introduction	22

6.Y.2	Solution details	22

6.Y.3	Evaluation	22

7	Conclusions	22

7.1	Key Issue #1: Authorization for AIMLE Service Security for AIML members	22

7.2	Key Issue #2: Secure AIMLE ML Model Access	23

Annex A: Change history	24

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document has the following objectives:

- Identify and study the authentication and authorization aspects for AIMLE services specified in TS 23.482 [3].

- Study the solutions to address the identified scenarios to support AIMLE service security.

NOTE 1: For the above objectives existing SEAL security aspects [2] need to be taken into account as SEAL architecture is used as baseline for the AIMLE architecture. As the AIMLE phase 2 study progress in SA6 [4], related progress can be taken into account when stable conclusion in SA6 is available if any security aspects need to be considered additionally for this security study.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.434, "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals".

[3]	3GPP TS 23.482, "Functional architecture and information flows for AIML Enablement Service".

[4]	3GPP TR 23.700-83, "Study on application layer support for AI/ML services Phase 2".

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Security Assumptions

The AIMLE security should use authorization aspects specified in TS 33.434 [2] as baseline.

The AIMLE server is deployed as a SEAL server, hence SEAL architecture is enhanced to incorporate the AIMLE service as shown below, where Figure 4-1 illustrates the service-based representation including AIMLE server as part of the SEAL framework as described in TS 23.482 Clause 5.2.1.1 [3].

Figure 4-1: SEAL functional model representation using service-based interfaces and including AIMLE function

# 5	Key Issues

Editor’s Note: This clause contains all the key issues identified during the study.

## 5.1	Key Issue #1: Authorization for AIMLE Service Security for AIML members

### 5.1.1	Key issue details

3GPP TS 23.482 [3] introduces support for AIMLE services, enabling AI/ML operations through interactions between the AIMLE client and AIMLE server(s) over the AIML-UU reference point, and between the VAL servers and AIMLE servers over AIML-S respectively. These services involve distributed AI/ML operations across multiple participants, necessitating robust security mechanisms to ensure that only authorized members participate in the AIMLE workflows. Further, TR 23.700-83 [4] describes AIMLE based MI model inference and AI Inference exposure services. Given the critical role of authorization in securing these workflows, it is important to assess whether the current security specifications are adequate.

Currently, the authorization aspects outlined in TS 33.434 [2] can be limited to address the security requirements of AIMLE services and related aspects specified in TS 23.482 [3] such as related to a) Federated Learning (FL), b) client related handling (registration, discovery, selection, selection subscription, and participation), c) transfers (task transfer, transfer learning, context transfer) d) ML Model (training capability evaluation, monitoring and control), e) Split operations and AIMLE assistance respectively. Therefore, this key issue aims to study whether enhancements to the authorization mechanisms specified in 3GPP TS 33.434 [2] are necessary to support AIMLE service security. The objective is to ensure trusted AIMLE members participation and usage to prevent unauthorized access of AIMLE operations.

Editor’s Note 1: Rel-20 related security aspects need to consider and align with the conclusions in TR 23.700-83 [4] is FFS.

### 5.1.2	Security threats

Unauthorized AIMLE members (e.g., FL members) participating in AIMLE services may gain access to data exchanged between AIMLE clients and servers.

Lack of robust authorization allows unreliable or unauthorized AIMLE members (e.g., FL members) to degrade the quality, efficiency, or availability of AIMLE operations.

### 5.1.3	Potential security requirements

The 3GPP system shall support authorization mechanisms for AIML members (e.g., FL members) utilising AIMLE services for various AIMLE procedures.

## 5.2	Key Issue #2: Secure AIMLE ML Model Access

### 5.2.1	Key Issue details

TS 23.482 [3] describes AIMLE services which supports ML Model retrieval, ML model training, ML model management (model information storage and discovery) ML model update, and ML model selection aspects. AIMLE Services uses SEAL as the fundamental architecture and the authorization aspects of SEAL Security in TS 33.434 [2] which allows requested service specific authorization which can be limited and necessary controls can be in place for the different ML access and management work flow authorization for the overall AIMLE based ML access security.

### 5.2.2	Security threats

Unauthorized AIMLE client(s)/ VAL server using AIMLE services may gain access to ML model data leading to leakage of model.

Lack of robust authorization allows unauthorized AIMLE client(s) or VAL servers to degrade the quality, efficiency, or availability of AIMLE operations.

### 5.2.3	Potential security requirements

The 3GPP system shall support authorization to secure AIMLE service-based ML Model operations such as retrieval, training, update, selection, and management (i.e., ML model information storage and discovery).

## 5.X	Key Issue #X: <Key Issue Name>

### 5.X.1	Key Issue details

### 5.X.2	Security threats

### 5.X.3	Potential security requirements

# 6	Solutions

Editor’s Note: This clause contains the proposed solutions addressing the identified key issues.

## 6.1	Solution #1: Authorization for AIMLE Services

### 6.1.1	Introduction

This solution address KI#1.

### 6.1.2	Solution details

AIMLE authorization related to AIML Services can reuse the authorization procedure specified in TS 33.434 [2] clause 5.2.2 (SEAL service authorization) and clause B.3.3 (SEAL service authorization) as the baseline where, SIM-S or AIMLE Server (with SIM capabilities) acts as an authorization server and issues access token to the AIMLE service consumer. The AIMLE service producer provides the requested services to the AIMLE service consumers by verifying the authorization of AIMLE service consumer i.e., on validating the access token claims as shown in Figure 6.1.2-1.

Figure 6.1.2-1: AIMLE Service Authorization

Step 1-3. The access token request, access token generation, response can be same as TS 33.434 [2] Clause B.3.7 Obtaining access token and B.3.6 Access token, with the adaptation that scope includes AIMLE service specific information.

Step 4-6. The AIMLE service Request/Response sent is same as each of request/response messages described in TS 23.482 [3] clause 8 related procedures with the following adaptations. i.e., The access token is sent in step 4 and on successful validation of AIMLE service specific information in the access token claims, the AIMLE service request is processed, and the response is provided.

The specific authorization related adaptations to AIMLE Service related procedures include the following:

- FL member registration:

- AIMLE Service: FLMemberRegistration Request/Response, FLMemberRegistration Update Request/Response, FLMemberRegistrationFetch Request/Response, FLMemberDeregistration Request/Response

- AIMLE Service Consumer: VAL Server, AIMLE Server

- AIMLE Service Producer: ML Repository

- Token Claims including scope: FL member ID/Requestor ID (i.e., AIMLE Service Consumer ID) as Subject, AIMLE service-related information as scope, FL member type (as Server or Client), FL member capabilities, Allowed ML Model ID list, FL member location information, Issuer as Authorization Server ID (i.e., SIM-S ID or AIMLE Server ID).

- FL related events subscription:

- AIMLE Service: FLEvents Subscribe/Notify

- AIMLE Service Consumer: VAL Server, AIMLE Server

- AIMLE Service Producer: ML Repository

- Token Claims including scope: FL member ID/Requestor ID as Subject, AIMLE service-related information as scope, FL member Type (Server or Client), Allowed FL member ID, Allowed FL related Events ID or name, Allowed ML Model ID list/ML Model Information for FL, Allowed notification target address, issuer as authorization server ID.

- HFL Training:

Process 1:

- AIMLE Service: MLModelTraining Request/Response

- AIMLE Service Consumer: VAL Server

- AIMLE Service Producer: AIMLE Server

- Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, AIML Model (e.g., Model ID/Type) and Model parameters, Dataset ID(s), Allowed FL members (Allowed List of member client IDs) to use as AI MLE clients for HFL (or) ML model training, Training Type (HFL/VFL/or both), Allowed AI MLE client selection/filtering criteria, Allowed ML Model ID list/ML Model Information for training, ML Model selection filtering criteria, issuer as authorization server ID

Process 2:

- AIMLE Service: HFLTraining Subscribe/Notify

- AIMLE Service Consumer: AIMLE Server

- AIMLE Service Producer: AIMLE Client

- VFL Training:

Process 1:

- AIMLE Service: MLModelTraining Request/Response

- AIMLE Service Consumer: VAL Server

  - AIMLE Service Producer: AIMLE Server

- Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, Allowed FL members (Allowed List of member client IDs) to use as AIMLE clients for VFL model training (e.g., per domain), Training Type (HFL/VFL/or both), Allowed AI MLE client selection/filtering criteria, Allowed ML Model ID list/ML Model Information for training, VFL Model selection filtering criteria, issuer as authorization server ID

Process 2:

- AIMLE Service: HFLTraining Subscribe/Notify

- AIMLE Service Consumer: AIMLE Server

- AIMLE Service Producer: AIMLE Client

- Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, Allowed FL members (Allowed List of member client IDs) to use as AI MLE clients for VFL model training (e.g., per domain), Training Type (HFL/VFL/or both), Allowed AI MLE client selection/filtering criteria, Allowed ML Model ID list/ML Model Information for training, VFL Model selection filtering criteria, issuer as authorization server ID

- FL member grouping:

Process 1:

- AIMLE Service: FLMemberGroupSupport Request/Response

- AIMLE Service Consumer: VAL Server

- AIMLE Service Producer: AIMLE Server

- Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, VAL service ID, AIML Model ID, ADAE Analytics ID, ML Model Profile Information (e.g., ID for which the FL grouping is to be used), ML Task Information/ID (e.g., FL Training task or FT Inference Task), Allowed FL members (Allowed List of member client IDs) to use as AI MLE clients/server for FL, issuer as authorization server ID

Process 2:

- AIMLE Service: FLGroupIndication Request/Response

- AIMLE Service Consumer: AIMLE Server

- AIMLE Service Producer: AIMLE Client

- Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, VAL service ID, AIML Model ID, ADAE Analytics ID, ML Model Profile Information (e.g., ID for which the FL grouping is to be used), ML Task Information/ID (e.g., FL Training task or FT Inference Task), Allowed FL members (Allowed List of member client IDs) to use as AI MLE clients/server for FL, issuer as authorization server ID.

- AIMLE Client Discovery:

- AIMLE Service: AIMLEClientDiscovery Request/Response

- AIMLE Service Consumer: VAL Server

- AIMLE Service Producer: AIMLE Server

- Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, Allowed maximum number of AIMLE clients, Allowed AIMLE Client discovery criteria such as List of allowed VAL service(IDs), Allowed service permission level usages (premium resource usage/standard resource usage/limited resource usage), Allowed ML model types (decision trees/linear regression/neutral networks/any model type), Allowed AIML operations/services (such as training, model transfer, model inference, model offload, model split), Allowed dataset requirements or handling, Allowed client location/Allowed location information for member client discovery/selection (Anywhere or by coordinates, civic addresses, network areas, or VAL service area ID), Allowed AIMLE Client task capabilities, issuer as authorization server ID.

- AIMLE Client Registration:

- AIMLE Service: AIMLEClientRegistration Request/Response, Update, Delete

- AIMLE Service Consumer: AIMLE Client

- AIMLE Service Producer: AIMLE Server

- Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, Allowed client profile(s), List of allowed VAL service(IDs) and allowed corresponding permission level(s), Allowed AIML operations/services (such as training, model transfer, model inference, model offload, model split), Allowed client location/Allowed location information for member client selection (Anywhere or by coordinates, civic addresses, network areas, or VAL service area ID), AIMLE Client capabilities, Allowed ML Model ID list/ML Model Information for AIMLE client usage, issuer as authorization server ID.

- AIMLE Client Selection:

- AIMLE Service: AIMLEClientSelection Request/Response

- AIMLE Service Consumer: VAL Server

- AIMLE Service Producer: AIMLE Server

- Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, Allowed maximum number AIMLE clients, List of allowed VAL service(IDs), Allowed AIMLE Client IDs, Allowed AIMLE client selection criteria i.e., [service permission level usages (premium resource usage/standard resource usage/limited resource usage), Allowed ML model types (decision trees/linear regression/neutral networks/any model type), Allowed AIML operations/services (such as training, model transfer, model inference, model offload, model split), Allowed dataset requirements or handling, Allowed client location/Allowed location information for member client discovery/selection, (Anywhere or by coordinates, civic addresses, network areas, or VAL service area ID), Allowed AIMLE Client task capabilities,], Allowed AIMLE Client Set ID(s), issuer as authorization server ID.

- AIML Client selection subscription and notification:

- AIMLE Service: AIMLEClientSelection Subscribe/Notify, Update, Unsubscribe

- AIMLE Service Consumer: VAL Server

- AIMLE Service Producer:

- Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, List of allowed VAL service(IDs), Allowed AIMLE client selection criteria/service requirements per VAL service ID i.e., [service permission level usages (premium resource usage/standard resource usage/limited resource usage), Allowed number of AIMLE Clients for selection, Allowed Notification endpoint for the selected AIMLE Client, issuer as authorization server ID.

- AIMLE Client Participation:

  - AIMLE Service: AIMLEClientParticipation Request/Response

  - AIMLE Service Consumer: AIMLE Server

  - AIMLE Service Producer: AIMLE Client

  - Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, List of allowed VAL service(IDs), Allowed AIMLE Client Set ID(s), Allowed AIMLE server ID(s), Allowed operation (Add/remove indicator), Allowed AIML model ID(s), Allowed AIML operations/services (such as training, model transfer, model inference, model offload, model split), Allowed AIMLE client selection criteria/service requirements per VAL service ID i.e., [service permission level usages (premium resource usage/standard resource usage/limited resource usage), Allowed dataset requirements or handling, issuer as authorization server ID.

- AIML Task Transfer:

- Type 1: AIMLE Service: (i) AIMLTaskTransferAssist Request/Response, (ii) AIMLESControlled AIMLTaskTransfer Request/Response

- AIMLE Service Consumer: AIMLE Client

- AIMLE Service Producer: AIMLE Server

- Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, List of allowed VAL service(IDs) and allowed corresponding permission level(s), Allowed AIML Task type or operations/services (such as training, model transfer, model inference, model offload, model split), Allowed ML Model ID list/ML Model Information for AIMLE client usage, Allowed Transfer mode (i.e., AIMLE server assisted/direct AIML task transfer/AIMLE server-controlled), Allowed time window for task transfer, Allowed AI/ML target members for task transfer, issuer as authorization server ID.

- Type 2: AIMLE Service: (i) AIMLTaskTransfer Request/Response, (ii) DirectAIMLTaskTransfer Request/Response

- AIMLE Service Consumer: (i)AIMLE Server (ii)AIMLE Client

- AIMLE Service Producer: AIMLE Client

- Token Claims including scope for (i): Requestor ID as Subject, AIMLE service-related information as scope, Allowed list of Source AI/ML Member ID(s), Allowed list of Target AI/Member ID(s), List of allowed VAL service(IDs) and allowed corresponding permission level(s), Allowed AIML Task type or operations/services (such as training, model transfer, model inference, model offload, model split), Allowed ML Model ID list/ML Model Information for AIMLE client usage, Allowed Transfer mode (i.e., AIMLE server assisted/direct AIML task transfer/AIMLE server-controlled), Allowed time window for task transfer, Allowed AI/ML target members for task transfer, issuer as authorization server ID.

- Token Claims including scope for (ii): Requestor ID as Subject, AIMLE service-related information as scope, List of allowed VAL service(IDs) and allowed corresponding permission level(s), Allowed AIML Task type or operations/services (such as training, model transfer, model inference, model offload, model split), Allowed ML Model ID list/ML Model Information for AIMLE client usage, Allowed Transfer mode (i.e., AIMLE server assisted/direct AIML task transfer/AIMLE server-controlled), Allowed time window for task transfer, Allowed AI/ML target members for task transfer, issuer as authorization server ID.

- AIMLE Context Transfer:

  - AIMLE Service: ContextTransfer Request/Response

  - AIMLE Service Consumer: AIMLE Server (e.g., S-EAS)

  - AIMLE Service Producer: AIMLE Server (e.g., T-EAS)

  - Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, Allowed service area information related to the source Edge AIMLE Server ID(s), Allowed list of Target Edge AIMLE Server ID(s) and service area information for context transfer, Allowed list of Target AIMLE Client ID(s) for which context transfer is to done, AIMLE context transfer services (request/response) as scope, List of allowed VAL service(IDs) and allowed corresponding permission level(s), Allowed AIML Task type or operations/services (such as training, model transfer, model inference, model offload, model split), Allowed ML Model ID list/ML Model Information for AIMLE client usage, List of Previous managing AIMLE server ID(s), issuer as authorization server ID.

- AIML service operations control and management procedure:

Process 1

- AIMLE Service: AIMLEServiceOperationsManagement Request/Response

- AIMLE Service Consumer: VAL Server

- AIMLE Service Producer: AIMLE Server

- Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, Allowed target AIMLE Client identifiers, Allowed target AIMLE Client set identifiers, Allowed AIML service operation identifiers (such as model training id, ml task id etc.), Allowed AIML service operation information, Allowed AIML service operation mode (such as start, stop), Allowed AIML service operation mode configuration, Allowed AIML service operation mode status reporting (such as periodic/event based), List of allowed VAL service(IDs) and allowed corresponding permission level(s), Allowed AIML Task type or operations/services (such as VFL/HFL), issuer as authorization server ID.

Process 2

- AIMLE Service: AIMLEClientServiceOperations Request/Response

- AIMLE Service Consumer: AIMLE Server, AIMLE Client

- AIMLE Service Producer: AIMLE Client

- Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, Allowed target AIMLE Client identifiers, Allowed target AIMLE Client set identifiers, Allowed AIML service operation identifiers (such as model training id, ml task id etc.), Allowed AIML service operation information, Allowed AIML service operation mode (such as start, stop), Allowed AIML service operation mode configuration, Allowed AIML service operation mode status reporting (such as periodic/event based), List of allowed VAL service(IDs) and allowed corresponding permission level(s), Allowed AIML Task type or operations/services (such as VFL/HFL), issuer as authorization server ID.

- Transfer Learning Enablement

  - Type 1: AIMLE Service: TLModelSelectionAssistance Request/Response

  - AIMLE Service Consumer: VAL Server

  - AIMLE Service Producer: AIMLE Server

  - Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, List of allowed VAL service(IDs) and allowed corresponding permission level(s), Allowed ML task ID(s),  Allowed ADAE analytic ID(s),  Allowed ML model profile, Allowed TL criteria, Allowed List of VAL UE ID(s), Allowed ML model requirement information, Allowed list of ML models (ML repository ID and address, ML model rating) for AIMLE service consumer, Allowed Transfer mode (i.e., VAL server triggered /UE triggered), Allowed AI/ML target members for transfer learning, issuer as authorization server ID.

  - Type 2: AIMLE Service: UE TLModelSelectionAssistance Request/Response

  - AIMLE Service Consumer: AIMLE Client

  - AIMLE Service Producer: AIMLE Server

  - Token Claims including scope: Requestor ID as Subject, AIMLE service-related information as scope, List of allowed VAL service(IDs) and allowed corresponding permission level(s), Allowed ML task ID(s),  Allowed ADAE analytic ID(s),  Allowed ML model profile, Allowed TL criteria, Allowed List of VAL UE ID(s), Allowed ML model requirement information, Allowed list of ML models (ML repository ID and address, ML model rating) for AIMLE service consumer, Allowed Transfer mode (i.e., VAL server triggered /UE triggered), Allowed AI/ML target members for transfer learning, issuer as authorization server ID.

Editor’s Note: Further details on how the parameters included in the token are used during the authorization verification by the resource server is FFS.

### 6.1.3	Evaluation

The solution uses the SEAL service authorization procedure as baseline with the following impacts:

To secure the SEAL based AIMLE Services, this solution provides enhancements to the access token claims (such as scope and audience) to indicate AIMLE procedure and information flow specific information to allow related verification at the AIMLE Service producer side before providing any service to AIMLE service consumers.

Editor’s Note: Additional evaluation is FFS.

## 6.2	Solution #2: Authorization of AIMLE clients acting as FL members for access to AIMLE Service Security

### 6.2.1	Introduction

This solution proposes the authorization of AIMLE clients in support of federated learning (FL). It ensures that only authorized clients (FL members) are selected, and that secure token-based verification is performed using authorization server. Tokens include only the minimum required claims such as ML model ID / Application Data Analytics Enablement (ADAE) analytics ID and ML model interoperability information to maintain security while ensuring interoperability.

### 6.2.2	Solution details

#### 6.2.2.1	The procedure for AIMLE clients’ authorization

Figure 6.2.2.1-1 Authorization’s procedure of AIMLE clients acting as FL members

1. The VAL server sends a FL member grouping support request to the AIMLE server. The request includes the requestor ID, security credentials, and FL grouping criteria (e.g., grouping method, member selection criteria). The initial request is to create the FL member grouping support as described in Step 1 of clause 8.17.2 of TS 23.482 [3]. The security credentials authenticate the VAL server's identity and authorize the grouping request, with validation by the AIMLE server.

2. Upon receiving the request, the AIMLE server validates whether the requestor is authorized to make it.

3. If authorized, the AIMLE server performs an FL member registration fetch with the ML repository based on the FL grouping criteria (see Step 3 of clause 8.17.2 in TS 23.482[3]).

4. The AIMLE server monitors AIMLE clients (FL members) to check whether they meet the selection criteria from step 1 as described in Step 4 of clause 8.13.2.2 of TS 23.482 [3]. AIMLE server interacts with NEF and/or SEAL services (including SEALDD) to set up monitoring. For location-based criteria, it uses SEAL-LMS (3GPP TS 23.434 [5] clauses 9.3.11/9.3.12) or 5GC services (e.g., NEF) to detect UEs entering or present in the target area.

5.a. Using monitoring results, the AIMLE server selects clients that meet the criteria and removes those that do not (e.g., due to location changes).

NOTE 1: The frequency at which monitoring results are provided is left to the implementation.

5.b. Each selected AIMLE client requests an access token from the AIMLE Server. The access token request sent to the AIMLE Server includes the following parameters: ML model ID / ADAE analytics ID and ML model interoperability information.

5.c. Upon receiving the request, the AMILE server issues the generated access token to the client.

6.a. If AIMLE client obtains the access token, the AIMLE client sends a service request message to AIMLE server, requesting the AIMLE server to join FL group. The message contains the ML model ID / ADAE analytics ID and ML model interoperability information, and access token.

6.b. The AIMLE server performs token verification. The AIMLE server obtains the ML model ID / ADAE analytics ID and ML model interoperability information contained in the access token and verifies whether they match the corresponding values in step 5.c.

6.c. In case of successful access token verification, AIMLE server retains the client.

7. The AIMLE server performs the FL member grouping, notifies selected AIMLE clients of their group membership, collects acknowledgements, and returns a FL member grouping support response to the VAL server that includes success/failure status, grouping details, and an optional expiration time for the grouping.

Editor’s Note: Whether and how an AIMLE server can perform the role of authorization server and token validator is FFS.

Editor’s Note: How the solution addresses the authorization aspects of AIMLE client related AIMLE service procedures in TS 23.482 [3] is FFS.

### 6.2.3	Evaluation

This solution ensures that only authorized AIMLE clients participate as members in FL process. It introduces a token-based authorization process handled by the AIMLE server with support from SEAL.

Editor’s Note: Further evaluation is FFS.

## 6.3	Solution #3: Re-using existing mechanisms

### 6.3.1	Introduction

This solution addresses key issue #1 (Authorization for AIMLE Service Security for AIML members) and key issue #2 (Secure AIMLE ML Model Access) by re-using existing mechanisms available in SEAL security architecture.

### 6.3.2	Solution details

There is no new interface specified for AIMLE services.

Editor’s Note: Further analysis and clarification on interfaces are FFS.

Thus, security for all the interfaces used in the AIMLE has already been addressed including the authorization aspects. Finer granular authorization such as who can be involved in the FL or who can access which ML model can be done locally at the server by using local policy.

Editor’s Note: Clarification on available security for AIMLE interfaces is FFS.

### 6.3.3	Evaluation

TBD

## 6.4	Solution #4: Authorization for Secure AIMLE based ML Model Access

### 6.4.1	Introduction

This solution address KI#2.

### 6.4.2	Solution details

AIMLE authorization related to AIML Services can reuse the authorization procedure specified in TS 33.434 [2] clause 5.2.2 (SEAL service authorization) and clause B.3.3 (SEAL service authorization) as the baseline where, SIM-S or AIMLE Server (with SIM capabilities) acts as an authorization server and issues access token to the AIMLE service consumer. The AIMLE service producer provides the requested services to the AIMLE service consumers by verifying the authorization of AIMLE service consumer i.e., on validating the access token claims as shown in Figure 6.1.2-1 and related step description in Clause 6.1.2 (See Solution #1).

The specific authorization related adaptations to AIMLE Service-related procedures include the following:

- ML Model retrieval:

  - AIMLE Service: MLModelRetrieval Request/Response, Subscribe/Notify, UpdateSubscription, Unsubscribe

  - AIMLE Service Consumer: AIMLE Client, VAL Server

  - AIMLE Service Producer: AIMLE Server

  - Token Claims including scope: Requestor ID (i.e., AIMLE Service Consumer ID) as Subject, AIMLE service-related information as scope, VAL service ID/information, VAL server ID/VAL UE ID/AIMLE Client ID etc., allowed ML model retrieval filters, and Model Information such as ML Model ID(s)/address or Analytics IDs, audience claim as (AIMLE server /AIMLE Model repository for the AIMLE related services), Issuer as Authorization Server ID (i.e., SIM-S ID or AIMLE Server ID).

- ML Model Training:

  - AIMLE Service: MLModelTraining Reqeust/Response

  - AIMLE Service Consumer: VAL Server

  - AIMLE Service Producer: AIMLE Server

  - Token Claims including scope: Requestor ID (i.e., AIMLE Service Consumer ID) as Subject, AIMLE service-related information as scope,Allowed Training Type (VFL or HFL or both VFL and HFL), Allowed List of AIMLE member client IDs, Allowed location information for member client selection, Allowed ML Model ID list/ML Model Information for training, allowed ML model training notification target address, ML Model selection filtering criteria, Issuer as Authorization Server ID (i.e., SIM-S ID or AIMLE Server ID).

- ML Model Management:

  - AIMLE Service: (i) ModelInformationStorage Request/Response (ii) ModelInformationDiscovery Request/Response

  - AIMLE Service Consumer: AIMLE Server, (VAL Server, AIMLE Client (via AIMLE Server))

  - AIMLE Service Producer: ML Repository

  - For (i) Token Claims including scope: Requestor ID (i.e., AIMLE Service Consumer ID) as Subject, AIMLE service-related information as scope, Allowed ML information (Model ID, Model Type, or ML model identified by Analytics ID (or) ML model address from where ML model can be downloaded), Allowed AIMLE client ID (or) ML model source identifier (e.g., VAL server ID, VAL client ID, Target ML repository information), Related VAL service ID(s), ML model address, base Model ID, analytics ID, Model size, domain information, Related Vendor ID(s), Issuer as Authorization Server ID (i.e., SIM-S ID or AIMLE Server ID).

  - For (ii) Token Claims including scope: Requestor ID (i.e., AIMLE Service Consumer ID) as Subject, AIMLE service-related information as scope, VAL service ID, ML model source identifier, Allowed List of AIMLE member client IDs, Allowed location information for member client selection, Allowed ML Model ID list/ML Model Information for discovery, ML model address, base Model ID, analytics ID, allowed ML model training notification target address, ML Model selection filtering criteria, domain information, required accuracy level, Issuer as Authorization Server ID (i.e., SIM-S ID or AIMLE Server ID).

- ML Model Training Capability Evaluation:

  - AIMLE Service: MLModelTrainingCapabilityEva Reqeust/Response

  - AIMLE Service Consumer: AIMLE Server

  - AIMLE Service Producer: AIMLE Client

  - Token Claims including scope: Requestor ID (i.e., AIMLE Service Consumer ID) as Subject, AIMLE service-related information as scope, Allowed availability time for supporting FL operations, Allowed test task information, Allowed AI/ML model ID and model parameters, Allowed dataset requirements (such as common feature ID(s), Data domain feature ID(s) list, Data source), etc., List of allowed VAL service(IDs) and allowed corresponding permission level(s), Allowed AIML Task type or operations/services (such as VFL/HFL), Issuer as Authorization Server ID (i.e., SIM-S ID or AIMLE Server ID).

- ML Model Update:

  - AIMLE Service: MLModelUpdate Request/Response

  - AIMLE Service Consumer: VAL server, ADAE server, AIMLE client

  - AIMLE Service Producer: AIMLE Server

  - Token Claims including scope: Requestor ID (i.e., AIMLE Service Consumer ID) as Subject, AIMLE service-related information as scope, Allowed ML Model ID, Allowed performance degradation information, Allowed ML model retrieval endpoint (such as URL, URI, IP address), delegated ML model information discovery service via AIMLE server ID(s) list, Issuer as Authorization Server ID (i.e., SIM-S ID or AIMLE Server ID).

- ML Model performance monitoring:

  - AIMLE Service: MLModelPerfMonitor Subscriber/Notify

  - AIMLE Service Consumer: VAL Server

  - AIMLE Service Producer: AIMLE Server

  - Token Claims including scope: Requestor ID (i.e., AIMLE Service Consumer ID) as Subject, AIMLE service-related information as scope, Allowed ML Model ID, Allowed Notification endpoint (such as URL, URI, IP address), Allowed AIML operation information (such as ML model training, VFL, HFL, TL etc.), List of VAL service ID, List of AIMLE client ID(s), AIMLE service KPI, Allowed monitoring report configuration, Allowed area of interest, Allowed validity time period, Allowed trigger actions, Issuer as Authorization Server ID (i.e., SIM-S ID or AIMLE Server ID).

- AIMLE assisted ML Model selection:

  - AIMLE Service: AssistedMLModelSelection Subscriber/Notify

  - AIMLE Service Consumer: VAL Server

  - AIMLE Service Producer: AIMLE Server

  - Token Claims including scope: Requestor ID (i.e., AIMLE Service Consumer ID) as Subject, AIMLE service-related information as scope, Allowed AIML Profile (such as list of Allowed ML Model ID(s), ML model requirements, Allowed AIMLE Client set ID(s),  Allowed AIMLE Client selection criteria, Allowed number of AIMLE clients, Allowed data set ID(s), Allowed Training requirements, Allowed Notification target endpoint (such as URL, URI, IP address), Allowed Notification settings etc.,), Delegated list of AIMLE Server IDs (to perform candidate ML model selection service, ML model information storage service etc., for the AIMLE service consumers), List of VAL service ID, Resource Owner ID as GPSI etc., Audience as AIMLE Server IDs, Issuer as Authorization Server ID (i.e., SIM-S ID or AIMLE Server ID).

Editor’s Note: Further details on how the parameters included in the token are used during the authorization verification by the resource server is FFS.

### 6.4.3	Evaluation

The solution uses the SEAL service authorization procedure as baseline with the following impacts:

To secure the SEAL based AIMLE Services, this solution provides enhancements to the access token claims (such as scope and audience) to indicate AIMLE procedure and information flow specific information to allow related verification at the AIMLE Service producer side before providing any service to AIMLE service consumers.

Editor’s Note: Additional evaluation is FFS.

## 6.5	Solution #5: FL member authorization for AIMLE services

### 6.5.1	Introduction

As specified in TS 23.482[3], the FL members consuming the AIMLE services are AIMLE or VAL server or VAL clients. There are several procedures defined such as ML model retrieval, ML model training, FL member registration, event subscription, AIMLE client registration/discovery/selection/participation so on.

As most of the interaction is between AIMLE client to AIMLE server or VAL server to AIMLE server, it is proposed to re-use the SEAL and VAL service authorization procedure as specified in TS 33.434[2].

### 6.5.2	Solution details

For any interaction between AIMLE clients and the AIMLE server, the AIMLE client is provided with the access token by the SIM-S as specified in 5.2 of clause 33.434[2].

For any interaction between VAL server and AIMLE server (e.g., model training), the VAL server is provisioned with an access token by out of band means which is scoped for accessing AIMLE server same as defined for VAL server accessing SEAL key management services in clause 5.3 of TS 33.434[2].

Editor’s Note: Clarification on interaction between VAL server and AIMLE server is FFS.

For any interaction between VAL server or AIMLE server and ML repository (e.g., FL member registration), the VAL server or AIMLE server is provisioned with an access token by out of band means which is scoped for accessing ML repository same as defined for VAL server accessing SEAL key management services in clause 5.3 of TS 33.434[2].

For the procedures like AIMLE client selection/participation or FL member (AIMLE clients) grouping, the AIMLE client is already registered towards ML repository/AIMLE server through SEAL service authorization as specified in 5.2 of clause 33.434[2], no additional authorization procedure is required.

Editor’s Note: Who performs the role of Authorization Server is FFS.

Editor’s Note: Further details on how the solution addresses the overall scope of AIMLE procedures between AIMLE members (FL members) related to KI#1 is FFS.

### 6.5.3	Evaluation

TBD

## 6.Y	Solution #Y: <Solution Name>

### 6.Y.1	Introduction

Editor’s Note: Each solution should list the key issues being addressed.

### 6.Y.2	Solution details

### 6.Y.3	Evaluation

Editor’s Note: Each solution should motivate how the security requirements of the key issues being addressed are fulfilled.

# 7	Conclusions

## 7.1	Key Issue #1: Authorization for AIMLE Service Security for AIML members

## Editor’s Note: This clause contains the agreed conclusions for Key Issue #1.

## 7.2	Key Issue #2: Secure AIMLE ML Model Access

Editor’s Note: This clause contains the agreed conclusions for Key Issue #2.

Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA3#123 | S3-252919 |  |  |  | AIMLE Service Security TR Skeleton | 0.0.0 |
| 2025-09 | SA3#123 | S3-253004 |  |  |  | Included Contributions: S3-253003, S3-253005 S3-253006 | 0.1.0 |
| 2025-10 | SA3#124 | S3-253701 |  |  |  | Included Contributions: S3-253134, S3-253697, S3-253698, S3-253699, S3-253700 | 0.2.0 |
| 2025-11 | SA3#125 | S3-254535 |  |  |  | Included Contributions: S3‑254569, S3‑254570, S3‑254571, S3‑254572, S3‑254573, S3‑254574, S3‑254575 , S3‑254727 | 0.3.0 |