3GPP TR 33.806 V0.1.0 (2014-01)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Pilot development of Security Assurance

Specification for MME Network Product class

(Release 13)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

LTE, UMTS, GSM, security, assurance, security evaluation

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2013, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	5

4	Network Product Class Description	5

4.1 Minimum set of functions defining the MME network product class	5

4.2 Scope of the SCAS for the MME network product class	6

4.2.1 3GPP-defined functionality	6

4.2.2 Non-3GPP-defined functionality	6

4.2.3 Interfaces	6

5	Security Problem Definition	7

5.1	Introduction	7

5.2	Critical assets	7

5.3	Analysis	7

5.3.1	Inside attacker capabilities	7

5.3.2	Types of attacks by insiders	8

5.3.3	External attackers	9

5.3.4	Attacker strength	9

5.4	Threats	9

5.4.1 Threats relating to 3GPP-defined interfaces	9

5.4.2 Other Threats	10

5.4.2.1 T3 Threats from the compromised UE or misbehaving UE	10

5.4.2.2 Security threats on MME software package integrity and anti-virus	10

5.4.2.3 T2 Disclosure of sensitive information in the storage	10

5.4.2.4 T1 Threat from the Internal attacks	10

5.4.2.5 Security threats on MME management and maintenance interfaces	11

5.4.2.6 Security threats on MME user account and password management	11

5.4.2.7 Threats of User identities	11

5.5	Security objectives	11

5.5.1 Objectives relating to 3GPP-defined interfaces	11

5.5.2 Other Objectives	12

6	Security Functional Requirements	12

6.1	Introduction	12

6.2	Security Functional Requirements Deriving from 3GPP Specifications	12

6.x	<Class x of SFRs>	13

6.3 R3 Security requirement for T3: Resource exhaustion attacks prevention	13

6.4 R2 security requirement for T2: Sensitive information storage security	14

6.5 R1 Security requirement for T1: Internal attacks prevention	14

6.6 Security requirements on MME Management and Maintenance interfaces	15

6.7 Security requirements on MME user account and password management	15

6.8 Requirements of User identities	15

6.9 O&M privilege management requirements on MME Management and Maintenance	15

7	Security Requirements related to Hardening	15

8	Security Assumptions on the Operational Environment	16

9	Basic Vulnerability Testing Requirements	16

Annex Z: Change history	16

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions and abbreviations

Editor's note: This section will initially contain the term definitions and abbreviations for SECAM. They will be derived from those contained in TR 33.916.

# 4	Network Product Class Description

## 4.1 Minimum set of functions defining the MME network product class

This clause defines the ‘MME network product class’. MME stands for ‘Mobility Management Entity’, cf. 3GPP TR 21.905.

According to TR 33.916, a network product class is a class of products that all implement a common set of 3GPP-defined functionalities. Therefore, in order to define the MME network product class it is necessary to define the common set of 3GPP-defined functionalities that is constitutive for an MME.

For the purposes of the present specification, we define this common set to be the list of functions contained in clause 4.4.2 of 3GPP TS 23.401, v8.x.0 [xx].  For the detailed description of these functions, TS 23.401, v8.x.0 [xx] or a later version may be applied.

Editor's note: The version v8.x.0 of TS 23.401 that is to be included here remains to be selected. It is to be a version that is approved at a meeting 3GPP SA meeting reasonably close to the 3GPP SA meeting approving the present SCAS.

NOTE 1: The reason why the definition of the common set of functions refers to a particular Release 8 version of TS 23.401, contrary to what is customary in 3GPP when referencing other 3GPP specifications, is that a Security Assurance Specification is to avoid having a moving target when defining a network product class. Nevertheless, the set of functions in clause 4.4.2 of 3GPP TS 23.401, v8.x.0 is expected to be stable, as only FASMO corrections are allowed to Release 8. Furthermore, this set is believed to be minimal, i.e. implemented by all MME network products, which may not be true for the corresponding set of functions from later releases of TS 23.401. For the description of these functions compliance with TS 23.401, v8.x.0 [xx] or a later version is allowed as, obviously, an MME network product should still remain a member of the MME class when it implements a FASMO correction to v8.x.0.

## 4.2 Scope of the SCAS for the MME network product class

Editor’s note: It is ffs whether the classification as below is suitable. It needs to be cross-checked with other classifications used in the present document

### 4.2.1 3GPP-defined functionality

An MME network product will, in many cases, implement MME functions from various releases of TS 23.401 and other pertinent 3GPP specifications, in addition to those from the minimal set of functions in clause 4.1. Vendors are, to a large extent, free to select the features implemented in their MME network products. E.g. an MME network product could lack support for relay nodes, as introduced in Release 10, but implement all other features defined up to and including Release 10. The set of MME functions actually implemented in an MME network product is then to be described in the SCAS instantiation. But the SCAS for the MME network product class needs to cover all such possible MME functions. Furthermore, it is to be avoided that a particular version of an SCAS becomes a moving target. This leads to the following requirement:

The present SCAS shall cover the security problems and security requirements for all MME functions described in the versions of 3GPP specifications approved at 3GPP SA#xy.

Editor's note: It is ffs whether it would be sufficient to mention the release of the specs. If it is decided to mention the SA plenary meeting number then the meeting 3GPP SA#xy remains to be selected. It is to be chosen reasonably close to the 3GPP SA meeting approving the present SCAS so as to still allow for proper consideration in the present SCAS of recent changes in MME functions incorporated in other specs.

### 4.2.2 Non-3GPP-defined functionality

The MME network product class will also contain functionality not or not fully covered in 3GPP specifications.

Examples include, but are not limited to, local or remote management functions.

Editor's note: It is ffs which functionality not or not fully covered in 3GPP specifications needs to be described in which level of detail.

### 4.2.3 Interfaces

Editor's note: This subclause is to include information about interfaces of the MME network product class. Title and content of this subclause are ffs.

This version of the present specification assumes that the MME product is implemented on dedicated hardware. Aspects of virtualisation and cloud are not taken into account in this version.

Editor's note: Aspects of virtualisation and cloud are ffs. They deserve separate study for finding out how to define the boundaries between the MME network product class and the hosting environment (e.g. shared HW and Virtual Machine) and which security assumptions to make on this environment.

# 5	Security Problem Definition

Editor's note: Details on the expected content of this clause are described in TR 33.805, clause 5.2.2.4.2.2. They are used here to further refine the structure of this clause.

## 5.1	Introduction

## 5.2	Critical assets

Editor's note: As specified by TR 33.805, clause 5.2.2.4.2.2, this subclause lists all critical assets. Each asset shall be given a unique identifier for later reference from the threats.

The critical assets of MME to be protected are:

-	User account data and credentials (e.g. passwords)

-	Log data.

Editor note: it is ffs if all of the log data is critical asset.

-	Configuration data, e.g. MME’s IP address, ports, VPN ID etc.

Editor note: it is ffs whether only security relevant part of the configuration data is critical asset

-	Operating System (OS)

Editor note: it is ffs whether only security relevant part of the OS or the whole OS is critical asset

-	Applications

Editor note: it is ffs what applications are critical asset.

-	Mobility Management data: e.g. subscriber’s identities (eg. IMSI), subscriber keys (i.e KNASenc, KNASint, NH), authentication parameters, address of serving eNB, APN name, data related to mobility management like UE status, UE’s IP address,etc., session management like PDN type, QoS and so on, or node selection and routing selection, e.g. IP address of UE related S/P-GW, selected routing connection based on UE’s identity, etc.

-	Sufficient processing capacity: that processing powers are not consumed close to limits.

-	Hardware, e.g. mainframe, board, power supply unit etc.

-	The  interfaces of MME to be protected and which are within SECAM scope: for example

-	Console interface, for local access: local interface on MME

-	O&M interface, for remote access: interface between MME and O&M system

Note: The detailed interfaces of the MME class shall be described in the clause 4,Network Product ClassDescription of this TR..

## 5.3	Analysis

### 5.3.1	Inside attacker capabilities

The term inside attacker is often used to describe an attacker with some form of privileged access to the target.  The term is not sufficiently clear on its own, and needs to be elaborated in the context of the MME to be useful in the SCAS.

In a most generic sense of the word, an inside attacker can target the MME in many ways, e.g., by:

-	access and modify configuration data

-	access and modify subscriber data

-    access subscriber traffic data and location data

-	access node statistics

-	modify software, firmware and operating systems

-	make physical modifications to boxes, connections and can add hardware (e.g., splitters) on cables.

-	replace the MME with any function of the attacker's choice

-	delete/view/modify logs

-	Control (shutdown) applications and processes on the Network element

-	Read/Modify security credentials

Which of these capabilities the attacker is in possession of, may depend on his/her current user-access role. If there is a role based access control in place, the attacker's capabilities may also depend on how easy it is for the attacker to change his/her user-access role.

Depending on which capabilities an inside attacker have, different countermeasures are more or less effective. Therefore, the attacker model, threats and countermeasures should not be based on the umbrella term "inside attacker", but rather on which particular capabilities the attacker has, which assets the attacker target and which interfaces the attacker uses to get to these assets.

### 5.3.2	Types of attacks by insiders

A system may or may not permit anonymous user access, i.e. without strong user identification. Logging measures may or may not be in place, and the inside attacker may or may not be knowledgeable on what system logging measures are active. Irrespective of user identification strength, and of logging measures (actual and perceived), it is clear that it is not always possible to directly prevent an inside attacker who chooses to attack regardless of detection mechanisms and regardless of own risk-taking and consequences. A powerfully inside attacker (or team of insider attackers) may also know of all countermeasures, and may be able to circumvent the countermeasures without risk to themselves.

Not all inside attackers have all the capabilities listed above. For example, one operations OAM engineer may be able to modify the configuration data, but may not be able to modify the software. An administrator of the hardware may, conversely, be able to upgrade the operating system on the node, but may not be able to modify the configuration data (without being detected). Attack threshold will depend on if the attacker can act anonymously or not,

When the capabilities of the inside attacker are sufficiently specified, it is possible to add counter measures, e.g., for protection (e.g., access control on the interfaces used for the attack), and for detection (e.g., logging user id and configuration changes).

The insider attacks against the MME may be classified as follows:

-	Attacks during manufacturing process. These are not part of the SCAS, but part of NESAG's work in GSMA.

-	Attacks on connections to and from the MME. These can be further categorised into attacks that are possible to perform using an operator-external interfaces and attacks on operator-internal interfaces. An operator-external interface to the MME is in this respect an interface that can be used by parties outside of the operator community, e.g., attacks via the NAS protocol. Operator-internal interfaces are interfaces that are accessible to personnel in the operator community. An example of an operator-interface could be the S10 reference point between two MMEs. An attacker may request the security context for a particular UE and, since it is part of the protocol description, the MME that gets the request responds. This would provide a potential internal attacker with access to security sensitive data. All these interfaces that are part of 3GPP specifications are out of scope for the attacker model used for the SCAS, since they are assumed covered by the relevant protocol specifications. However, the SCAS may still add requirements on that the protocol implementation used on these interfaces should be fuzzed.

-	Attacks by authorized and authenticated personnel via remote and local OAM can only be handled by personnel management at the operator or companies contracted by the operator to run the network operations. However, as an aid in deterring and detecting insider attacks the MME can provide logging information of user id and of events where, e.g., configuration has been changed.

Prevention of attacks where an insider modifies or accesses assets that the attacker is authorized and authenticated to modify or access are left out of scope of the SCAS. Access control and logging mechanisms can be supported to help detecting these types of attacks is possible by the operator. Such measures also act as attack deterrents. Note however that if logs can be modified, as permitted by the capabilities of the role(s) that are accessible to the attacker,  then even this countermeasure renders less effective.

### 5.3.3	External attackers

External attackers are considered to be those that have no privileged access to the target. That is, an attacker that only has access to the MME via the external interfaces identified in the MME model. Note that the line between an external and internal attacker is not clear cut. One type of external attacker is a user which can only access the MME via the terminal, i.e., using the NAS protocol. Another type of external attacker is someone with access to an interconnect network, and via this network can access the MME via, e.g.,. the S10 interface. The latter may be considered as being a semi-insider.

Attacks, threats and countermeasures shall not be described in terms of an attacker being an insider or an external attacker, but rather in terms of the capabilities of the attacker, there is no need to further distinguish between insider and external attackers.

### 5.3.4	Attacker strength

A sufficiently powerful attacker may bribe and blackmail people on the inside. In that way the attacker can create a team of inside attackers with the necessary combination of capabilities to perform the attack desired. As seen from above it is not possible to completely protect against this type of attacker,  Another indirect countermeasure (besides, for example, proper authentication and authorization in combination with logging) for such attack types can include personnel management, which however is not within the scope of MME SCAS.

Editor's note: SA3 need to discuss how powerful attackers the MME SCAS shall aim to consider and what residual risks shall remain.

## 5.4	Threats

Editor's note: TR 33.805, clause 5.2.2.4.2.2 also requires considerations on the attacker model. They are to be included in this subclause as it may make sense to consider the attacker model together with the threat. How to further structure this subclause will be decided when more information on modelling attackers and threats is available.

Editor note: It is ffs whether the classification as below is suitable. It needs to be cross-checked with other classifications used in the present document.

### 5.4.1 Threats relating to 3GPP-defined interfaces

The threats relating to 3GPP-defined MME interfaces, cf. clause 4.2.1, may have been sufficiently covered, explicitly or implicitly, in the course of the work on 3GPP security specifications. There is no need to repeat this work for the purposes of the present SCAS, and these threats and risks are therefore not considered here separately.

NOTE: Not all threats and risks covered by security mechanisms in existing 3GPP security specifications may have been adequately documented in a 3GPP TS or TR. They may have also been addressed in contributions to 3GPP Working Group meetings. A good source for these threats and risks is 3GPP TR 33.821. Note, however, the disclaimer in clause 1 of this TR. Note also that threats that relate to actions local to the MME and/or do not affect interoperability may also not have been addressed by existing 3GPP work.

When threats relating to 3GPP-defined MME interfaces are found that are not sufficiently covered in existing 3GPP security specifications they need to be addressed in the present SCAS. Generic threats, e.g. threats relating to protocol robustness, that also apply to 3GPP-defined interfaces are covered in section 5.4.2.

### 5.4.2 Other Threats

#### 5.4.2.1 T3 Threats from the compromised UE or misbehaving UE

Threat Reference: to be done later

-	Threat Category: to be done later

-	Threat Description: MME can be denial-of-service attacked by a compromised or misbehaving UE. For example, the attacker can control a huge number of compromised or misbehaving UEs to request access to one MME at the same time; these UE to continually send attach request and detach request to the MME. The processing resource on the MME can be exhausted at express speed and the MME becomes unable to process other, valid NAS signalling requests

-	Asset: processing capacity

-	Risk: to be done later

-	Security Objectives: Signaling congestion method and compromised or misbehaving UE detection method are needed.

#### 5.4.2.2 Security threats on MME software package integrity and anti-virus

Security threats exist from software package publication to install/upgrade. The attacker may tamper software package with virus code or Trojan horse, etc. Using tampered software packages in the MME may result in attacks in the LTE network, including information leakage and unauthorized use of network resources.

Editor’s Note: Threats needs to be mapped back to which assets are under threat.

Editor’s Note: Threats’ relation to attacker model needs FFS.

#### 5.4.2.3 T2 Disclosure of sensitive information in the storage

Threat Reference: to be done later

-	Threat Category: to be done later

-	Threat Description: MME stores some sensitive information (i.e. communication keys (i.e KNASenc, KNASint, KeNB), administrator password). An attacker (insiders or equipment maintainer from the vendor) can use the access authorization to access the sensitive information. The attacker also can access user’s sensitive information in clear text via OAM. The attacker can launch futher attacks if the sensitive information has been accessed by the attacker. For example, the attacker can use user’s keys to tamper or fake signaling. The attacker can also use user’s identity and serving eNB’s address to locate and track user.

-	Asset: communication keys (i.e KNASenc, KNASint, KeNB) and administrator password on MME

-	Risk: to be done later

-	Security Objectives: sensitive information protection is needed.

#### Editor’s Note: Threats’ relation to attacker model needs FFS.5.4.2.4 T1 Threat from the Internal attacks

Threat Reference: to be done later

-	Threat Category: to be done later

-	Threat Description: The malicious employee or his/her co-worker misuses the network access and management authorization to attack MME.

-	Asset: MME data and traffic such as network management data, interface configuration data, mobility management data, OS and application software, hardware.

-	Risk: MME can not work correctly and/or critical information (e.g. configuration information) disclosure.

-	Security Objectives: strong user identificationt shall be reinforced and the log functions shall be supported.

#### 5.4.2.5 Security threats on MME management and maintenance interfaces

-	attacker may gain unauthorized access through one of the management or maintenance interfaces

-	attacker may gain control of the MME and potentially the control of the system, resulting in compromise of sentitive user data, system data, and management data.

-	Attacker may disrupt and disable normal system operations.

-	Gaining access to the MME may also allow the attackers to gain access to other network elements such as HSS, S-GW, and eNB through S6a, S11, and S1 interfaces respectively. The results can be devastating.

Editor’s Note: Threats needs to be mapped back to which assets are under threat.

Editor’s Note: Threats’ relation to attacker model needs FFS.

#### 5.4.2.6 Security threats on MME user account and password management

One default user password may be provided on MME and may not be modified in time. The attacker can get this password for low clearance level user, even high clearance level user from document or other approach. With the default password, the attacker can accesss MME, modify configuration and interference the LTE network.

User password may have low level strength, with not enough character numbers, or comprosed of simple characters. The attacker can get such kind passwords with fewer attempts by brute force.

The attacker may get user password, and not detected by legal user. In the situation, security threats can be eliminated by modifiying passwords. For convenience, user may perform modification with historical password, known by the attacker, which would bring security theat.

The means for user password storage is important. The storage should use encryption techniques to avoid information leaking.

The attacker may use brute force to get passwords, which is simply a matter of time.

One user can login from several computers at the same time, which may cause collision configuration or other conflict, which would bring security threats.

#### 5.4.2.7 Threats of User identities

Identities of mobile network subscribers are critical for user privacy. Leakage of these user’s identities can lead to loss of privacy, e.g., tracing of a user. Protection of user’s identities is also requirement from regulators.

## 5.5	Security objectives

Editor's note: As specified by TR 33.805, clause 5.2.2.4.2.2, this subclause lists all security objectives derived from the threats. Each objective shall be given a unique identifier for later reference from the requirements and shall point to the threats it addresses.

### 5.5.1 Objectives relating to 3GPP-defined interfaces

The security objectives relating to 3GPP-defined interfaces, cf. clause 4.2.1, may have been sufficiently covered, explicitly or implicitly, in the course of the work on 3GPP security specifications. Such objectives are therefore not considered here separately as there is no need to repeat this work.

When threats relating to 3GPP-defined MME interfaces are found that are not sufficiently covered in existing 3GPP security specifications they need to be addressed in the present SCAS. Generic objectives, e.g. objectives relating to protocol robustness, that also apply to 3GPP-defined interfaces are covered in section 5.4.2.

### 5.5.2 Other Objectives

# 6	Security Functional Requirements

Editor's note: Details on the expected content of this clause are described in TR 33.805, clause 5.2.2.4.2.3.

Editor's note: Security requirements according to TR 33.805, clause 5.2.2.4.2.3 also include hardening requirements and assumptions on the environment. We propose to consider them in separate main clauses for purely editorial reasons, namely in order to reduce the number of hierarchy levels for the subclause numbering.

Editor's note: Clause 6 is proposed to be structured further by grouping non-3GPP Security Functional Requirements according to themes. This grouping is FFS. Examples of such groupings are provided by [CC] or, in a simpler form, but based on CC, by [ NDPP]. 3GPP-related Security Functional Requirements are proposed to be contained in a subclause of their own as they are not expected to be listed in detail, but addressed by wholesale reference to the relevant 3GPP specifications.

Editor's note: It needs to be explained in this clause how security compliance testing is addressed.

## 6.1	Introduction

## 6.2	Security Functional Requirements Deriving from 3GPP Specifications

NOTE: The term ‘security requirements’ is used in the present SCAS in a different, and more comprehensive, way from 3GPP TS 33.401, the EPS security architecture specification. Clause 5 of TS 33.401 contains high-level security features and requirements, while later clauses of TS 33.401 contain detailed security mechanisms that are required to be implemented by a compliant EPS system. The present SCAS considers both, the high-level security features and requirements and the detailed security mechanisms, to be Security Functional Requirements.

For the purpose of the Security Assurance Specification for the MME product class, we distinguish three categories of requirements (cf. also TR 33.916, clause 5):

1)	Security functional requirements related to protocols and behaviours necessary for secure interoperability between nodes from different vendors that require a certain positive behaviour of a 3GPP function.

This category of requirements is already covered by the interoperability and conformance testing performed independently of SCAS already today. So, nothing remains to be done in the present SCAS for this category.

Editor's note: It is ffs whether to include a reference to documentation for this interoperability and conformance testing.

2)	Security functional requirements related to protocols and behaviours necessary for secure interoperability between nodes from different vendors that require that a 3GPP function does not perform a certain action.

This category of requirements may not be covered by the interoperability and conformance testing performed independently of SCAS already today. In this case, the present SCAS document develops test cases for these requirements unless available from other sources.

Editor's note: Security requirements of the second category need to be identified, and test cases need to be defined. If they are available from other sources a reference is to be included.

3)	Security functional requirements not related to protocols and behaviours necessary for secure interoperability between nodes from different vendors, but rather deal with security features which shall be supported by the network products and consequently strictly related to their implementation.

This category of requirements is within the scope of the present SCAS.

Editor's note: Determine whether there are security requirements of the third category for the MME network product class. If so, test cases need to be defined. If they are available from other sources a reference is to be included.

## 6.x	<Class x of SFRs>

Editor's note: The description of Security Functional Requirements shall follow the template given here, cf. TR 33.805, clause 5.2.2.4.2.3.3:

-	Requirement Name: each security requirement is assigned a unique name. The name preferably indicates the topics covered by the requirement.

-	Requirement reference: a unique identifier. The precise convention for the structure of the reference is ffs.

-	Requirement Description: a detailed description of the security requirement.

-	Threat References: the unique identifiers assigned to the threats the requirement intends to meet.

-	Test Case: defines how the requirement shall be tested, the expected skills and tools to be used to produce the test outputs.

-	Requirement evidences: the expected test results

## 6.3 R3 Security requirement for T3: Resource exhaustion attacks prevention

-	Requirement name: Resource exhaustion attacks prevention

-	Requirement reference: to be done later

-	Requirement Description:

1)	R3-1: Method for prevention of signaling congestion method shall be supported by MME

2)	R3-2: The network, e.g. the MME, needs functionality to detect the compromised or misbehaving UE.

-	Threat reference: T3

-	Test case:

1)	Check if MME has supported the NAS signalling congestion.prevention

2)	Check if the network can provide the method to detect the compromised or misbehaving UE.

-	Requirement evidences: A document in free form describing: the signaling congestion method and compromised or misbehaving UE detection method.

Editor’s Note: some overload control of MME has been defined in 23.401, yet it needs ffs to see if it cater for this requirements of SAS.

Editor note: it is ffs if the CC has the related requirements and whether the requirements are needed to be mapped CC if CC has.

## 6.4 R2 security requirement for T2: Sensitive information storage security

-	Requirement name: Sensitive information security

-	Requirement reference: to be done later

-	Requirement Description:

1)	R2-1: The keys read access rights shall be restricted

2)	R2-2: The sensitive information shall not be revealed as clear text.

Editor note: it is ffs on how to merge this with password protection.

-	Threat reference: T2

-	Test case:

1)	Review the documentation provided by the vendor descriping how to store the sensitive information.

2)	Check if the keys can be read as clear text.

-	Requirement evidences: A document in free form describing: the storage method of the sensitive information.

## 6.5 R1 Security requirement for T1: Internal attacks prevention

-	Requirement name: internal attacks prevention

-	Requirement reference: to be done later

-	Requirement Description:

1)	R1-1:  Each User must be strongly authenticated and uniquely identifiable.

2)	R1-2: The log and audit function shall be supported and be turned on to record the operations on MME.

Editor note: it is ffs whether we need to list what actions should be logged and this may be in another document later.

3)  R1-3: The operation authorization of MME users should be defined.

-	Threat reference: T1

-	Test case:

1)	Whether it is possible for two different, separated individuals to easily get access to the same user account, at:

a)	OS level

b)	Application(s) level

2)	Check if MME supports the log function, that the log function has been turned on, and that the following actions are logged (at OS level and at Application(s) level):

a)	login and log out；User id and time stamp

b)	(FFS)

c)	(FFS)

3)	Whether it is possible for MME users to get undefined/unauthorized operation permission.

-	Requirement evidences: A document in free form describing: the personnel management, the deployment environment of MME which events were generated, and the output from the log reader.

## 6.6 Security requirements on MME Management and Maintenance interfaces

The MME shall support mechanisms to provide confidentiality and integrity protection of the communication between itself and the OAM network.

## 6.7 Security requirements on MME user account and password management

- Consistent security policy should be adopted for user account and password management on MME. Consistent security policy includes password management policy, user account freeze policy and rejecting repeat login policy.

- Password management policy:

  - Password modification shall be performed by force after initial login.

  - Password strength shall be configurable. Passwords shall consist of uppercase, lowercase, numeric and special characters. The number of characters in the passwords shall be configurable.

  - Password shall be modified based on password management policy. Historical passwords shall not be allowed up to a certain number. The number of disallowed historical passwords shall be configurable

  - Password storage shall support one-way hash algorithm with salt value.

- User account lock-out policy:

  - The maximum number of user account login attempts should be configurable.

  - There shall be a delay in allowing a user attempt to log in again when login attempt numbers exceeds the maximum number. This delay should be configurable.

## 6.8 Requirements of User identities

The user identities stored in MME include IMSI, IMEI, MSISDN, IP address of UE. These user identities shall be protected when they are stored, transferred and processed in the MME.

## 6.9 O&M privilege management requirements on MME Management and Maintenance

A Role based access control system uses a centrally managed set of controls which determines how users interact with domains and resources. The domains could be Fault management, Performance management System Admin , etc. The RBAC system controls how users or groups of users are allowed access to the various domains and what type of operation they can perform, i.e., the specific operation command or command group (e.g. View, Modify, Execute).

# 7	Security Requirements related to Hardening

Editor's note: An example is given in TR 33.805, clause 5.2.2.4.2.3.

Editor's note: The description of Security Requirements related to Hardening shall follow the template given here, cf. TR 33.805, clause 5.2.2.4.2.3.3:

-	Requirement Name: each security requirement is assigned a unique name. The name preferably indicates the topics covered by the requirement.

-	Requirement reference: a unique identifier. The precise convention for the structure of the reference is ffs.

-	Requirement Description: a detailed description of the security requirement.

-	Threat References: the unique identifiers assigned to the threats the requirement intends to meet.

-	Test Case: defines how the requirement shall be tested, the expected skills and tools to be used to produce the test outputs.

-	Requirement evidences: the expected test results

# 8	Security Assumptions on the Operational Environment

Editor's note: Assumptions on the environment complement the security requirements on the TOE. This clause could e.g. state that an MME is assumed to be operated in a physically secured environment.

Editor's note: The description of Security Requirements related to assumptions shall follow the template given here, cf. TR 33.805, clause 5.2.2.4.2.3.3:

-	Assumption Name: each security assumption on the operational environment is assigned a unique name.

-	Assumption reference: a unique identifier. The precise convention for the structure of the reference is ffs.

-	Assumption Description: a detailed description of the assumption.

-	Threat References: the unique identifiers assigned to the threats the assumption intends to meet.

-	Test Case and Requirement evidences: are not applicable

# 9	Basic Vulnerability Testing Requirements

Editor's note: The WID clearly states “that the MME SCAS shall describe... Basic Vulnerability Testing activities”. But TR 33.805, clause 5.2.4.4, also states “As Basic Vulnerability Testing is universally applicable for all Network Product Classes, the requirements for this testing category are expected to be specified as a general SCAS module. This general SCAS module will then be linked and potentially amended by SCASs for individual Network Product Classes”. Nevertheless, BVT is included here as the present TR is meant to be used for the SCAS piloting activity, and there needs to be a place for BVT piloting.

Editor's note: It is ffs how to structure the description of BVT requirements.

Annex Z:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2014-03 |  |  |  |  | Initial version after SA3#74 email approval | - | 0.1.0 |