3GPP TSG SA WG3 (Security) Meeting #79bis	S3-151629

Sophia Antipolis, France, 29 – 30 June 2015

Source:	Alcatel-Lucent, Telecom Italia

Title:	Resolving Editor’s Note in B.3.3.3.1

Document for:	Discussion/approval

Agenda Item:	4

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution proposes to resolve the Editor’s Note in clause B.3.3.3.1.

# Discussion

This contribution proposes to resolve the Editor notes in clause B.3.3.3.1 - ‘Resource exhaustion attacks prevention’.

There are 4 Editor notes:

- Editor’s note: the use of the terms “Resource exhaustion”, “overload” and “congestion” needs to be rewiewed. It is FFS if the title should also include detection.

- Editor’s note: The text “or misbehaving UE” needs further clarifications and will require an associated test case.

- Editor's Note: some overload control of MME has been defined in 23.401, yet it needs FFS to see if it cater for this requirements of SAS.

- Editor’s note: it is FFS as to what additional test cases are required for these requirements.

Focussing on the Editor notes 1 to 3 first as these have potential impact on requirements.

The 3 requirements in sub-clause B.3.3.3.1 are made up of one from DT catalogue and the last two from the TR. sub-clause 6.5. If we focus on the last two requirements and editor notes 2 and 3. TS 23.401 has a section on MME Control of Overload (4.3.7.4). Sections 4.3.7.4. contains text relating to the behaviour of UE’s and the assumption is that this may be the origin of the use of the term ‘mis-behaving UE’. TS 23.401 sections 4.3.7.4.2.2 and 4.3.7.4.2.3 also discusses congestion on the signalling interface.

Specifically TS 23.401 has: “The MME shall contain mechanisms for avoiding and handling overload situations. These can include the use of NAS signalling to reject NAS requests from UEs”. (Section 4.3.7.4.1)

So the proposal is that requirements 2 and 3 in B.3.3.3.1 will be dealt with in the same manner as we dealt with requirements in B.3.2 - SFRs deriving from 3GPP specifications.

So requirements 2 and 3 in B.3.3.3.1 will be deleted. A note will be added to B.3.2 that mentions TS.23.401. Test cases for the ‘new’ SFR will be proposed in a separate contribution and the test cases D.3.3.3.1.1 and D.3.3.3.1.2 will be removed and replaced by new ones in a separate contribution.

At one of the SCAS teleconference meetings the question was posed as to what the difference was between the requirements in Annex B3.3.3.1 and B.3.3.3.3. Both seem to cover network product overload.  3.3.3.1  seems to be dealing with an overload where the overload control mechanism and DoS prevention mechanisms act as designed. 3.3.3.3 seems to describe the situation when the load is excessive and the network product needs to take actions to prevent total shutdown and loss of availability /security is imminent.

In this contribution we propose that the title for requirement in Annex B.3.3.3.1 be amended to ‘System handling during overload situations’ to make it distinct from B..3.3.3.3 and secondly to make it a generic requirement for all network products.

In contribution S3-151628 we propose that the title be changed ‘System handling of excessive overload situations’.

It is proposed that the test case in Annex D.3.3.3.3 be used to test requirements B.3.3.3.1 and B.3.3.3.3  The 4th Editor note in B.3.3.3.1 can be deleted.

As for the editor note on including the word ‘detection’ the relevant sections in TS.23.401 deals with both detection and prevention.

The proposals above deal with Editor notes 1 to 4.

# Proposal

***	START 1st CHANGE	***

### B.3.2	SFRs deriving from 3GPP specifications

In the present clause we describe the approach taken towards SFRs deriving from 3GPP specifications and the corresponding test cases. The selected SFRs and the test cases for them can be found in Annex D.

It is assumed for the purpose of the present SCAS that an MME conforms to all mandatory security-related provisions pertaining to an MME in

- 3GPP TS 33.401 “EPS security architecture”

- other 3GPP specifications that make reference to TS 33.401 or are referred to from TS 33.401. (e.g. TS.23.401)

3GPP has decided to develop test specifications for the UE in the TSs of the 34-series under the responsibility of Working Group RAN5. 3GPP saw, however, no need to develop test specifications for network elements. For network elements, 3GPP rather trusts that tests are run under the responsibility of the vendors.

Security procedures pertaining to an MME are typically embedded in mobility management procedures and are hence assumed to be tested together with them. Examples include:

- AKA authentication is embedded in an Attach procedure or a TAU procedure;

- Security Mode Control is embedded in an Attach procedure or a TAU procedure;

- The derivation of a mapped security context is embedded in inter-RAT mobility procedures.

It is the purpose of the present SCAS to identify security requirements from the EPS security architecture that require special attention in testing as they may

(a) lead to vulnerabilities when not satisfied;

(b) not be captured through ordinary testing activity for mobility management procedures;

(c) address security-relevant failure cases and exceptions or ‘negative’ requirements of the kind: “The MME shall not…”

It is not an intention of the present SCAS to provide an exhaustive set of test cases that would be sufficient to demonstrate conformance of all security procedures with the above-mentioned specifications.

***	END OF 1st CHANGE	***

***	START 2nd CHANGE	***

- B.3.3.3.1	 System handling during overload situations

-

Requirement Name: System handling during overload situations -

Requirement Reference: to be done later

-	Requirement Description:

1) The system shall provide security measures to deal with overload situations which may occur as a result of a denial of service attack or during periods of increased traffic. In particular, partial or complete impairment of system availability shall be avoided. Potential protective measures include:

• Restricting of available RAM per application

• Restricting of maximum sessions for a Web application

• Defining the maximum size of a dataset

• Restricting CPU resources per process

• Prioritizing processes

• Limiting of amount or size of transactions of an user or from an IP address in a specific time range

-	Threat Reference: Denial of Service

Security Objective references: tba.

Test case:Refer to Annex D.3.3.3.3

***	END OF 2nd CHANGE	***

***	START 3rd CHANGE	***

***	END OF 3rd CHANGE	***