3GPP TSG SA WG3 (Security) Meeting #79bis	S3-151671

Sophia Antipolis, France, 29 – 30 June 2015	revision of S3-151621

Source:	Nokia Networks

Title:	Mapping TR 33.806, B.4.6, to TS 33.116 and TS 33.117

Document for:	Discussion/approval

Agenda Item:	4

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution:

It was agreed at SA3#79 to include MME-specific requirements into TS 33.116 and generic requirements into TS 33.117. It was further agreed to take clauses from TR 33.806 and map them to the corresponding clauses of the two TSs. The present contribution maps TR 33.806, B.4.6, to TS 33.116 and TS 33.117.

The first section of the contribution copies the text from TR 33.806, B.4.6 (based on the pCR agreed at conf calls#6 on 19 June, 2015, and with the revisions of that pCR accepted), and adds Word comments explaining which part of the text is believed to be MME-specific and which is generic. It turns out that, for this clause, there are no MME-specific parts except that ‘MME’ is mentioned in one place although it can be easily replaced with ‘network product’ to make the text generic.. The existing Word comments from TR 33.806 were removed for better readability of the new comments.

The second section of the contribution provides a pCR to TS 33.117, v0.0.3.

The third section of the contribution provides a pCR to TS 33.116, v0.0.3.

In the present contribution, only the requirements themselves are mapped, not the test cases.

The technical content in S3-151671 does not differ from that in S3-151621.

# Annotated text from TR 33.806

### B.4.6	Network Devices

##### B.4.6.1	Traffic separation

Requirement Name: tba

Requirement Reference: to be done later

Requirement Description:

The MME shall support physical or logical separation of O&M and control plane traffic. See RFC 3871 [9] for further information.

Threat References: Threat of eavesdropping on MME management and maintenance interface data, Threat of man-in-the-middle attack on MME management and maintenance interface, Threat of modification of information in transit on MME management and maintenance interface, Security threats caused by lack of MME traffic isolation

Security Objective references: tba.

Test case: tba

# pCR to TS 33.117 (generic requirements)

***	START OF CHANGE	***

### 5.3.5	Network Devices

#### 5.3.5.1	Traffic Separation

Requirement Name: tba

Requirement Reference: to be done later

Requirement Description:

The network product shall support physical or logical separation of O&M and control plane traffic. See RFC 3871 [9] for further information.

Threat References: tba

Security Objective references: tba.

#### Test case: tba

***	END OF CHANGE	***

# pCR to TS 33.116 (MME-specific adaptations of generic requirements from TS 33.117)

***	START OF CHANGE	***

### 5.3.5	Network Devices

There are no MME-specific additions to clause 5.3.5 of TS 33.117.

***	END OF CHANGE	***