3GPP TSG SA WG3 (Security) Meeting #80                                                         S3-151711

24-28 August 2015, Tallinn (Estonia)

Source:	Telecom Italia

Title:	Adding a test case related to the requirement 5.2.6.2.3 “GTP-C Filtering”

Document for:	Discussion/approval

Agenda Item:

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution:

This contribution proposes to add a test case related to the requirement 5.2.6.2.3 of TS 33.117.

# Introduction

This contribution proposes to add a test case related to the requirement 5.2.6.2.3 of TS 33.117.

Note that the Requirements Reference in the test case template used in TR 33.806 is no longer needed here as, in TS 33.117, the test case immediately follows the requirement.

# Proposed pCR

***	BEGIN OF CHANGE ***

##### 5.2.6.2.3	GTP-C Filtering

Editor's Note: it is ffs whether this requirement can be generalized to cover both GTP-C and GTP-U or that a separate requirement or no requirement for GTP-U is needed.

Requirement Name: GTP-C Filtering

Requirement Reference: to be done later

Requirement Description:

The following capability is conditionally required:

-	For each message of a GTP-C-based protocol, it shall be possible to check whether the sender of this message is authorized to send a message pertaining to this protocol.

NOTE 1: 	The check could be performed e.g. against a whitelist or blacklist of permitted message type / sender identity combinations.

-	At least the following actions should be supported when the check is satisfied:

-	Discard: the matching message is discarded.

-	Accept: the matching message is accepted.

-	Account: the matching message is accounted for, i.e. a counter for the rule is incremented. This action can be combined with the previous ones. This feature is useful to monitor traffic before its blocking.

This requirement is conditional in the following sense: It is required that at least one of the following two statements holds:

-	The Network Product supports the capability described above and this is stated in the product documentation.

-	The Network Product's product documentation states that the capability is not supported and that the Network Product needs to be deployed together with a separate entity which provides the capability described above.

NOTE 2: 	Such a separate entity could e.g. be a GTP Firewall.

NOTE 3: 	Test cases for this separate entity are not provided in the present document, but are believed to be similar to them.

Threat References: tba

Security Objective references: tba.

Test case:

Test Name: TC_GTP-C_FILTERING

Purpose:

To verify that the network product provides filtering functionalities for incoming GTP-C messages. In particular this test case verifies that:

- The network product provides filtering of incoming GTP-C messages on any interface.

- It is possible to block all GTP-C messages on those network product interfaces where they are unwanted.

- It is possible to specify defined actions for each rule.

Procedure and execution steps:

Pre-Conditions:

- The network product has at least two physical interfaces, named if1 and if2

- The tester has the privileges to configure GTP-C filtering on the network product.

- The manufacturer declares that the GTP-C filtering is supported.

- The manufacture includes a guideline to configure the GTP-C filtering in the documentation accompanying the network product.

- A network traffic generator or a pcap file containing the GTP-C messages is available.

- A network traffic analyser on the network product (e.g. tcpdump) is available.

Execution Steps

- The tester log in the network product.

- The tester configures the network product with the following rules:

  - Accept only GTP-C EchoRequest messages on if1;

  - Discard all GTP-C messages on if2.

  - For each rule above the accounting is also enabled.

- The tester turns on the network traffic analyzer on if2.

- The tester sends on if2 EchoRequest messages replying a pcap file or using a network generator.

  - Using the network analyzer the tester verifies that the network product correctly receives the EchoRequest messages on if2.

  - Using the accounting, the tester verifies that the messages are discarded and that any response is sent back by the network product.

- The tester sends to if1 EchoRequest messages replying a pcap file or using a network generator.

  - Using the network analyzer, the tester verifies that the messages are correctly received by the network product.

  - The tester verifies that the GTP-C EchoRequest messages are not discarded because EchoResponse messages are sent back by the network product.

- The tester verifies that the matching messages are correctly accounted for both rules.

- The tester sends to if1 GTP-C messages different from EchoRequest replying a pcap file or using a network generator.

  - Using the network analyzer, the tester verifies that the messages are correctly received by the network product.

  - Using the accounting,  the tester verifies that the messages are discarded and that any response is sent back by the network product.

- The tester deletes the previous rules and configures a new rule, i.e. to accept only GTP-C EchoRequest on if1 coming from a certain IP Address named IP1.

- The tester sends GTP-C EchoRequest messages with source IP Address set to IP1.

  - Using the network analyzer, the tester verifies that the messages are correctly received by the network product.

  - The tester verifies that the GTP-C EchoRequest messages are not discarded and EchoResponse messages are sent back by the network product.

- The tester sends GTP-C EchoRequest messages with source IP Address set to IP2 different from IP1 using a network traffic generator or replying a pcap file.

  - Using the network analyzer the tester verifies that the messages are correctly received by the network product.

  - The tester verifies that the GTP-C EchoRequest messages are discarded and that no EchoResponse messages are sent back.

Expected Results:

- For steps 4, 5, 6 and 7 the tester receives GTP-C EchoResponse messages from if1 only.

- For steps 4, 5, 6 and 7 the messages matching the rules are correctly accounted.

- For steps 8, 9, 10 the tester receives GTP-C EchoResponse messages only for the authorized source IP address.

Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

- The used tool(s) name and version  information,

- Settings and configurations used,

- Pcap trace

- Screenshot

- Test result (Passed or not)

***	END OF CHANGE	***