3GPP TSG SA WG3 (Security) Meeting #81	S3-152301

9-13 November 2015 Anaheim(US)		revision of S3-13abcd

Source:	Huawei, HiSilicon

Title:	Mapping test case from 6.7 of 33.806 to 5.2.3.4.4.1 Network Product Management and Maintenance interfaces in 33.117

Document for:	Discussion/approval

Agenda Item:	7.10

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: Mapping testcase from 6.7 of 33.806 to 5.2.3.4.4.1 Network Product Management and Maintenance interfaces in 33.117

- Introduction

Mapping testcase from 6.7 of 33.806 to 5.2.3.4.4.1 Network Product Management and Maintenance interfaces in 33.117.

- Proposed pCR

***	BEGIN OF CHANGE	***

# 5.2.3.4.4.1 	Network Product Management and Maintenance interfaces

Requirement Name: Network Product Management and Maintenance interfaces

Requirement Reference: to be done later

Requirement Description: The protocols used for the network product management shall support mutual authentication mechanisms.

Threat References: TBA

Security Objective references: Secure Network Product Administration.

Test case:

- Pre-conditions: The network product is powered on, and has authentication credential with Entity A. The tester can trigger authentication between network product and specific management entities. Entity A has authentication credential shared witth the network product and entity B does not have authentication credential with network product.

- Steps taken to perform the test:

- The tester check authentication mechanism configuration on network product, set one of options which supports mutual authentication.

- The tester triggers communication between network product and entity A which starts the mutual authentication between the network product and entity A.

- Then, the tester triggers communication between network product and entity B which starts the mutual authentication between the network product and entity B.

- Expected results:

- Mutual authentication is successful and communication between network product and entity A established successfully.

- Mutual authentication fails and communication between network product and entity B cannot be established.

***	END OF CHANGE	***