3GPP TSG SA WG3 (Security) Meeting #81	S3-152308

9-13 November 2015 Anaheim(US)		revision of S3-13abcd

Source:	Huawei, HiSilicon

Title:	Modify test case of 5.2.2.4.1 of 33.116

Document for:	Discussion/approval

Agenda Item:	7.10

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution proposes to modify testcase of 5.2.2.4.1 bidding down prevention in X2-handovers of 33.116.

- Introduction

The test case of 5.2.2.4.1 only contains positive result that is if the UE’s capability is the same, the MME will go on executing the procedure. However, it is hard for a tester to determine whether the MME has checked the UE’s capability. The same result can also be obtained if the MME doesn't do the check. Therefore, we propose to add a negative test to confirm that the MME do compares the UE’s capability.

- Proposed pCR

***	BEGIN OF FIRST CHANGE	***

5.2.2.4.1 	Bidding down prevention in X2-handovers

Requirement Name: Bidding down prevention in X2-handovers

Requirement Reference: TBA

Requirement Description: "The MME shall verify that the UE EPS security capabilities received from the eNB are the same as the UE EPS security capabilities that the MME has stored." as specified in TS 33.401, clause 7.2.4.2.2."

Threat References: TBA

Security Objective References: TBA

Test Case:

Purpose:

Verify that bidding down is prevented in X2-handovers.

Pre-Conditions:

Test environment with (target) eNB. eNB may be simulated.

Execution Steps

Positive:

The MME receives the path-switch message with the sameUE EPS security capabilities.

Negative:

The MME receives the path-switch message with different UE EPS security capabilities

Expected Results:

Positive:

The MME verifies that the received UE EPS security capabilities are identical to the ones stored in the MME.

Negative:

The MME may log the event.