3GPP TSG SA WG3 (Security) Meeting #81                                                      S3-152331

9-13 November 2015, Anaheim (USA)

Source:	China Mobile, Telecom Italia,CATR

Title:	Adding test case to the requirement 5.2.3.4.1.1 on System functions shall not be used or accessed without successful authentication and authorization

Document for:	Discussion/approval

Agenda Item:	8.1

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution proposes to add the test case to the requirement 5.2.3.4.1.1 of TS 33.117 and to remove from this same requirement a sentence that is already addressed by 5.2.3.4.1.2.

Introduction

This contribution proposes:

- to add the test case to the requirement 5.2.3.4.1.1 of  TS 33.117.

Note that the Requirements Reference in the test case template used in TR 33.806 is no longer needed here as, in TS 33.117, the test case immediately follows the requirement.

Proposed pCR

***	BEGIN OF CHANGES	***

##### 5.2.3.4.1	Authentication policy

###### 5.2.3.4.1.1	System functions shall not be used or accessed without successful authentication and authorization.

Requirement Name: System functions shall not be used or accessed without successful authentication and authorization.

Requirement Reference: to be done later

Requirement Description:

The usage of a system function or access to sensitive data shall only be possible via unambiguous user identification and successful authentication on basis of the user name and at least one authentication attribute. System functions comprise, for example network services (like SSH, SFTP, Web services), local access via a management console, local usage of operating systems and applications. This requirement shall also be applied to accounts that are only used for communication between systems. An exception to the authentication and authorisation requirement are functions for public use such as those for a Web server on the Internet, via which information is made available to the public.

Threat References: TBA

Security Objective references: tba.

Test case:

Test Name: TC_SYS_FUN_USAGE_OR_ACCESS

## Purpose:

To ensure that system functions shall not be used or accessed without successful authentication and authorization.

## Procedure and execution steps:

## Pre-Conditions:

- The manufacturer shall supply the list of system functions which include network services, local access via a management console, local usage of operating systems and applications etc.

- The manufacturer shall supply the list of access entries for system functions and sensitive data such as SSH, SFTP.

## Execution Steps

The accredited evaluator’s test lab is required to execute the following steps:

- The tester tries to use system functions available via network interfaces and to access the sensitive data which are listed by the manufacturer without using user name and an authentication attribute

- The tester verifies that local access entries to use operating systems and applications and to access the sensitive data which are listed by the manufacturer require successful authentication on basis of the user name and at least one authentication attribute.

## Expected Results:

- The network product does not allow to access to any system function and sensitive data provided by the manufacturer without a successful user authentication.

## Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

- Description of executed tests and commands

- Relevant output (e.g. Screenshot)

- Test result (Passed or not)

***	END OF CHANGES	***