3GPP TSG SA WG3 (Security) Meeting #81                                                        	 S3-152379

# 9 – 13 November, 2015, Anaheim, USA	update of S3-151708

Source:	Nokia Networks

Title:	Providing test cases related to the requirement 5.2.3.6.1 ''Security event loggin''

Document for:	Discussion/approval

Agenda Item:	7.10

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution:

This contribution proposes to add test cases to the requirement 5.2.3.6.1 of TS 33.117.

# Introduction

S3-151708 submitted to SA3#80 was updated to S3-152023 during that meeting and then withdrawn. It appears that S3-152023 was never created. The present contribution is based on the proposed update of 1708 entitled "S3-151708_TestCase-5.2.3.6.1_SecurityEventLogging -no ssh - condensed.docx " that was provided by Nokia on 25 August, with minor editorial changes.

Based on the discussion held during SA3#80,  this contribution proposes:

- To mandate the support of four of the event logs currently listed in the requirement 5.2.3.6.1 of  TS 33.117 v1.0.0;

- To let the support of  “Change of group membership for accounts” event optional;

- To add a column Event Type that can be used to classify the events;

- To add a Timestamp to each logged event;

- To add a test case for each mandatory event;

- Changes for the “Requirement Description” which are not intended to be editorial and need to be better discussed.

- Some minor editorial changes.

Note that the Requirements Reference in the test case template used in TR 33.806 is no longer needed here since in TS 33.117 the test cases immediately follows the requirement.

Word comments serve to motivate the changes and are to be removed before implementing the pCR.

# Proposed pCR

***	BEGIN OF FIRST CHANGE	***

#### 5.2.3.6	Logging

##### 5.2.3.6.1	Security event logging

Requirement Name: Security event logging

Requirement Reference: TBA

Requirement Description: Security events shall be logged together with a unique system reference (e.g., host name, IP or MAC address) and the exact time the incident occurred. For each security event, the log entry shall include user name and/or  timestamp and/or performed action and/or result and/or length of session and/or values exceeded and/or value reached.

RFC 3871 [4] section 2.11.10 specifies the minimum set of security events. Each vendor shall document what security events the product logs so it can be verified by testing.

In particular, it shall be possible to log the following events (which are intended to be supported by the network product and which can be enabled by default at manufacturing time or at a later time by the operator):

| Event Type | Description | Event data to be logged |
| --- | --- | --- |
| Incorrect Login Attempts | Incorrect login attempts | • Username, • Source IP address if remote access • Timestamp. |
| Administrative Access | System access with accounts with administrator rights | • Username, • Access timestamp, • Length of session, • Source IP address if remote access |
| Account administration | Account administration, i.e. configure, delete, enable, and disable | • Administrator username, • Administered account, • Activity performed (configure, delete, enable and disable), • Timestamp |
| Resource Usage | Critical rise in system values such as disk space, CPU load over a longer period. | • Value exceeded, • Value reached, (Here suitable threshold values shall be defined depending on the individual system.) • Timestamp. |

In addition, optionally it shall be possible to log also the following event (if supported):

| Event Types | Description | Event data to be logged |
| --- | --- | --- |
| Change of group membership for accounts | Any change of group membership for accounts | • Administrator account, • Administered account, • Activity performed (group added or removed) • Timestamp. |

Threat References: TBA

Security Objective references: tba.

Test case:

Test Name: TC_SECURITY_EVENT_LOGGING

## Purpose:

To verify that the network product correctly logs all required security event types.

## Procedure and execution steps:

## Pre-Conditions:

- The following information shall be provided by the documentation accompanying the network product:

- The log where the event is recorded (e.g. in the Security Log rather than, for instance, System log) and how it can be accessed (e.g. the complete path).

- If the event type is enabled by default or how to enable it.

- What O&M services can be used on the Network Product in the configuration according to the pre-requisites for testing in clause 5.1 and how to use them.

- The tester has the needed administrative privileges to sufficiently perform the tests

- If needed for testing specific O&M services, a tester machine is available.

## Execution Steps

## For each O&M service perform the following test steps

- The Tester sequentially triggers each security event listed in the requirement, while covering each option detailed in the individual security event descriptions.

- The Tester verifies whether the security events, and their individual options, were correctly logged. In particular it is verified whether they include at least the event data specified as required to be logged.

## Expected Results:

## All security events are appropriately logged, including all required event data.

## Expected format of evidence:

The testing report contains the following information for each security event:

- List of O&M services

- Commands executed per O&M services

- The relevant parts of the logs in appropriate form (e.g. file, screenshot)

- Test result (Passed or not)

***	END OF CHANGES	***