3GPP TSG SA WG3 (Security) Meeting #81	S3-152300

9-13 November 2015 Anaheim(US)		revision of S3-13abcd

Source:	Huawei, HiSilicon

Title:	Mapping D.3.3.4.6.2 Role based access control system to 5.2.3.4.6.2 in 33.117

Document for:	Discussion/approval

Agenda Item:	7.10

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution proposes to Mapping D.3.3.4.6.2 Role based access control system to 5.2.3.4.6.2 in 33.117

# Introduction

We propose to Mapping D.3.3.4.6.2 Role based access control system to 5.2.3.4.6.2 in 33.117.

# Proposed pCR

***	BEGIN OF FIRST CHANGE	***

5.2.3.4.6.2	Role-based access control

Requirement Name: tba

Requirement Reference: tba

Requirement Description:

The network product shall support Role Based Access Control (RBAC). A role-based access control system uses a set of controls which determines how users interact with domains and resources. The domains could be Fault Management (FM), Performance Management (PM), System Admin, etc. The RBAC system controls how users or groups of users are allowed access to the various domains and what type of operation they can perform, i.e. the specific operation command or command group (e.g. View, Modify, Execute).

The network product supports RBAC, in particular, for OAM privilege management for network product Management and Maintenance, including authorization of the operation for configuration data and software via the network product console interface.

Security Objective references: tba.

Test case: tba

## Purpose:

Verify that users are granted with role-based privileges.

## Procedure and execution steps:

## Pre-Conditions:

- The Network Product is powered on.

- The tester has privilege to create a new user account.