3GPP TSG-SA WG3 Meeting #86Bis			Tdoc S3-170944

Busan, Korea, 27-31 March 2017

Title:	LS on security termination for the User Plane in 5G

Response to:	-

Release:	Release 15

Work Item:	5GS_Ph1

Source:	SA3

To:	RAN2, RAN3

Cc:	-

Contact Person:

#### Name:	Stefan Schröder

Tel. Number:

E-mail Address:	Stefan.Schroeder@telekom.de

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S3-170943

1. Overall Description:

SA3 has reached the attached interim agreement that the 5G UP security termination point is in the RAN and located in the PDCP layer. Security for this agreement depends on the assumptions that

- the gNB is split into a Central Unit (CU) and a Distributed Unit (DU)

- the PDCP (control plane and user plane) and RRC layers are terminated in the CU

- the CU can be deployed in an operator’s data centre in a physically secure location

It is SA3's understanding that the requirement to host CUs in a few central data centres has impact on RAN2/RAN3 design decisions regarding the split, e.g. on the latency requirements for the interface between DU and CU.

SA3 acknowledges that there may also be deployments requiring monolithic gNB without this split, but those would need additional security measures for the backhaul, e.g. using IPsec similar to LTE.

2. Actions:

To RAN2 and RAN3

ACTION: 	RAN2 and RAN3 are kindly requested to take the above into account and provide feedback on their progress.

3. Date of Next TSG-SA WG3 Meetings:

SA3#87	15-19 May 2017	Ljubljana, Slovenia

SA3#88	7-11 August 2017	Dali City, China