# 3GPP TSG SA WG3 (Security) Meeting #87	S3-171478

# 15-19 May, 2017, Ljubljana, Slovenia 	update of S3-171312

# Source:	Nokia

# Title:	V2X key issue on malicious UE behaviour clarified

# Document for:	Approval, Discussion

Agenda Item:	8.2 / V2X (TR 33.885)

# 1	Decision/action requested

Adding details to key issue for further consideration in Rel-14 or eV2X.

# 2	References

[1]       3GPP TR 33.885

# 3	Rationale

Adding details to key issue for further consideration in Rel-14 or eV2X. Key issue is about malicious UE, but text details are rather referring on malware or wrong sensors of a possibly honest UE. Thus overall, the key issue should be clarified and it is proposed to provide a description on this.

# 4	Detailed proposal

## **** Begin Change ****

## 5.16	Key Issue #16: Detectability of Malicious LTE-V2X UE Behavior - achieving trust and confidence in messages

### 5.16.1	Key issue details

A good security solution will prevent sending of false messages by malicious users or, if this is not possible, at least it should allow the receiving UE to make a decision whether to trust the message- This prevents that a V2X LTE UE will process a false warning message.

However, it may not be clear for a UE when receiving a message, whether the sending UE is malicious or it has just a mal-functioning sensor that led to this message. Securing the communication only protects against attacks on the air interface, it does not prevent cases where the UE sends out faulty messages due to malware, faulty sensors, or on purpose. As a result, a receiving UE may not be able to tell from the security aspects of a received message alone whether the content of the message is trustworthy. And so, the problem that occurs is that a UE that receives for example an imminent crash warning, may take an inappropriate action which may lead to unsafe road conditions. Consequently, the aim of V2X to improve road safety may not be achieved. Thus, the key issue should address how a receiving UE could achieve trust in such a situation.

In short, the following problems need to be addressed:

How does the receiving UE detect and handle false messages due to

- malware on the UE,

- sensor failures or

- being a malicious UE.

These problems cannot be addressed by only securing the communication channel and so other solutions should be investigated, i.e. how to achieve confidence or trust in the messages that a UE receives either via broadcast over PC5 or over eMBMS.

### 5.16.2	Security threats

A V2X UE may be fooled into taking wrong decisions leading to unsafe road conditions.

Receiving a malicious message may lead the V2X UE to take the wrong action.

### 5.16.3	Security requirements

A V2X UE should have the means to achieve trust or confidence in the messages received.

**** End Change ****