3GPP TSG-SA3 Meeting #89	S3-173390

# Reno, Nevada, United States, 27th Nov 2017 - 1st Dec 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.180 | CR | 0043 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [eMCSEC] Addition of Security Gateway to TS 33.180 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NCSC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCSec |  |  |  |  |  | Date: |  |  | 2017-11-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Security Gateway functionality needs to be added to TS 33.180 to allow an MC domain to communicate with a non-3GPP system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution introduces the concept of a Security Gateway and defines its functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will not be any Security Gateway functionality in Rel-15. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*********************Start of change 1****************************

## 5.10	MC Security Gateway (SeGy)

### 5.10.1	General

The MC Security Gateway is a network function that terminates all MC security functionality from MC domains to allow MC signalling and media to be provided to an external system.

### 5.10.2	Functional model for the MC Security Gateway

A MC Security Gateway (SeGy) communicates with MC domains as a partner MC domain. The SeGy acts as a MC domain towards the MC System on one interface, and provides the signalling and media unencrypted towards an external system. Consequently, the SeGy appears as a MC domain using all security mechanisms towards the MC Domain, and a MC domain using no security towards the external system. Figure 5.10.2-1 shows the role of the SeGy in context.

Figure 5.10.2-1: MC Security Gateway (SeGy)

The SeGy is always in a different security domain to existing MC Domains. In Figure 5.10.2-1, the MC Domain is in Security Domain A, whereas the SeGy is in Security Domain X. This allows the risk of terminating security to be isolated to the SeGy, and allows the use of the SeGy to be communicated to clients that communicate with the SeGy.

### 5.10.3	Functions of a MC Security Gateway

#### 5.10.3.1	Components of a MC Security Gateway

At a high-level, the MC Security gateway is composed of four components:

-	A Psuedo KMS

-	A Pseudo GMS and IdM

-	Pseudo MCX Server(s)

-	Pseudo MC clients

These components are shown in Figure 5.10.2-2.

Figure 5.10.3.1-1: Components of a MC Security Gateway (SeGy)

#### 5.10.3.2	Pseudo KMS

The SeGy contains a KMS function. This establishes the SeGy as containing its own security domain (Security Domain X). The Pseudo KMS does not perform key management functions with any clients, but allows the SeGy to represent external system functions and users as members of the SeGy’s security domain within the MC System.

The Pseudo KMS shall cross-sign with KMSs in partner domains that use the SeGy. This means that the SeGy’s KMS Certificate shall be provided to the KMS in an external domain (and vice-versa). As a consequence of cross-signing, users in partner security domains will be able to securely communicate with (users represented by) the SeGy. As cross-signing is a manual process, no communication is required between KMSs in partner domains and the SeGy’s pseudo KMS.

The SeGy shall create a KMS Certificate as defined in Annex D. The KMS Certificate generated by the SeGy shall include the information that the Certificate originates from an MC Security Gateway, and shall also define the domain of users that the SeGy represents. This type of KMS Certificate is known as a SeGy KMS Certificate. The use of a SeGy KMS Certificate ensures that MC domains and MC clients that use the SeGy are aware that a gateway is in use. It is recommended that a visual reference is provided to MC users when communicating with a user whose KMS URI corresponds to a SeGy.

In partner systems, the Pseudo KMS shall never be a Migration KMS, only an External KMS.

#### 5.10.3.3	Pseudo GMS and IdM

Should the external security domain contain groups, the MC Security Gateway will need to establish a Psuedo GMS. The GMS will perform the security functionality of a GMS towards partner MC domains. In terms of security, the GMS will create and add GMKs to SIP Notify messages sent to both MC clients and GMSs in partner domains. The SeGy will also require a Pseudo IdM to allow external clients to request an access token to request the GMS’s group document(s). Consequently, should the external security domain contain groups, the SeGy shall support all the security functionality of the GMS that is defined in this specification. Specifically the SeGy shall support:

-	Inter-domain IdM functionality defined in Clause 5.1.4 to allow access to group documents.

-	GMK distribution functionality defined in Clauses 5.7 and 7.3.

-	Verify EAR elements attached to incoming signalling messages from external security domains as defined in Clause 9.6.

-	Attach EAR elements to outgoing signalling messages as defined in Clause 9.6.

The SeGy is able to sign and encrypt messages as the GMS using key material provided by the Pseudo KMS.

#### 5.10.3.3	Pseudo MCX Server or IS Proxy

The SeGy performs the security functions of an IS Proxy (or equivalently, the MCX Server) towards external security domains. Specifically, the SeGy may:

-	Establish and use a SPK with other security domains as defined in Clauses 5.5 and 9.

-	Verify EAR elements attached to incoming signalling messages from external security domains as defined in Clause 9.6.

-	Attach EAR elements to outgoing signalling messages as defined in Clause 9.6.

The SeGy is able to sign and encrypt messages as the IS Proxy using key material provided by the Pseudo KMS.

#### 5.10.3.4	Pseudo MC clients

For each external user whose media is routed through the SeGy, the SeGy performs the security functions of an MC client on behalf of the external user. As an external user is signalled from the MC domain, or sends signalling from the external domain, the SeGy creates security credentials on behalf of the user using the Pseudo KMS. Consequently, any group keys or private calls directed towards an external user can be decrypted by the SeGy and stored for future use to decrypt the user’s communication and forward the unencrypted communication towards the external domain.

Specifically, the SeGy shall:

-	Support end-to-end security functionality for MCPTT, MCVideo and MCData defined in Clauses 7 and 8.

-	Verify EAR elements attached to incoming signalling messages from external security domains as defined in Clause 9.6.

-	Attach EAR elements to outgoing signalling messages as defined in Clause 9.6.

### 5.10.4	Interworking using a MC Security Gateway

#### 5.10.4.1	General

Interworking with Land Mobile Radio Systems is defined in TS 23.283. An interworking function (IWF) is required to allow the MC System to interwork with Land Mobile Radio Systems.

#### 5.10.4.2	MC Secuirty Gateway and the IWF

The functional model for the IWF is shown in Figure 5.10.4.2-1. Where the IWF terminates the security of the MC Domain, the IWF contains a SeGy for terminating the security of the MC domain at the IWF. The IWF can then process the unencrypted signalling and media for use in the Land Mobile Radio System.

Figure 5.10.4.2-1: Functional model for application plane for interworking