# 3GPP TSG SA WG3 (Security) Meeting #89	S3-173505

# 27 November- 1 December, 2017, Reno (US)	revision of S3-173492

# Source:	Huawei, Hisilicon, CATR, China Unicom

# Title:	Threats and requirement to KI#1

# Document for:	Approval

Agenda Item:	8.4 - FS_ NETSLICE-MGT_Sec

# 1	Decision/action requested

This contribution adds threats and requirements to the key issue #1 on interface security for TR 33.811

# 2	References

[1]	3GPP TR 28.801 V2.0.1 (2017-09), "Study on management and orchestration of network slicing for next generation network"

# 3	Rationale

The descrption of the key issue KI#1 on “unauhorized access to management exporsure interface” was agreed in SA3 #88bis meeting in Singapore. The threats and potential security requirement sections are still missing. This contribution proposes text for the above two sections.

# 4	Detailed proposal

pCR

***	BEGINNING of CHANGES	***

## 5.1	Key issue #1: Unauthorized access to Management Exposure Interface

Editor’s note: This key issue needs to be updated according to TS 28.530 and TS 28.531.

### 5.1.1	Key issue details

Network slicing allows operators to offer customised services to customers. It is possible for 5G systems, based on operators’ policies, to provide standardized APIs to creation, modification, deletion, monitoring, and update of services of network slices [4].

A management interface between the Network Slice Management Function (NSMF) and the Communication Service Management Function (CSMF) is described in [5] for this purpose.

The CSMF is responsible for translating the service related requirement to network slice related requirements and inform the NSMF of the operator’s network through the management interface. For example, a customer may want to have a specific network slice instance created by the operator. He will inform the operator’s network through CSMF and the management exposure interface, the NSMF of the operator network may create the network slice instance for the customer accordingly. This feature may be charged.

Other than the network instance creation, the operator may allow activation /deactivation, modification of configuration, deletion, and/or monitoring of a network slice instance.

This management interface will need to be secured so that only authorized parties can create, alter, and delete network slice instances. Without secure protection, an attacker could:

-	use charged for services in an unauthorized way

-	create a network slice instance to deny services to or track customers who expect to use a specific network instance

-	deny services to customers using an existing slice instance by modifying slice services

-	perform a man-in-the-middle attack by modifying a slice instance to reroute the traffic maliciously

-	deny services by deleting a slice instance

Attackers could gain access to this interface,

-	if it is not secured

-	if they can pretend to be a genuine network manager

-	if they are able to replay a valid message

-	if they can modify a valid message undetected

### 5.1.2	Security threats

If the interface is not secured, attackers may gain access to capabilities for the network management without authorization. Attackers may create network slice instances requiring significant network resources or a large number of network slice instances to exhause the network resources and potentially bring down the network. Attackers may also modify the configuration of other customers’ slice instances to fail their SLA. Attackers could replay management messages causing repeated management operations (e.g. creating duplicated network slices) and false charging etc.

### 5.1.3	Potential security requirements

- A communication service customer shall be authenticated by the network before accessing to the slice management interface.

- A communication service customer shall authenticate the network before accessing to the slice management interface.

- The slice management interface shall only be accessed by authorized communication service customers.

- The management capabilities that a communication service customer is allowed to use are defined by the HPLMN.

- The slice management interface shall be designed securely to ensure that security features cannot be bypassed.

- It shall be possible to integrity protect the slice management interface messages.

- It shall be possible to confidentiality protect the slice management interface messages.

- It shall be possible to protect the slice management interface messages against replay attacks.

***	END OF CHANGES	***