# 3GPP TSG SA WG3 (Security) Meeting #91-Bis	S3-181863

# 21 – 25 May, 2018, La Jolla, US

# Source:	China Mobile,Huawei

# Title:	Addition of authorization for slice management interface solution

# Document for:	Approval

# Agenda Item:	8.3 - FS_ NETSLICE-MGT_Sec

# 1	Decision/action requested

SA3 is kindly requested to accept the proposed evaluation as in section 6 into TR33.811 v0.5.0.

# 2	References

[1]			3GPP TR 33.811 v 0.5.0: “Study on security aspect of 5G network management slicing”.

# 3	Rationale

Add authorization for slice management interface solution for the TR 33.811 [1] solution #1.1.

# 4	Detailed proposal

*************** Start of Change 1 ****************

## 6.1				Solution #1.1: Security procedures for Management Exposure Interface

### 6.1.1	Introduction

This solution addresses key issue of #1 unauthorized access to management exposure interface.

### 6.1.2	Solution details

#### 6.1.2.1	Security within operator’s trust domain

It is up to the operator’s security policy to protect the interface.

#### 6.1.2.2	Security when CSMF is outside the operator’s trust domain

##### 6.1.2.2.1	Mutual authentication

Mutual authentication is performed between CSMF and NSMF based on client and server certificate using TLS [6], or based on pre-shared key using TLS-PSK [7].

Certificate based authentication follows the profiles given in 3GPP TS 33.310 [8], clauses 6.1.3a and 6.1.4a. The structure of the PKI used for the certificate is out of scope of the present document. The key distribution of PSK is up to the operator’s security policy and out of scope of the present document.

##### 6.1.2.2.2	Security profiles

TLS is used to provide integrity protection, replay protection and confidentiality protection for the management exposure interface. The support of TLS on the management exposure interface is mandatory.

##### 6.1.2.2.3	Authorization of CSMF’s slicing management service requests

After the mutual authentication, NSMF determines whether the CSMF is authorized to send requests through management interface.  One option of the authorization mechanism is to check the local policy. Another option is that OAuth 2.0 [9] can be used as the authorization mechanism.

### 6.1.3	Evaluation

*************** END of Change 1 ****************