3GPP TSG-SA WG3 Meeting #91-Bis 	S3-181765

# La Jolla (US), 21-25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0193 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Key distribution and derivation scheme (Keys in the UE) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2018-05-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The input parameters for derivation of KAMF, KAMF’, KgNB and KN3IWF that have been added in TS 33.501 Annex A have not been reflected in the figures in Clause 6.2.2 Key derivation and distribution scheme. Also, the algorithm type distinguishers are not aligned with those agreed in Annex A.8. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Figure 6.2.2-2 is updated based on the input parameters agreed in Annex A. Also, the algorithm type distinguishers have been aligned with those agreed in Annex A.8. Other minor editorials have been made. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete figure in TS 33.501 Clause 6.2.2.2 Key derivation and distribution scheme. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Start of Changes *****

#### 6.2.2.2	Keys in the UE

For every key in a network entity, there is a corresponding key in the UE.

Figure 6.2.2-2 shows the corresponding relations and derivations as performed in the UE.

Editor's note: Figure needs to be updated according to the determination of the keys which should be stored in USIM.

Editor's note: Figure needs to be updated according to the decision of KAUSF storage in USIM.

Figure 6.2.2-2: Key distribution and key derivation scheme for 5G for the UE

Keys in the USIM

The USIM shall store the same long-term key K that is stored in the ARPF.

During an authentication and key agreement procedure, the USIM shall generate key material from K that it forwards to the ME.

If provisioned by the home operator, the USIM shall store the home network public key used for concealing the SUPI.

Keys in the ME

The ME shall generate the KAUSF from the CK, IK received from the USIM. The generation of this key material is specific to the authentication method and is specified in clause 6.1.3.

When 5G AKA is used, the generation of RES* from RES shall be performed by the ME.

Storage of the KAUSF at the UE is optional. If the USIM supports 5G parameters storage, KAUSF  shall be stored in the USIM. Otherwise, KAUSF shall be stored in the non-volatile memory of the ME.

Editor's Note: It is FFS to define the event(s) that triggers the storage of KAUSF in the ME or in the USIM.

The ME shall perform the generation of KSEAF from the KAUSF . If the USIM supports 5G parameters storage, KSEAF shall be stored in the USIM. Otherwise, KSEAF shall be stored in the non-volatile memory of the ME.

Editor's Note: It is FFS to define the event(s) that triggers the storage of KSEAF in the ME or in the USIM.

The ME shall perform the generation of KAMF. If the USIM supports 5G parameters storage, KAMF shall be stored in the USIM. Otherwise, KAMF shall be stored in the non-volatile memory of the ME.

Editor's Note: It is FFS to define the event(s) that triggers the storage of KAMF in the ME or in the USIM.

The ME shall perform the generation of all other subsequent keys that are derived from the KAMF.

Editor's Note: It is FFS whether any subsequent keys that are derived from the KAMF needs to be stored at ME and if so, which keys.

Any 5G security context, KAUSF and KSEAF that are stored at the ME shall be deleted from the ME if:

a)	the USIM is removed from the ME when the ME is in power on state;

b)	the ME is powered up and the ME discovers that the USIM is different from the one which was used to create the 5G security context;

c)	the ME is powered up and the ME discovers that there is no USIM is present at the ME.

***** End of Changes *****