3GPP TSG-SA WG3 Meeting #91BIS 	S3-181797

# La Jolla (US), 21-25 MAY 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 199 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorilal Corrections to 33.501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2018-05-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are some editorial errors needed to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fixes to format (subscripts, format) Correction of typos Fixing the refernce section nummbers |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Eidtorial errors are still there |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.3, 5.11.2, 6.1.1.2, 6.2.2.2, 6.4.2.1, 6.4.3.1, 6.4.3.2, 9.2, 9.3, 9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

5.9.3	Requirements for e2e core network interconnection security

5.9.3.1	General

The present sub-clause contains requirements common to sub-clauses 5.9.2 and 5.9.3.

A solution for e2e core network interconnection security shall satisfy the following requirements.

The solution shall support application layer mechanisms for addition, deletion and modification of message elements by intermediate nodes except for specific message elements described in the present document.

NOTE:	Typical example for such a case is IPX providers modifying messages for routing purposes.

The solution shall provide confidentiality and/or integrity end-to-end between source and destination network for specific message elements identified in the present document. For this requirement to be fulfilled, the SEPP – cf [2], clause 6.2.17 shall be present at the edge of the source and destination networks dedicated to handling e2e Core Network Interconnection Security. The confidentiality and/or integrity for the message elements is provided between two SEPPs of the source and destination PLMN.

**** Next Changes ****

5.11.2	Requirements for algorithm selection

a)	UE in RRC_Connected and a serving network shall have agreed upon algorithms for

-	Ciphering and integrity protection of RRC signalling and user plane (to be used between UE and gNB)

-	Ciphering and integrity protection of RRC signalling and ciphering of user plane (to be used between UE and ng-eNB)

-	NAS ciphering and NAS integrity protection (to be used between UE and AMF)

b)	The serving network shall select the algorithms to use dependent on

-	the UE security capabilities of the UE,

-	the configured allowed list of security capabilities of the currently serving network entity

c)	The UE security capabilities shall include NR NAS algorithms for NAS level, NR algorithms for AS level and LTE algorithms for AS level if the UE supports E-UTRAN connected to 5GC.

**** Next Changes ****

6.1.1.2	EAP framework

The EAP framework is specified in RFC 3748 [27]. It defines the following roles: peer, pass-through authenticator and back-end authentication server. The back-end authentication server acts as the EAP server, which terminates the EAP authentication method with the peer. In the 5G system, when EAP-AKA' is used, the EAP framework is supported in the following way:

-	The UE takes the role of the peer.

-	The SEAF takes the role of pass-through authenticator.

-	The AUSF takes the role of the back-end authentication server.

**** Next Changes ****

6.2.2.2	Keys in the UE

For every key in a network entity, there is a corresponding key in the UE.

Figure 6.2.2-2 shows the corresponding relations and derivations as performed in the UE.

Editor's note: Figure needs to be updated according to the determination of the keys which should be stored in USIM.

Editor's note: Figure needs to be updated according to the decision of KAUSF storage in USIM.

Figure 6.2.2-2: Key distribution and key derivation scheme for 5G for the UE

Keys in the USIM

The USIM shall store the same long-term key K that is stored in the ARPF.

During an authentication and key agreement procedure, the USIM shall generate key material from K that it forwards to the ME.

If provisioned by the home operator, the USIM shall store the home network public key used for concealing the SUPI.

Keys in the ME

The ME shall generate the KAUSF from the CK, IK received from the USIM. The generation of this key material is specific to the authentication method and is specified in clause 6.1.3.

When 5G AKA is used, the generation of RES* from RES shall be performed by the ME.

Storage of the KAUSF at the UE is optional. If the USIM supports 5G parameters storage, KAUSF shall be stored in the USIM. Otherwise, KAUSF shall be stored in the non-volatile memory of the ME.

Editor's Note: It is FFS to define the event(s) that triggers the storage of KAUSF in the ME or in the USIM.

The ME shall perform the generation of KSEAF from the KAUSF. If the USIM supports 5G parameters storage, KSEAF shall be stored in the USIM. Otherwise, KSEAF shall be stored in the non-volatile memory of the ME.

Editor's Note: It is FFS to define the event(s) that triggers the storage of KSEAF in the ME or in the USIM.

The ME shall perform the generation of KAMF. If the USIM supports 5G parameters storage, KAMF shall be stored in the USIM. Otherwise, KAMF shall be stored in the non-volatile memory of the ME.

Editor's Note: It is FFS to define the event(s) that triggers the storage of KAMF in the ME or in the USIM.

The ME shall perform the generation of all other subsequent keys that are derived from the KAMF.

Editor's Note: It is FFS whether any subsequent keys that are derived from the KAMF needs to be stored at ME and if so, which keys.

Any 5G security context, KAUSF and KSEAF that are stored at the ME shall be deleted from the ME if:

a)	the USIM is removed from the ME when the ME is in power on state;

b)	the ME is powered up and the ME discovers that the USIM is different from the one which was used to create the 5G security context;

c)	the ME is powered up and the ME discovers that there is no USIM is present at the ME.

**** Next Changes ****

6.4.2.1	Multiple active NAS connections with different PLMNs

TS 23.501 [2] has a scenario when the UE is registered to a VPLMN's serving network via 3GPP access and to another VPLMN's or HPLMN's serving network via non-3GPP access at the same time. When the UE is registered in one PLMN's serving network over a certain type of access (e.g. 3GPP) and is registered to another PLMN's serving network over another type of access (e.g. non-3GPP), then the UE has two active NAS connections with different AMF's in different PLMNs. As described in clause 6.3.2.1, the UE shall independently maintain and use two different 5G security contexts, one per PLMN serving network. Each security context shall be established separately via a successful primary authentication procedure with the Home PLMN. All the NAS and AS security mechanisms defined for single registration mode are applicable independently on each access using the corresponding 5G security context.

**** Next Changes ****

6.4.3.1	NAS input parameters to integrity algorithm

The input parameters to the NAS 128-bit integrity algorithms as described in Annex D shall be set as follows.

The KEY input shall be equal to the key KNASint.

The BEARER input shall be equal to the NAS connection identifier.

The DIRECTION bit shall be set to 0 for uplink and 1 for downlink.

The COUNT input shall be constructed as follows:

**** Next Changes ****

6.4.3.2	NAS integrity activation

NAS integrity shall be activated using the NAS SMC procedure or after an inter-system handover from EPC.

Replay protection shall be activated when integrity protection is activated, except when the NULL integrity protection algorithm is selected. Replay protection shall ensure that the receiver only accepts each incoming NAS COUNT value once using the same NAS security context.

Once NAS integrity has been activated, NAS messages without integrity protection shall not be accepted by the UE or the AMF. Before NAS integrity has been activated, NAS messages without integrity protection shall only be accepted by the UE or the AMF in certain cases where it is not possible to apply integrity protection.

NAS integrity shall stay activated until the 5G security context is deleted in either the UE or the AMF. It shall not be possible to change from non-NULL integrity protection  to NULL integrity protection.

**** Next Changes ****

9.2	Security requirements and procedures on N2

Editor's Note: includes equivalent of S1-MME messages, like key transport to base station, as well as message protection, i.e. backhaul link security)

N2 is the reference point between the AMF and the 5G-AN. It is used, among other things, to carry NAS signalling traffic between the UE and the AMF over 3GPP and non-3GPP accesses.

The transport of control plane data over N2 shall be integrity, confidentiality and replay-protected.

In order to protect the N2 reference point, it is required to implement IPsec ESP and IKEv2 certificates based authentication as specified in sub-clause  9.5.1of the present document. IPsec is mandatory to implement on the gNB. On the core network side, a SEG may be used to terminate the IPsec tunnel.

NOTE: 	The use of cryptographic solutions to protect N2 is an operator's decision. In case the gNB has been placed in a physically secured environment then the 'secure environment' includes other nodes and links beside the gNB.

Editor's Note: The implementation requirement on the 5G-RAN side in general and the gNB in particular is FFS.

9.3	Security requirements and procedures on N3

N3 is the reference point between the 5G-AN and UPF. It is used to carry user plane data from the UE to the UPF.

The transport of user data over N3 shall be integrity, confidentiality and replay-protected.

In order to protect the traffic on the N3 reference point, it is required to implement IPsec ESP and IKEv2 certificates based authentication as specified in sub-clause 9.5.1 of the present document with confidentiality, integrity and replay protection. IPsec is mandatory to implement on the gNB. On the core network side, a SEG may be used to terminate the IPsec tunnel.

NOTE: 	The use of cryptographic solutions to protect N3 is an operator's decision. In case the gNB has been placed in a physically secured environment then the 'secure environment' includes other nodes and links beside the gNB.

Editor's Note: The implementation requirement on the 5G-RAN side in general and the gNB in particular is FFS.

QoS related aspects are further described in sub-clause 9.5.1 of the present document.

9.4	Security requirements and procedures on Xn

Xn is the interface connecting 5G-RAN nodes. It is used to carry signalling and user plane data.

The transport of control plane data and user data over Xn shall be integrity, confidentiality and replay-protected.

In order to protect the traffic on the Xn reference point, it is required to implement IPsec ESP and IKEv2 certificates based authentication as specified in sub-clause 9.5.1 of the present document with confidentiality, integrity and replay protection. IPsec shall be supported on the gNB.

NOTE: 	The use of cryptographic solutions to protect Xn is an operator's decision. In case the gNB has been placed in a physically secured environment then the 'secure environment' includes other nodes and links beside the gNB.

QoS related aspects are further described in sub-clause 9.5.1 of the present document.

9.5	Interfaces based on DIAMETER or GTP

9.5.1	Protection at the network layer

Editor's Note: Solutions in this sub-clause may apply, in particular, when there are no intermediaries modifying messages, e.g. in intra-domain situations.

Here, the solution from EPS can be adapted to use in 5G.

The protection of IP based interfaces for 5GC and 5G-AN shall be supported according to NDS/IP as specified in TS 33.210 [3]. Traffic on interfaces carrying control plane signalling shall be integrity and replay protected according to NDS/IP. In addition to the mandatory integrity protection, traffic carrying subscriber specific sensitive data, e.g. cryptographic keys, shall be also confidentiality protected according to NDS/IP.

NOTE:	According to TS 33.210 [3] and TS 33.310 [5], in case control plane interfaces are trusted, then the use of protection based cryptographic mechanisms are not needed.Whenever IPsec ESP is required, then it shall be implemented according to RFC 4303 [4] as profiled by TS 33.210 [3]. For IPsec implementation, tunnel mode is mandatory to implement while transport mode is optional.

Whenever IKEv2 certificates based authentication is required, then it shall be implemented according to TS 33.310 [5]. The certificates shall be implemented according to the profile described in TS 33.310 [5]. IKEv2 shall be implemented conforming to the IKEv2 profile described in TS 33.310 [5].

QoS considerations

if the sender of IPsec traffic uses DiffServ Code Points (DSCPs) to distinguish different QoS classes, either by copying DSCP from the inner IP header or directly setting the encapsulating IP header's DSCP, the resulting traffic may be reordered to the point where the receiving node's anti-replay check discards the packet. If different DSCPs are used on the encapsulating IP header, then to avoid packet discard under one IKE SA and with the same set of traffic selectors, distinct Child-SAs should be established for each of the traffic classes (using the DSCPs as classifiers) as specified in RFC 4301 [6].

Editor's Note: This might need to be revised depending on the progress on the QoS architecture in other working groups.

Editor’s Note: Intro of  “9.5.2	Protection at the application layer“ is  ffs with the 2 following ENs addressed if clause is added.

Editor's Note: Solutions in this sub-clause may apply, in particular, when there may be intermediaries modifying messages, e.g. in roaming situations.

Editor's Note: This sub-clause is to include solutions satisfying the requirements on e2e security in clause 5.9. It is to take into account the work performed by GSMA FASG DESS on e2e security for selected DIAMETER AVPs. It is to also take into account solutions 10.1 and 10.2 in clause 5.10.4 of TR 33.899. When the solution(s) involve a Public Key Infrastructure then details of the use of the PKI are to be provided, e.g. by reference to TS 33.310.

Editor's Note: it is ffs whether a sub-clause „9.5.3	Authorization aspects“ is needed. It was implicit in NDS/IP in earlier generations, but application layer protection may entail new aspects.

**** END of Changes ****