# 3GPP TSG SA WG3 (Security) Meeting #91Bis	S3-181683

# 21- 25 May 2018, La Jolla (USA)	revision of S3-17xabc

# Source:	 Nokia

# Title: 	Discussion paper: EN on Simultaneous activation of NAS context

# Document for:	Discussion and endorsement

Agenda Item:	7.2.5

# 1	Decision/action requested

Endorse the recommendations

# 2	References

[1]	TS 33.501

# 3	Rationale

In TS 33.501 6.4.2, existing Editor’s Note:

“6.4.2	Security for multiple NAS connections

Editor's Note: It is FFS if an explicit indication is required to signal whether to activate a newly created partial NAS security context simultaneously on all the NAS connections or only the current one. “

# 4 	Discussion

1. For clause 6.9.5.2 (S3-181566) we have agreed that,

“1.	The SEAF/AMF shall not initiate a primary authentication or NAS SMC procedure in case a primary authentication or a NAS SMC procedure is ongoing on a parallel NAS connection. Authentication procedures followed by a NAS SMC procedures taking the new 5G security context into use, shall be performed on one NAS signalling connection at a time.

2.	When the AMF has sent a NAS Security Mode Command to a UE in order to take a new KAMF into use and receives a context transfer request for the UE from another AMF, the AMF shall wait for the completion of the NAS SMC procedure before transferring the context. “

2. In mobility, when a UE moves from one gNB to another gNB or one N3IWF to another, the N2 message for the handover is not going to be concurrent, it will always be one by one. Mobility in one connection cannot automatically assume mobility for another connection.

3. This means UE has to indicate mobility for each connection separately to the AMF.

4. Hence a newly created partial NAS security context can not be activated simultaneously to multiple connections automatically.

5. If there are multiple NAS connections active at an AMF, then during mobility in a target AMF if same 3gpp and non3gpp connections to be activated, one connection cannot indicate the AN node (gNB or N3IWF) required for the AN connection.

5 	Proposal

Delete the Editor’s Note. It doesn’t look feasible to activate a newly created partial NAS security context simultaneously on all the NAS connections. Clarify that a newly created partial NAS security context is activated only on the NAS connection reporting the mobility, NAS context is enabled one by one for multiple connections. Corresponding CR is S3-181684