3GPP TSG-SA WG3 Meeting #91Bis 	S3-181843

# La Jolla (US), 21-25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 33.501 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections and clarification to handover from 5GS to EPS over N26 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2018-05-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This change is based on living CR in S3-181329. The procedure for IW HO from 5GS to EPS needs to be corrected and clarified. The target MME could be a legacy MME. According to TS 29.274, which describes Forward Relocation Request message sent from source MME to target MME on S10 interface, the MM context is included in Forward Relocation Request message from source MME to target MME. The MM context used to transfer a EPS security context is described in Figure 8.38-5 in TS 29.274. The security parameters transferred are: Kasme, NH, NCC, used NAS integrity algorithm, used NAS encryption algorithm. Source MME does not transfer the EPS NAS keys to the target MME. It is therefore proposed that target MME derives the EPS NAS keys from the newly derived K'ASME key received from source AMF, instead of source AMF deriving the EPS NAS keys. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following corrections and clarifications are proposed: - in step2, a need to align with clause 8.6.1 which now says that NAS COUNTs are set to 0. Therefore, a reference to clause 8.6.1 is added. - in step 9, a need to align with clause 8.6.1 which now says that NAS COUNTs are set to 0. Therefore, a reference to clause 8.6.1 is added. - in step 2, delete the sentence about source AMF deriving the EPS NAS keys from K'ASME key. - in step 4, add that target MME derives the EPS NAS keys from the received K'ASME key. - some editorial corrections and duplicate text in clause 8.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

************* START OF CHANGES *************

**** Change 1 (based on Living CR S3-181329) ****

### 8.3.2	Procedure

Figure 8.3.2-1 Handover from 5GS to EPC over N26

NOTE:	This procedure is based on clause 4.11.1.2.1 in TS 23.502 and only includes steps and description that are relevant to security.

If the UE is initially registered and connected to the 5GC, the 5GC has a current security context for the UE. The current 5G security context may be a mapped 5G security context resulting from a previous mobility from EPC, or a native 5G security context resulting from a primary authentication with the 5GC.

1.	The gNB sends a Handover Required message to the AMF, including UE’s identity and UE’s security capabilities.

2.	When the source AMF performs a handover procedure to the EPC, after checking the UE's access rights and security capabilities, the source AMF shall prepare a UE context including a mapped EPS security context for the target MME. To construct the mapped EPS security context, the source AMF shall derive a K’ASME using the KAMF key and the current downlink 5G NAS COUNT of the current 5G security context as described in clause 8.6.1 and then increments its stored downlink 5G NAS COUNT value by one.

The source AMF shall select the EPS NAS algorithms identifiers (it has stored) to be used in the target MME at interworking handover to EPS, for encryption and integrity protection.

NOTE 1: 	A legacy target MME is expecting to receive the selected EPS NAS algorithms identifiers over N26 from the source AMF as the target MME belives the source AMF is another MME. The source AMF has therefore provisioned the EPS NAS security algorithms identifiers to be used at interworking handover to EPS to the UE in the 5G NAS SMC in 5G access as described in clause 6.7.2.  The target MME could re-select different EPS NAS algorithms though to be used with the UE by running a NAS SMC in the following Tracking Area Update procedure.

The uplink and downlink EPS NAS COUNT associated with the newly derived K'ASME key  are set to the values as described in clause 8.6.1. The eKSI for the newly derived K’ASME key is defined as described in clause 8.6.1.

The source AMF shall also derive the initial KeNB key from the K'ASME key and the uplink NAS COUNT as specified in Annex A.3 of TS 33.401 [10] using 232-1 as the value of the uplink NAS COUNT parameter.

NOTE 2:	The source AMF and the UE only uses the 232-1 as the value of the uplink NAS COUNT for the purpose of deriving KeNB and do not actually set the uplink NAS COUNT to 232-1. The reason for choosing such a value not in the normal NAS COUNT range, i.e., [0, 224-1] is to avoid any possibility that the value may be used to derive the same KeNB again.

The source AMF subsequently derives NH two times as specified in clause A.4 of TS 33.401 [10]. The {NH, NCC=2} pair is provided to the target MME as a part of UE security context in the Relocation Request message.

3.	The source AMF shall transfer the UE security context (including new K'ASME, , eKSI, uplink and downlink EPS NAS COUNT’s, UE EPS security capabilities, selected EPS NAS algorithms identifiers) to the target MME in the Relocation Request message. The UE NR security capabilities may be sent by the source AMF as well.

4.	When the target MME receives Relocation Request message from source AMF, then the target MME shall derive EPS NAS keys (i.e., KNASenc and KNASint) from the received K'ASME key with the received EPS NAS security algorithm identifiers as input, to be used in EPC as described in Annex A.7 in TS 33.401 [10]. The target MME needs to include the {NH, NCC=2} pair and the UE security capabilities in the S1 HANDOVER REQUEST message to the target LTE eNB. The UE security capabilities include the UE EPS security capabilities received from the source AMF.

5.	Upon receipt of the S1 HANDOVER REQUEST from the target MME, the target LTE eNB shall compute the KeNB to be used with the UE and proceed as described in clause 7.2.8.4.3 in TS 33.401[10].

6.	The target MME shall includes the target to source transparent container received from the target LTE eNB in the Relocation Response message sent to the source AMF.

7.	The source AMF shall include the target to source transparent container in the Handover command sent to the source gNB.

8.	The source gNB shall include the target to source transparent container in the Handover command sent to the UE.

Upon the reception of the Handover Command message, the UE shall derive the mapped EPS security context, i.e. derive  K'ASME from KAMF as described in clause 8.6.1 using the current downlink 5G NAS COUNT. After the derivation the UE shall increment its stored downlink 5G NAS COUNT value by one.

9.	 The eKSI for the newly derived K'ASME key is defined as described in clause 8.6.1. The UE shall also derive the EPS NAS keys (i.e. KNASenc and KNASint) as the MME did in step 4 using the EPS NAS security algorithms identifiers stored in the ME and provisioned by the AMF to the UE in 5G NAS SMC in earlier 5G access. The UE shall also derive the initial KeNB from the K'ASME and the uplink  NAS COUNT as specified in Annex A.3 of TS 33.401 [10] using 232-1 as the value of the uplink NAS COUNT parameter.

The UE shall also derive the {NH, NCC=2} pair as described in A.4 of TS 33.401 [10] and further derive the KeNB to be used with the UE by performing the key derivation defined in Annex A.5 in TS 33.401[10]. The UE shall derive the AS RRC keys and the AS UP keys based on the KeNB and the received AS EPS security algorithms identifiers selected by the target eNB as described in Annex A.7 in TS 33.401 [10]. The uplink and downlink EPS NAS COUNT associated with the derived EPS NAS keys are set to  the values as described in clause 8.6.1. The UE shall immediately take into use the newly created mapped EPS security context, both for NAS and AS communication.

************** END OF CHANGES **************