# 3GPP TSG SA WG3 (Security) Meeting #91	S3-181180

# 16 - 20 April 2018, Belgrade (RS)	revision of S3-180838

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.401 | CR | 0655 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the Subscription Concealed Identifier to EPC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  |  | Date: |  | 2018-04-08 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Since the permanent identity (IMSI) never changes, revealing it would help to gather information on the identity and location of the user. It is important to provide an identity privacy support mechanism that ensures the UE never reveals the IMSI to passive or active attackers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR makes use of asymmetric cryptography to provide for the confidentiality of the user’s permanent identity (IMSI) in transit. The solution recommends to use ECIES to protect the permanent identity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE can reveal the IMSI to passive or active attackers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.3, 5.1.1, 6.1.3, 7.1, 15.1, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR … |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Next change *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.003: "Numbering, addressing and identification".

[4]	3GPP TS 33.102: "3G security; Security architecture".

[5]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[6]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[7]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[8]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[9]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[10] – [11]	Void.

[12]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification"

[13]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[14]	3GPP TS 35.215: "Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications"

[15]	NIST: "Advanced Encryption Standard (AES) (FIPS PUB 197) "

[16]	NIST Special Publication 800-38A (2001): "Recommendation for Block Cipher Modes of Operation".

[17]	NIST Special Publication 800-38B (2001): "Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication".

[18] – [20]	Void.

[21]	3GPP TS 36.331:"Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol specification".

[22]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[23]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Service aspects; Service principles".

[24]	3GPP TS 25.331: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Radio Resource Control (RRC); Protocol Specification ".

[25]	3GPP TS 44.060: "3rd Generation Partnership Project; Technical Specification Group GSM/EDGE Radio Access Network; General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol.

[26]	3GPP TS 23.122: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[27]	3GPP TS 33.320: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Security of Home Node B (HNB) / Home evolved Node B (HeNB)".

[28]	(void)

[29]	ETSI TS  102 484 V10.0.0: "Smart Cards; Secure channel between a UICC and an end-point terminal".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 31.116 "Remote APDU Structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications".

[32]	ETSI TS 102 221 V9.2.0: "Smart Cards; UICC-Terminal interface; Physical and logical characteristics".

[33]	3GPP TS 35.221: "Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 1: EEA3 and EIA3 specifications".

[34]	RFC 4301: "Security Architecture for the Internet Protocol".

[35]	3GPP TS 22.346: "Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1".

[36]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[37]	3GPP TS.33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[38]	IETF RFC 7296: " Internet Key Exchange Protocol Version 2 (IKEv2)".

[39]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications, IEEE Std.".

[40]	3GPP TS 36.463: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Wireless LAN (WLAN); Xw application protocol (XwAP)".

[41]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[42]	3GPP TS 38.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[aa]	SECG SEC 1: Recommended Elliptic Curve Cryptography, Version 2.0, 2009. Available http://www.secg.org/sec1-v2.pdf

[bb]	SECG SEC 2: Recommended Elliptic Curve Domain Parameters, Version 2.0, 2010. Available at http://www.secg.org/sec2-v2.pdf

[cc]                      IETF RFC 7748: "Elliptic Curves for Security".

\***** Next change *****

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 33.102 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Access Security Management Entity: entity which receives the top-level keys in an access network from the HSS. For E-UTRAN access networks, the role of the ASME is assumed by the MME

Activation of security context: the process of taking into use a security context.

Authentication data: Data that is part of a security context or of authentication vectors.

Chaining of KeNB: derivation of a new KeNB from another KeNB (i.e., at cell handover)

Current EPS security context: The security context which has been activated most recently. Note that a current EPS security context originating from either a mapped or native EPS security context may exist simultaneously with a native non-current EPS security context.

ECM-CONNECTED state: This is as defined in TS 23.401 [2]. The term ECM-CONNECTED state corresponds to the term EMM-CONNECTED mode used in TS 24.301 [9].

ECM-IDLE state: As defined in TS 23.401 [2]. The term ECM-IDLE state corresponds to the term EMM-IDLE mode used in TS 24.301 [9].

EPS-Authentication Vector: KASME, RAND, AUTN, XRES

EPS security context: A state that is established locally at the UE and a serving network domain. At both ends "EPS security context data" is stored, that consists of the EPS NAS security context, and the EPS AS security context.

NOTE 1:	An EPS security context has type "mapped", "full native" or "partial native". Its state can either be "current" or "non-current". A context can be of one type only and be in one state at a time. The state of a particular context type can change over time. A partial native context can be transformed into a full native. No other type transformations are possible.

EPS AS security context: the cryptographic keys at AS level with their identifiers, the Next Hop parameter NH, the Next Hop Chaining Counter parameter NCC used for next hop access key derivation, the identifiers of the selected AS level cryptographic algorithms, counters used for replay protection and SCG Counter used as freshness input into S-KeNB derivations. Note that the EPS AS security context only exists when cryptographically protected radio bearers are established and is otherwise void.

NOTE 2: NH and NCC need to be stored also at the MME during connected mode.

EPS AS Secondary Cell security context: This context consists of the cryptographic keys for SeNB (KUPenc), the identifier of the selected AS SC level cryptographic algorithm and counters used for replay protection.

EPS NAS security context: This context consists of KASME with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values. In particular, separate pairs of NAS COUNT values are used for each EPS NAS security contexts, respectively. The distinction between native and mapped EPS security contexts also applies to EPS NAS security contexts. The EPS NAS security context is called "full" if it additionally contains the keys KNASint and KNASenc and the identifiers of the selected NAS integrity and encryption algorithms.

Full native EPS security context: A native EPS security context for which the EPS NAS security context is full according to the above definition. A full native EPS security context is either in state "current" or state "non-current".

Forward security: In the context of KeNB key derivation, forward security refers to the property that, for an eNB with knowledge of a KeNB, shared with a UE, it shall be computationally infeasible to predict any future KeNB, that will be used between the same UE and another eNB. More specifically, n hop forward security refers to the property that an eNB is unable to compute keys that will be used between a UE and another eNB to which the UE is connected after n or more handovers (n=1 or 2).

Legacy security context: A security context which has been established according to TS 33.102 [4].

Mapped security context: Security context created by converting the current security context in the source system to a security context for the target system in inter-system mobility, e.g., UMTS keys created from EPS keys. The EPS NAS security context of a mapped security context is full and current.

Native EPS security context: An EPS security context whose KASME was created by a run of EPS AKA.

Non-current EPS security context: A native EPS security context that is not the current one. A non-current EPS security context may be stored along with a current EPS security context in the UE and the MME. A non-current EPS security context does not contain an EPS AS security context. A non-current EPS security context is either of type "full native" or of type "partial native".

Partial native EPS security context: A partial native EPS security context consists of KASME with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values, which are initially set to zero before the first NAS SMC procedure for this security context. A partial native EPS security context is created by an EPS AKA, for which no corresponding successful NAS SMC has been run. A partial native context is always in state "non-current".

Re-derivation of NAS keys: derivation of new NAS keys from the same KASME but including different algorithms (and no freshness parameter)

Refresh of KeNB: derivation of a new KeNB from the same KASME and including a freshness parameter

Re-keying of KeNB: derivation of a new KeNB from a new KASME in ECM-CONNECTED (i.e., . to activate a partial native EPS security context, or to re-activate a non-current full EPS security context)

Re-keying of NAS keys: derivation of new NAS keys from a new KASME

International Mobile Concealed Identity: The IMCI is a one-time use subscription identifier, which contains the concealed subscription identifier, e.g., MSIN.

Subscription Identifier De-concealing Function: This service is offered by the network function HSS in the home network of the subscriber. The Subscription Identifier De-concealing Function (SIDF) is responsible for de-concealing the IMSI from the IMCI.

UE security capabilities: The set of identifiers corresponding to the ciphering and integrity algorithms implemented in the UE. This includes capabilities for EPS AS and NAS, and includes capabilities for UTRAN and GERAN if these access types are supported by the UE.

UE EPS security capabilities: The UE security capabilities for EPS AS and NAS.

User plane: Within the context of TS 33.401, this means the data path between UE and Serving Gateway that does NOT go via the MME.

(User) Data via MME: User Data sent to or from the UE that uses an RRC connection established using the Control Plane CIoT EPS optimisation specified in TS 23.401[2].

IOPS-capable eNB: an eNB that has the capability of IOPS mode operation, which provides local IP connectivity and Public Safety services to IOPS-enabled UEs via a Local EPC when the eNB has lost backhaul to the Macro EPC or it has no backhaul to the Macro EPC.

IOPS network: an IOPS network consists of one or more eNBs operating in IOPS mode and connected to a Local EPC.

Local EPC: a Local EPC is an entity which provides functionality that eNBs in IOPS mode of operation use, instead of the Macro EPC, in order to support Public Safety services.

Macro EPC: the EPC which serves an eNB when it is not in IOPS mode of operation.

Nomadic EPS: a deployable system which has the capability to provide radio access (via deployable IOPS-capable eNB(s)), local IP connectivity and Public Safety services to IOPS-enabled UEs in the absence of normal EPS.IOPS-enabled UE: is an UE that is configured to use networks operating in IOPS mode.

***** Next change *****

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AES	Advanced Encryption Standard

AK	Anonymity Key

AKA	Authentication and Key Agreement

AMF	Authentication Management Field

AN	Access Network

AS	Access Stratum

AUTN	Authentication token

AV	Authentication Vector

ASME	Access Security Management Entity

Cell-ID	Cell Identity as used in TS 36.331 [21]

CK	Cipher Key

CKSN	Cipher Key Sequence Number

C-RNTI	Cell RNTI as used in TS 36.331 [21]

CRL	Certificate Revocation List

DeNB	Donor eNB

DoS	Denial of Service

DSCP	Differentiated Services Code Point

EARFCN-DL	E-UTRA Absolute Radio Frequency Channel Number-Down Link

ECM	EPS Connection Management

EEA	EPS Encryption Algorithm

EIA	EPS Integrity Algorithm

eKSI	Key Set Identifier in E-UTRAN

EMM	EPS Mobility Management

eNB	Evolved Node-B

EPC	Evolved Packet Core

EPS	Evolved Packet System

EPS-AV	EPS authentication vector

E-UTRAN	Evolved UTRAN

gNB	Next Generation Node-B

GERAN	GSM EDGE Radio Access Network

GUTI	Globally Unique Temporary Identity

HE	Home Environment

HFN	Hyper Frame Number

HO	Hand Over

HSS	Home Subscriber Server

IK	Integrity Key

IKE	Internet Key Exchange

IMEI	International Mobile Station Equipment Identity

IMEISV	International Mobile Station Equipment Identity and Software Version number

IMCI	International Mobile Concealed Identity

IMSI	International Mobile Subscriber Identity

IOPS	Isolated E-UTRAN Operation for Public Safety

IRAT	Inter-Radio Access Technology

ISR	Idle Mode Signaling Reduction

KDF	Key Derivation Function

KSI	Key Set Identifier

LWIP	LTE WLAN RAN Level Integration using IPSec

LSB	Least Significant Bit

LSM	Limited Service Mode

LWA	LTE-WLAN Aggregation

MAC-I	Message Authentication Code for Integrity (terminology of TS36.323 [12])

MACT	Message Authentication Code T used in AES CMAC calculation

MeNB	Master eNB

ME	Mobile Equipment

MME	Mobility Management Entity

MME-RN	MME serving the RN

MS	Mobile Station

MSC	Mobile Switching Center

MSIN	Mobile Station Identification Number

NAS	Non Access Stratum

NAS-MAC	Message Authentication Code for NAS for Integrity (called MAC in TS24.301 [9])

NASDVM	Non Access Stratum - Data via MME

NCC	Next hop Chaining Counter

NH	Next Hop

OCSP	Online Certificate Status Protocol

OTA	Over-The-Air (update of UICCs)

PCI	Physical Cell Identity as used in TS 36.331 [21]

PDCP	Packet Data Convergence Protocol

PLMN	Public Land Mobile Network

PRNG	Pseudo Random Number Generator

PSK	Pre-shared Key

P-TMSI	Packet- Temporary Mobile Subscriber Identity

RAND	RANDom number

RAU	Routing Area Update

RN	Relay Node

RRC	Radio Resource Control

SCG	Secondary Cell Group

SEG	Security Gateway

SGSN	Serving GPRS Support Node

SIM	Subscriber Identity Module

SMC	Security Mode Command

SeNB	Secondary eNB

SgNB	Secondary gNB

SN	Serving Network

SN id	Serving Network identity

SQN	Sequence Number

SRB	Source Route Bridge

SRVCC	Single Radio Voice Call Continuity

S-TMSI	S-Temporary Mobile Subscriber Identity

TAI	Tracking Area Identity

TAU	Tracking Area Update

UE	User Equipment

UEA	UMTS Encryption Algorithm

UIA	UMTS Integrity Algorithm

UICC	Universal Integrated Circuit Card

UMTS	Universal Mobile Telecommunication System

UP	User Plane

USIM	Universal Subscriber Identity Module

UTRAN	Universal Terrestrial Radio Access Network

WT	WLAN Termination as used in TS 36.300 [30]

XRES	Expected Response

***** Next change *****

### 5.1.1	User identity and device confidentiality

User identity confidentiality is as defined by TS 33.102 [4] subclause 5.1.1

From subscriber's privacy point of view, the MSIN, the IMEI, and the IMEISV should be confidentiality protected.

The IMSI should not be transferred in clear text over E-UTRAN except routing information, e.g. Mobile Country Code (MCC) and Mobile Network Code (MNC).

The home network public key shall be stored in the USIM.

The ME shall support the null-scheme.

If the home network has not provisioned the public key in USIM, the IMSI protection in initial registration procedure is not provided. In this case, the null-scheme shall be used by the ME.

Based on operator's decision, indicated by the USIM, the calculation of the IMCI shall be performed either by the USIM or by the ME.

NOTE 1: 	If the indication is not present, the calculation is in the ME.

In case of an unauthenticated emergency call, privacy protection for IMSI is not required.

Provisioning, and updating the home network public key in the tamper-resistant hardware shall be in the control of the home network operator.

NOTE 2:	The provisioning and updating of the home network public key is out of the scope of the present document. It can be implemented using, e.g. the Over the Air (OTA) mechanism.

Subscriber privacy enablement shall be under the control of the home network of the subscriber.

The UE shall provide its equipment identifier IMEI or IMEISV to the network, if the network asks for it in an integrity-protected request.

The IMEI and IMEISV shall be securely stored in the terminal.

The UE shall not send IMEI or IMEISV to the network on a network request before the NAS security has been activated.

NOTE 1:	When the UE has no IMSI, no valid GUTI, or no valid P-TMSI during emergency attach, the IMEI is included before the NAS security has been activated.

The IMEI or IMEISV shall be sent in the NAS protocol.

NOTE 2:	In some cases, e.g., the very first attach procedure, MSIN has to be sent to network in cleartext. When NAS confidentiality protection is beyond an operator option, IMEI and IMEISV can not be confidentiality protected.

***** Next change *****

### 6.1.3	User identification by a permanent identity

The user identification mechanism should be invoked by the serving network whenever the user cannot be identified by means of a temporary identity (GUTI). In particular, it should be used when the serving network cannot retrieve the IMSI based on the GUTI by which the user identifies itself on the radio path.

The mechanism described in figure 6.1.3-1 allows the identification of a user on the radio path by means of the concealed subscriber identity (IMCI).

Figure 6.1.3-1: User identity query

The mechanism is initiated by the MME that requests the user to send its IMCI.

The UE shall calculate a fresh IMCI from IMSI using the home network public key, and respond with Identifier Response carrying the IMCI.

NOTE 1:	If the UE is using any other scheme than the null-scheme, the IMCI does not reveal the IMSI. MME may initiate authentication to receive IMSI as specified in clause 6.1.

In case the UE registers for Emergency Services and receives an Identifier Request, the UE shall use the null-scheme for generating the IMCI in the Identifier Response.

NOTE 2: 	Registration for Emergency does not provide subscription identifier confidentiality.

***** Next change *****

## 7.1	Mechanism for user identity confidentiality

### 7.1.1	Subscription concealed identifier

The IMCI is a privacy preserving identifier containing the concealed IMSI.

The UE should generate a IMCI using a protection scheme (see Annex X) with the raw public key that was securely provisioned in control of the home network.

The UE should construct a scheme-input from the subscription identifier part of the IMSI, as specified by the protection scheme. The UE should execute the protection scheme with the constructed scheme-input as input and take the output as the scheme-output.

The UE should not conceal the home network identifier, e.g., Mobile Country Code (MCC) or Mobile Network Code (MNC).

The UE should construct the IMCI consisting following data fields:

-	The protection scheme identifier that represents the null-scheme or a non null-scheme specified in Annex X or a protection scheme specified by the HPLMN.

-	The home network public key identifier that represents a public key provisioned by the HPLMN. In case of null-scheme being used, this data field should be set to null.

-	The home network identifier.

-	The protection scheme-output that represents the output of a public key protection scheme specified in Annex X or a protection scheme specified by the HPLMN.

NOTE 1: 	The format of the IMSI protection scheme identifiers is defined in Annex X.

NOTE 2:	The identifier and the format of the scheme output are defined by the protection schemes in Annex X. In case non-null schemes, the freshness and randomness of the IMCI will be taken care of by the corresponding IMSI protection schemes.

The UE should include a IMCI only to the following NAS messages:

-	If the UE is sending the Initial Attach request message to a PLMN for which the UE does not already have a GUTI and it is configured for "AttachWithIMSI", the UE should include the IMCI in the Attach Request message, or

-	If the UE includes a GUTI when sending a re-attach request to a PLMN and, in response, receives an Identity Request message, then the UE should include a IMCI in the Identity Response message.

Editor's Note: The maximum length of the IMCI needs to be aligned with the maximum length of the SUCI.

Editor's Note: It is FFS if the UE is allowed to send the IMCI in the Identifier Response message in response to an Identifier Request message from the network.

The UE should never generate a IMCI using "null-scheme" only in the following cases:

-	if the UE is making an unauthenticated emergency session and it does not have a GUTI to the chosen PLMN; or

-	if the home network has configured "null-scheme" to be used; or

- 	if the home network has not provisioned the public key needed to generate a IMCI.

If the operator's decision, indicated by the USIM, is that the USIM shall calculate the IMCI, then the USIM shall not give the ME any parameter for the calculation of the IMCI including the home network public key, the home network public key identifier, protection scheme profile, and the protection scheme identifier. If the ME determines that the calculation of the IMCI, indicated by the USIM, should be performed by the USIM, the ME shall delete any previously received or locally cached parameters for the calculation of the IMCI including the home network public key, the home network public key identifier, protection scheme profile, and the protection scheme identifier. The operator should use proprietary identifier for protection schemes if the operator chooses that the calculation of the IMCI should be done in USIM.

When IMCI calculation is performed by the ME, the ME shall request the USIM to provide the IMCI calculation information, including the IMSI, the home network public key, the home network public key identifier, and the protection scheme identifier. If the protection scheme is the null-scheme, the USIM should provide the IMSI and the protection scheme identifier. In case there is no indication from the USIM and the USIM provides only the IMSI to the ME, the ME should calculate the IMCI using the null-scheme.

### 7.1.2	Subscription identifier de-concealing function (SIDF)

SIDF is responsible for de-concealing the IMSI from the IMCI. SIDF is using the private key part of the privacy related home network public/private key pair that is securely stored in the home operator's network. The de-concealment shall take place at the HSS. Access rights to the SIDF shall be defined, such that only a network element of the home network is allowed to request SIDF.

Editor's note: One HSS can comprise several HSS instances. How to find the right HSS instance is FFS keeping in mind that the SIDF needs to be invoked first.

### 7.1.3	Subscription temporary identifier

The MME shall allocate a GUTI to a UE in order to support the subscriber identity confidentiality. The GUTI is defined in TS 23.003 [3].

S-TMSI, the shortened form of the GUTI, is used to support the subscriber identity confidentiality with more efficient radio signalling procedures (e.g. paging and Service Request). A new GUTI shall be sent to the UE only after a successful activation of NAS security.

***** Next change *****

## 15.1	General

Support for IMS Emergency Sessions is defined in the TS 23.401 [2]. Limited service state of a UE is defined in TS 23.122 [26]. IMS Emergency Sessions can be made by normally attached UEs or UEs attached for EPS emergency bearer services. IMS Emergency Services can be authenticated or unauthenticated as defined in clauses below. It depends on the serving network policy if unauthenticated IMS Emergency Sessions are allowed. Any behaviour not explicitly specified as being special to IMS Emergency Sessions is handled in accordance to normal procedures.

The E-UTRAN Initial Attach procedure, with Attach Type "Emergency", is used by UEs that need to receive EPS emergency bearer services but cannot receive normal services from the network.

For an Initial Attach with Attach Type "Emergency" the UE includes the IMCI in the Attach request if the UE does not have a valid GUTI. The UE shall include the IMEI when the UE has no IMSI, no valid GUTI according to [2].

When involved in an Attach for EPS emergency bearer services the MME applies the parameters from MME Emergency Configuration Data for the EPS emergency bearer establishment. Any potentially stored IMSI related subscription data is ignored by the MME according to [2].

When involved in an Attach for EPS emergency bearer services the MME does not send any Notify Request to an HSS.

A UE attached for EPS emergency bearer services using NULL algorithms shall keep the NULL algorithms and corresponding NAS COUNTs when in EMM-IDLE mode so that it is reachable for subsequent IMS Emergency Sessions without the need to attach for EPS emergency bearer services again. The NULL algorithms shall be de-selected and corresponding NAS COUNTs shall be removed when the UE goes to EMM-DEREGISTERED state or when another EPS NAS security context is activated.

The MME or UE shall always release any established non-emergency bearers, when the authentication fails in the UE or in the MME.

***** Next change *****

Annex X (normative):
Protection schemes for concealing the subscription permanent identifier

# X.1 	Introduction

The present Annex specifies the protection schemes for concealing the subscription permanent identifier.

# X.2 	Null-scheme

The null-scheme shall be implemented such that it returns the same output as the input, which applies to both encryption and decryption.

When using the null-scheme, the IMCI does not conceal the IMSI and therefore the newly generated IMCIs do not need to be fresh.

NOTE 1:	The reason for mentioning the non-freshness is that, normally, in order to attain unlinkability (i.e., to make it infeasible for over-the-air attacker to link IMCIs together), it is necessary for newly generated IMCIs to be fresh. But, in case of the null-scheme, the IMCI does not conceal the IMSI. So unlinkability is irrelevant.

NOTE 2:	The null-scheme provides no privacy protection.

The scheme properties for this null scheme shall be the following:

-	scheme identifier									: <TBD>

-	size of scheme-output				: <TBD> bits

# X.3 	Elliptic Curve Integrated Encryption Scheme (ECIES)

## X.3.1	General

The use of ECIES for concealment of the IMSI shall adhere to the SECG specifications [aa] and [bb]. Processing on UE side and home network side are described in high level in clauses X.3.2 and X.3.3.

## X.3.2	Processing on UE side

The ECIES scheme shall be implemented such that for computing a fresh IMCI, the UE shall use the provisioned public key of the home network and freshly generated ECC (elliptic curve cryptography) ephemeral public/private key pair according to the ECIES parameters provisioned by home network. The processing on UE side shall be done according to the encryption operation defined in [aa].

The final output shall be the concatenation of the ECC ephemeral public key, the ciphertext value, the MAC tag value, and any other parameters, if applicable.

NOTE:	The reason for mentioning "any other parameter, if applicable" in the final output is to allow cases, e.g. when point compression is used then depending upon the type of ECC curve the sender may be required to send an additional sign indication.

Editor's Note: The format and encoding of the final output needs to be finalized by CT WG1.

The Figure X.3.2-1 illustrates the UE's steps.

# Figure X.3.2-1: Encryption based on ECIES at UE

## X.3.3	Processing on home network side

The ECIES scheme shall be implemented such that for deconcealing a IMCI, the home network shall use the received ECC ephemeral public key of the UE and the private key of the home network. The processing on home network side shall be done according to the decryption operation defined in [aa].

NOTE:	Unlike the UE, the home network does not need to perform a fresh ephemeral key pair generation for each decryption. How often the home network generates new public/private key pair and how the public key is provisioned to the UE are out of the scope of this clause.

The Figure X.3.3-1 illustrates the home network's steps.

# Figure X.3.3-1: Decryption based on ECIES at home network

## X.3.4	ECIES profiles

### X.3.4.0	General

Unless otherwise stated, the ECIES profiles follow the terminology and processing specified in SECG version 2 [aa] and [bb]. The profiles shall use "named curves" over prime fields.

Profile A shall use its own standardized processing for key generation (section 6 of RFC 7748 [cc]) and shared secret calculation (section 5 of RFC 7748 [cc]). The Diffie-Hellman primitive X25519 (section 5 of RFC 7748 [cc]) takes two random octet strings as input, decodes them as scalar and coordinate, performs multiplication, and encodes the result as an octet string. The shared secret output octet string from X25519 shall be used as the input Z in the ECIES KDF (section 3.6.1 of [aa]).

Profile B shall use point compression to save overhead and shall use the Elliptic Curve Cofactor Diffie-Hellman Primitive (section 3.3.2 of [aa]) to enable future addition of profiles with cofactor h ≠ 1. For curves with cofactor h = 1 the two primitives (section 3.3.1 and 3.3.2 of [aa]) are equal.

The profiles shall not use backwards compatibility mode (therefore are not compatible with version 1 of SECG).

### X.3.4.1	Profile <A>

The ME and SIDF shall implement this profile. The ECIES parameters for this profile shall be the following:

-	EC domain parameters							: Curve25519 [cc]

-	EC Diffie-Hellman primitive					: X25519 [cc]

-	point compression								: N/A

-	KDF												: ANSI-X9.63-KDF [aa]

-	Hash												: SHA-256

-	SharedInfo1										:  R(the ephemeral public key octet string – see [aa] section 5.1.3)

-	MAC												: HMAC–SHA-256

-	mackeylen										: 256

-	maclen											: 64

-	SharedInfo2										: the empty string

-	ENC												: AES–128 in CTR mode

-	enckeylen											: 128

-	backwards compatibility mode					: false

The scheme properties for this ECIES profile <A> shall be the following:

-	scheme identifier									: <TBD>

-	size of scheme-output				: <TBD> bits

NOTE:	Care should be taken when using unique schemes for small groups of users, as this may impact the effectiveness of the privacy scheme for these users.

### X.3.4.3	Profile <B>

The ME and SIDF shall implement this profile. The ECIES parameters for this profile shall be the following:

-	EC domain parameters							: secp256r1 [bb]

-	EC Diffie-Hellman primitive					: Elliptic Curve Cofactor Diffie-Hellman Primitive [aa]

-	point compression								: true

-	KDF												: ANSI-X9.63-KDF [aa]

-	Hash												: SHA-256

-	SharedInfo1										: R (the ephemeral public key octet string – see [aa] section 5.1.3)

-	MAC												: HMAC–SHA-256

-	mackeylen										: 256

-	maclen											: 64

-	SharedInfo2										: the empty string

-	ENC												: AES–128 in CTR mode

-	enckeylen											: 128

-	backwards compatibility mode					: false