3GPP TSG-SA WG3 Meeting #91 	S3-181357

# Belgrade (Serbia), 16-20 April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0127 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of KAUSF source for 5G AKA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2018-04-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Inconsistent information on where key KAUSF is derived for 5G AKA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction in 6.2.1 that KAUSF is derived in ARPF for 5G AKA and not in AUSF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

### 6.2.1	Key hierarchy

Requirements on 5GC and NG-RAN related to keys:

a)	The 5GC and NG-RAN shall allow for use of encryption and integrity protection algorithms for AS and NAS protection having keys of length 128 bits and for future use the network interfaces shall be prepared to support 256 bit keys.

b)	The keys used for UP, NAS and AS protection shall be dependent on the algorithm with which they are used.

Figure 6.2.1-1: Key hierarchy generation in 5GS

The keys related to authentication (see Figure 6.2.1-1) include the following keys: K, CK/IK. In case of EAP-AKA', the keys CK', IK' are derived from CK, IK as specified in clause 6.1.3.1.

The key hierarchy (see Figure 6.2.1-1) includes the following keys: KAUSF, KSEAF, KAMF, KNASint, KNASenc, KN3IWF, KgNB, KRrCint, KRRCenc, KUPint and KUPenc.

Keys for AUSF in home network:

-	KAUSF is derived by ME and ARPF (for 5G AKA) or AUSF (for EAP-AKA’)..  KAUSF is derived by ME and ARPF from CK, IK in case of 5G AKA and by ME and AUSF from CK', IK' in case of EAP-AKA'. AUSF and ME derive from KAUSF the anchor key KSEAF.

Key for AMF:

-	KAMF is a key derived by ME and SEAF from KSEAF. KAMF is further derived by ME and source AMF when performing horizontal key derivation.

Keys for NAS signalling:

-	KNASint is a key derived by ME and AMF from KAMF, which shall only be used for the protection of NAS signalling with a particular integrity algorithm.

-	KNASenc is a key derived by ME and AMF from KAMF, which shall only be used for the protection of NAS signalling with a particular encryption algorithm.

Key for gNB:

-	KgNB is a key derived by ME and AMF from KAMF. KgNB is further derived by ME and source gNB when performing horizontal or vertical key derivation.

Keys for UP traffic:

-	KUPenc is a key derived by ME and gNB from KgNB, which shall only be used for the protection of UP traffic with a particular encryption algorithm.

-	KUPint is a key derived by ME and gNB from KgNB, which shall only be used for the protection of UP traffic between ME and gNB with a particular integrity algorithm.

Keys for RRC signalling:

-	KRRCint is a key derived by ME and gNB from KgNB, which shall only be used for the protection of RRC signalling with a particular integrity algorithm.

-	KRRCenc is a key derived by ME and gNB from KgNB, which shall only be used for the protection of RRC signalling with a particular encryption algorithm.

Intermediate keys:

-	NH is a key derived by ME and AMF to provide forward security as described in Clause A.10.

-	KgNB* is a key derived by ME and gNB when performing a horizontal or vertical key derivation as specified in Clause 6.9.4.1.1 using a KDF as specified in Clause A.11.

-  K'AMF is a key that can be derived by ME and AMF when the UE moves from one AMF to another during inter-AMF mobility as specified in Clause 6.9.3 using a KDF as specified in Annex <FFS>.

**** End of Changes ****