3GPP TR 33.cde V0.2.0 (2018-09)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Study on authentication and key management for applications;

based on 3GPP credential in 5G

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

MCC selects keywords from stock list.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	5

3.1	Definitions	5

3.3	Abbreviations	5

4	Scenario	6

4.1 Scenario #1: <Scenario name>	6

5	Key Issues	6

5.2	Key Issue #2: Transport independent procedure definition	7

5.2.1 Issue details	7

5.2.2 Security Threats	8

5.2.3 Potential architecture requirements	8

5.3	Key Issue #X: Mutual authentication between UE and anchor function	8

5.3.1 Key Issue details	8

5.3.2 Security Threats	8

5.3.3 Potential security requirements	9

5.5	Key Issue #5: User privacy	9

5.5.1 Issue details	9

5.5.2 Security Threats	9

5.5.3 Potential security requirements	10

5.6 Key Issue #6: secure communication between UE and application server	10

5.6.1 Issue details	10

5.6.2 Security threats	10

5.6.3 Potential security requirements	10

6 Candidate Solutions	11

6.1 Solution #1: Introducing third party key to AKMA 	11

6.1.1 Introduction	11

6.1.2 Solution details	11

6.X Solution <X>: <Solution Name>	13

6.x.1 Introduction	13

6.x.2 Solution details	13

6.x.3 Evaluation	13

7. Evaluation and conclusion	13

Annex <A>: <Annex title>	14

A.1	<Subtitle>	14

Annex <X>: Change history	15

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

1	Scope

The present document specifies key issues, derived requirements and potential solutions to support authentication and key management aspects for applications and 3GPP services based on 3GPP credentials in 5G, including the IoT use case. It analyzes issues and requirements for:

-	providing authentication and key management procedures to applications and 3GPP services in 5G scenarios which allow the UE to securely exchange data with an application server

-	decoupling these procedures from the the transport protocol, in order to allow for the adaption to differernt application layer protocols

The document takes into account new solutions as well as potential adaptations to existing ones such as GBA described in TS33.220 and BEST described in TS33.163, in order to support the above mentioned requirements with procedures and protocols defined in SBA.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[3]	3GPP TS 33.163: "Battery Efficient Security for very low Throughput Machine Type Communication (MTC) device (BEST)".

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

5GS	5G System

5GC	5G Core

AKA	Authentication and Key Agreement

AKMA	Authentication and Key Management for Applications

AKMA AF	AKMA Application Function

EPS	Evolved Packet System

GBA	Generic Bootstrapping Architecture

GAA	Generic Authentication Architecture

NAF	Network Application Function

SBA	Service Based Architecture

4	Scenario

4.1 Scenario #1: <Scenario name>

5	Key Issues

## 5.1	Key Issue #1: Security Anchor

### 5.1.1 Issue detail

The GBA/GAA features specified in TS 33.220 [2] leverage the EPS/UMTS authentication infrastructure (especially the HSS) to provide the security between the UE and an application function in the network with which the UE interacts on the User Plane. It should be noted that GBA uses UMTS AKA and that the HSS provides the CK/IK to the BSF instead of Kasme.

Figure 5.1.1-1 below shows the architecture of the features. GBA allows mutual authentication and the establishment of shared keys between the UE and BSF over the Ub interface. GAA, on the other hand, enables using such shared keys for protecting the access to a NAF. In principle GBA keys can be used to secure any protocol between a UE and a NAF over the Ua interface.

Figure 5.1.1-1: GBA and GAA reference architecture from TS 33.2020 [2]

Since the AKMA feature is intended to leverage the 5GS authentication infrastructure to provide similar services, it is understood that GBA/GAA would be one of the starting points for the architectural design of AKMA. However, due to differences between the 5GS and EPS/UMTS there is no direct equivalent of the BSF and HSS in the 5GC. These differences include, but are not limited to, the following:

-	The subscription data including the AKA credentials are stored in the UDM. However, it is another function, the AUSF, that is directly involved in the Primary Authentication procedure towards the serving PLMN.

-	The Primary Authentication procedure establishes a shared key (KAUSF) between the UE and the AUSF while no such key exits in the EPS key hierarchy.

-	The Primary Authentication is terminated in the AUSF by comparison to EPS where it is terminated in the MME.

-	All the internal interfaces in the 5GC are SBA-based by comparison to the DIAMETER-based Zh and Zn interfaces in GBA.

As shown in Figure 2, the AKMA architecture will naturally include an AKMA Application Function with which the UE communicates over the User Plane. The AKMA AF interacts with an anchor function, the BSF-equivalent, in the 5G Core. It is only logical to assume that such an anchor function is needed to authenticate the UE and potentially to provide key management services towards the AKMA AF.

Figure 5.1.1-2: Role of the anchor function in the AKMA architecture

Editor’s Note: Figure for non-standalone scenario is FFS.

Therefore, solutions to this key issue must address the following aspects.

-	How the anchor function is realized.

-	The interfaces involving the anchor function, the UE, the AKMA AF and other 5GS functions.

-	The procedures flow for the UE authentication and the management of the resulting bootstrapped keys used to secure the communication between the UE and the AKMA AF.

### 5.1.2 Security Threat

Not applicable.

### 5.1.3 Potential architectural requirement

The AKMA architecture shall support an anchor function in the 5GC for UE authentication. This function can be realized by a standalone or an existing function.

5.2	Key Issue #2: Transport independent procedure definition

5.2.1 Issue details

In AKMA, application server needs to be able to securely exchange data with a UE based on the result of authentication and key derivation between mobile network and UE.

In AKMA, there are three different communication interfaces, namely, (1)the communication between UE and 3GPP network,  (2) between UE and application server, and (3) between 3GPP network and application server. It is necessary to design the appropriate procedures Considering the stage-3 work, the protocol used for AKMA procedure can be divided into two categories:

1. Using an existing transport protocol

The existing protocols for carrying parameters and transferring data refer to the protocols well designed and widely used by 3GPP, IETF and/or other standard organizations, e.g. PDCP layer protocol, TCP/IP, etc. Using such protocols can bring benefit for the procedure design, as the work can concentrated on the signaling/message flows. There will not be a need to pay much attention on considering how to design message type, format, and any other details as they are well defined in the protocols.

However, using existed protocol may bring some issue. If the communication is through specific application layer protocol, it will bring requirement for transport layer protocol. For example, if the communication is based on HTTP, then TCP shall be applied between UE and mobile network.

However, for some kinds of UEs, especially UE used for IoT, the resource is limited. It will influence UE can only implement few protocols due to its memory and calculation limitation. If application server communicates with UE by using specific application protocol, it implies that UE may could not implement other protocols. It raises the requirement for the communication between UE and mobile network. If the communication is based on specific protocol, some kinds of UE that could not implement such protocol is not able to support AKMA feature. That may limit AKMA usage.

2. Designing specific protocol for AKMA

Compared to using existed protocol, designing a specific protocol for AKMA allows for as much freedom as possible to design protocol types, formats and content. So specific protocol can be designed more flexible to fit for various lower layer protocols.

However, designing such specific protocol is generally difficult and it is debatable whether the protocol will be sufficiently robust. What is more, as it is newly defined, there will not be existing implementations. If only a custom designed protocol will be used, adoption of AKMA may be hampered by the lack of these implementations and competition of existing protocols. Depending on the use case, therefore, it should be considered to reuse existing protocols and only design new ones if existing protocols do not meet the specific requirements of AKMA.

5.2.2 Security Threats

N/A

5.2.3 Potential architecture requirements

5.3	Key Issue #X: Mutual authentication between UE and anchor function

5.3.1 Key Issue details

To allow UEs securely communicating and exchanging data with an application server using the authentication and key management procedures for applications in 5G scenarios, it is expected that the AKMA framework would be leveraged. Therefore, in order to establish secure communication between the UE and the application server, the UE and the anchor function need to be able to mutually authenticate each other based on the 5G authentication framework first before allowing the application server to leverage this authentication in order to establish secure communication between the UE and the application server.

5.3.2 Security Threats

Without authentication in the UE, an illegal UE may communicate with the anchor function and access AKMA services.

A fake anchor function may communicate with the UE that could potentially lead to the loss and exposure of user privacy.

5.3.3 Potential security requirements

The UE and the anchor function shall be able to mutually authenticate each other based on 5G credentials using the 5G authentication framework.

## 5.4	Key Issue #4: Authentication framework

### 5.4.1	Issue details

The 5GS AKMA framework needs an authentication framework so that only legitimate UEs can use the AKMA services.

For example, it needs to be studied whether the AKMA authentication framework can leverage the fact that the primary authentication in the 5GS produces a key called the KAUSF at the AUSF in the HPLMN and the UE. The primary authentication meaning the one used to allow 5GS access to that UE. If that KAUSF could be the root key for the AKMA authentication framework, there would be no need for yet another authentication and therefore beneficial for IoT devices both signalling and processing wise. Recall that - in GBA/GAA architecture, the UE authentication (called boostrapping) was separate and additional authentication than the primary/access authentication providing access to the 3GPP system.

A careful analysis is required on effects of potential security compromise of AKMA authentication on 3GPP primary authentication and vice-versa.

### 5.4.2	Security Threats

Without a proper security design, compromise on AKMA authentication can jeopardize security on 3GPP side.

### 5.4.3	Potential security requirements

The system shall support a secure authentication framework to allow only legitimate UEs to use AKMA services.

The system shall prevent a potential security compromise of AKMA authentication from propagating to the 3GPP primary authentication and NAS/AS security.

Editor's Note: The above requirements are non-exhaustive and could evolve during the study.

5.5	Key Issue #5: User privacy

5.5.1 Issue details

The Subscription Permanent Identifier (SUPI) is considered sensitive information, since attackers may identify an individual subscriber through his/her permanent ID. Combined with other kinds of information, such as geographic location, an attacker may be able to trace a subscriber, or obtain access to further sensitive information. Thus, the Subscription Permanent Identifier needs to be protected.

Meanwhile, the SUPI being the basis for providing any service in 5G a network, must be known to the operator. This means that the operator is obliged to ensure that the SUPI is not revealed to any other parties.

When an operator wants to provide authentication and key management to an application server, it must have the ability to exchange information about a subscriber to enable the application server to determine the identity of its user. Hence, there is a need for another kind of identifier (permanent and/or temporarily) to identify users between the 3GPP network and an application server. And the MNO should be able to map the other kind of identifier to the permanent identifier of the MNO domain.

5.5.2 Security Threats

The Subscription Permanent Identifier may be leaked to unauthorized parties.

The application server may be unable to identify the user.

The operator may be unable to identify the users SUPI based on the new identifier between 3GPP network and application server.

5.5.3 Potential security requirements

SUPI shall not be revealed to application servers.

The 3GPP network shall be able to recover the SUPI based on an alternative identifier used between 3GPP network and application server.

5.6 Key Issue #6: secure communication between UE and application server

5.6.1 Issue details

In current BEST[3] and GBA[2] solutions, 3GPP network is responsible to derive Keys(e.g. KE2Menc, KE2Mint , Ks_(int/ext)_NAF) for UE and application server derived from the root subscriber authentication key K. However, the application server may not want to use this key, which is derived from the 3GPP network authentication key K. They may have a policy, which requires they use their own independently generated key (e.g. application specific key), but still require the use of features provided by the 3GPP network to distribute such a key. The mechanism can satisfy the demand of application providers who do not wish to establish the secure connection by using only a 3GPP credential.

In some scenarios, such as when the UE sends sensitive data to application server, the application security policy may require that the 3GPP network operator does not have accesses to that information. In addition, the services provided by the application server may be accessed by multiple applications. Therefore, it is desirable that a solution that addresses this key issue supports establishment of separate application specific keys for each application that are served by the application server.

5.6.2 Security threats

3GPP network may get access to sensitive data transferred between UE and applications which is protected by the key derived from 3GPP network, or from 3GPP network and a pre-shared key (i.e., non-3GPP credential) if the pre-shared key gets compromised.

5.6.3 Potential security requirements

TBD

## 5.7	Key Issue #7: Protecting subscriber's personal information in control and data traffic

### 5.7.1	Issue details

This key issue is about potential personal information contained in various control and data traffic messages.

If AKMA architecture uses some form of content in control and or data traffic which is privacy sensitive, those content need to be protected against attacks.

By attacks, it is meant that unauthorized entities attempt to identify subscriptions by getting hold of the privacy sensitive content in one or more protocol messages.

### 5.7.2	Security Threats

Unprotected privacy sensitive content in control and or data traffic make it easier for attackers to potentially identify subscribers.

### 5.7.3	Potential security requirements

The system shall support protecting the privacy sensitive content in control and data traffic used in the AKMA architecture.

Editor's Note: It is FFS which AKMA interfaces are required to protected privacy sensitive content.

## 5.8	Key Issue #8: Protection of AKMA architecture interfaces

### 5.8.1	Issue details

The interfaces utilized by the AKMA architecture between the 5G system and the 3GPP services and application functions (commonly called AKMA AF) are supposed to transfer key material and and therefore needs to be properly evaluated.

### 5.8.2	Security Threats

In case the interfaces used by AKMA architecture lack confidentiality, integrity and replay protection between authenticated endpoints it will be possible for an attacker to eavesdrop, alter data unnoticed and replay packets.

### 5.8.3	Potential security requirements

The interfaces utilized by the AKMA architecture between the 5G system and the 3GPP services and application functions shall support confidentiality, integrity and replay protection between authenticated endpoints.

## 5.9	Key Issue #9: Key separation for AKMA AFs

### 5.9.1	Issue details

In a scenario where the 5G system provides cryptographic keys to AKMA Application Functions (either 3GPP services or third party applications), it is important to have key separation. In the sense that two separate AKMA AFs never utilize the same key.

### 5.9.2	Security Threats

If there is no key separation it can lead to a situation where one AKMA AF can decrypt traffic intended for another AKMA AF.

It would also allow the possibility for an actor to inject malicious packages which the UE would conclude as cryptographically correct.

### 5.9.3	Potential security requirements

The AKMA architecture shall support key separation for different AKMA AFs.

6 Candidate Solutions

6.1 Solution #1: Introducing third party key to AKMA

6.1.1 Introduction

The secure transferring between the UE and the 3rd party not only requires secure connection, but to some extent protects data from leakage to untrusted parties even including MNOs, especially for some large CIoT corporations. Current GBA solution provides secure connection for the application providers based on 3GPP credentials, however, it lacks mechanism to ensure end to end security. Therefore, introducing a third party key to AKMA is an optional ability provided by 3GPP networks to protect data from UE all the way to the application server. The 3rd party key is defined as a secret key shared by the application server and the UE for application level communication. According to 3rd party service security requirements, whenever necessary to application providers, they can choose to use derived keys from 3GPP credentials and 3rd party keys to secure the end to end connection. In this way, application providers are able to control over the key material specifically.

6.1.2 Solution details

The proposed solution takes the current GBA procedure for example (Note: The related network elements and procedures in AKMA is FFS, the following figure only illustrates the 3rd party key involving procedure). During the procedure using bootstrapped security association, after NAF fetches Ks_(ext/int)_NAF from BSF, if necessary, the 3rd party executes end to end key derivation and sends to UE an e2e flag indicating the use of combination key scheme. According to the e2e flag, the UE derives the end to end key which is used for the following secure connection between UE and 3rd party.

Editor’s Note: It’s FFS that the 3rd party mentioned above could be key management service provider or application provider.

The e2e_key is derived according to:

e2e _key = KDF (Ks_(ext/int)_NAF, Ka);

where Ka is the 3rd party key defined in 6.X.1.

Editor’s Note: The derivation algorithm is FFS.

6.X Solution <X>: <Solution Name>

6.x.1 Introduction

6.x.2 Solution details

6.x.3 Evaluation

7. Evaluation and conclusion

Annex <A>:
<Annex title>

A.1	<Subtitle>

Annex <X>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018.5 | 91-bis | S3-181813 |  |  |  | TR skeleton | 0.0.1 |
| 2018.5 | 91-bis | S3-182077 |  |  |  | Scope is revised based on S3-182054 | 0.1.0 |
| 2018.9 | 92-bis | S3-183157 |  |  |  | Updated based on S3-183154, S3-183155, S3-183156, S3-183006, S3-183158, S3-183159, S3-183160, S3-183161, S3-183162, S3-183163 | 0.2.0 |