3GPP TSG-SA3 Meeting #93	S3-183236

# Spokane, United States, 12th Nov 2018 - 16th Nov 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0403 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of encrypted IEs on the N32 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Telekom Deutschland GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  | Date: |  |  | 2018-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Existing text on default behaviour of encrypted IEs in the SEPP is misleading. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds two new SEPP requirements to clause 13.2.4.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Insecure handling of encrypted IEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### *** First Change ***

### 13.2.4	N32-f connection between SEPPs

#### 13.2.4.1	General

The SEPP receives the HTTP/2 request/response messages from the Network Function. It performs the following actions on these messages before they are sent on the N32-f interface to the SEPP in the other PLMN:

a) It parses the incoming message and reformats it to produce the input to JWE (clause 13.2.4.3).

b) It applies JSON Web Encryption (JWE) [x] on the input created in a) to protect the reformatted message (clause 13.2.4.4).

c) It encapsulates the resulting JWE object into a HTTP/2 message (as the body of the message) and sends to the SEPP in the other PLMN over the N32-f interface.

The path between the two SEPPs may take them via the cIPX and pIPX nodes. These IPX nodes may modify messages as follows:

a) The IPX node recovers the unencrypted (cleartext) section of the HTTP message (in the JWE object), modifies it according to the modification policy, and calculates an "operations" JSON Patch object. It creates a temporary JSON object with "operations" and few other parameters for replay protection etc. (clause 13.2.4.5.1).

b) The temporary JSON object is input into JSON Web Signature (JWS) [45] to create a JWS object (clause 13.2.4.5.2).

c) The JWS object is appended to the received message and sent to the next hop.

The JWS objects generated by the two IPX providers form an auditable chain of modifications that are applied to the parsed message at the receiving end after verifying that the patches conform to the modification policy.

Encryption of IEs take place end to end between cSEPP and pSEPP.

A SEPP shall not include IEs in the clear that are encrypted elsewhere in the JSON object.

A SEPP shall verify that an intermediate IPX has not moved or copied an encrypted IE to a location that would be reflected back from the producer NF in an IE without encryption.

### *** End of Changes ***