# 3GPP TSG SA WG3 (Security) Meeting #95-Bis	S3-191922

# Sapporo (Japan), 24-28 June 2019	revision of S3-19abcd

# Source:	Qualcomm Incorporated

# Title: 	Evaluation of the shared key based MIB/SIB protection solution

# Document for:	Endorsement

Agenda Item:	8.9

# 1	Decision/action requested

This contribution proposes an evaluation text for solution #14: Shared key based MIB/SIB protection

# 2	References

[1]	 TR 33.809

# 3	Rationale

This paper proposes an evaluation text for the solution #14.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR 33.809 [1].

***** START OF FIRST CHANGES *****

### 6.14.3 	Evaluation

This solution fulfils the potential security requirement of the KI #2 when the UE is in the RRC-Connected state.

This solution does not filful the potential security requirement of the KI #2 when the UE is in the RRC-Idle or the RRC-Inactive state.

This solution does not require any additional key provisioning and setup procedure. This solution only requires transporting the hashes of MIB/SIBs or MIB/SIBs using secure RRC signalling.

Since the MIB/SIBs mostly contain the radio configuration information that is used for the UE to make a connection to the network and stay in the connected state, enabling to detect the modification of MIB/SIBs during the connection establishment would significantly reduce the impact of SI modification by false base station.

This solution does not prevent the UE from camping on a false base station, but it should be noted that protecting the MIB/SIB message itself cannot prevent the UE from camping on a false base station because a false base station can always replay the MIB/SIBs acquired by legitimate base stations.

***** END OF FIRST CHANGES *****