3GPP TSG-SA WG3 Meeting #95Bis 	S3-191938

# Sapporo(Japan), 24-28 June 2019

# Source: Nokia, Nokia Shanghai Bell

# Title:     Nokia comments on R2-1908467 Reply LS on Handling of UE radio network capabilities in 4G and 5G

# Document for:	Discussion

Agenda Item:

3GPP TSG RAN WG2#106		R2-1908467

Reno, Nevada, USA, 13th – 17th May, 2019

# Title:	Reply LS on Handling of UE radio network capabilities in 4G and 5G

# Response to:	R2-1908112/CVD Doc 2019-0018_001 Handling of UE radio network capabilities in 4G and 5G from GSMA CVD Governance Team

# Release:

# Work Item:

Source:	RAN2

To:	GSMA CVD Governance Team

CC:	SA3, RAN, SA

Contact Person:

#### Name:	Sudeep Palat

Tel: 		+44-1793402336

E-mail Address:	sudeep.k.palat@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN2 thanks GSMA CVD Governance Team for the LS on Handling of UE radio network capabilities in 4G and 5G.

There are some benefits in retrieving the UE AS capability before security activation (e.g., it allows network to apply the same sequence of combined Security Mode Command + Reconfiguration for RRC connection setup for Registration and Service request procedures) and, in this scenario, to verify against any man-in-the-middle attack, it is possible in current LTE and NR specifications to rerun the UECapabilityEnquiry procedure after security activation.

Nokia comment: There may be signalling optimization benefit in doing UECapabilityEnquiry before AS SMC, but creates a privacy issue. Hackers have succeeded in extracting lot of information about the UE using this procedure. Re-running the UECapabilityEnquiry procedure after AS SMC doesn’t alleviate the privacy issue.

Further, RAN2 confirms that the RAN2 specifications for LTE (Rel-15 as well as previous releases) and NR already allow running the UECapabilityEnquiry procedure after activation of security.

Nokia comment: Since running the UECapabilityEnquiry procedure after activation of security is already allowed, this should be mandated and running it before the AS SMC should discouraged.

RAN2 can make the necessary updates to the specifications to include a recommendation on network implementation that security should be applied for UECapabilityEnquiry procedure if requested by SA3.

Nokia comment: SA3 should endorse the above for implementation.

2. Actions:

None

3. Date of Next RAN2 Meetings:

3GPP RAN2#107			26th – 30th August 2019			Prague, CZ

3GPP RAN2#107bis			14th – 18th October 2019			China