3GPP TSG-SA WG3 Meeting #95-Bis 	S3-192219

# Sapporo(Japan), 24-28 June 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0636 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to Initial NAS message protection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  |  | Date: |  | 2019-06-17 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to current specification TS 33.501 if the UE has 5G NAS security context then the UE encrpts cleartext and non-clear text IEs using the ciphering method of the 5G NAS security context and sends in the Initial NAS message. This behavior also applies for the case when the assigned ciphering algorithm in 5G NAS security context is NULL ciphering (NEA0) i.e. by local regulation PLMN selects null ciphering. This behavior brings following issue. - A UE is registered to a PLMN and 5G NAS security context is established with null ciphering method (NEA0) due to local regulation (local regulation of a country). - The UE changes to another PLMN(e.g. a PLMN of another country) which does not allow null ciphering according to local requirement. In this scenario the UE will sends non-cleartext IE will null ciphering as per the 5G NAS secruity context established in step 1. In the above secnario the UE violates the local requirement of the PLMN and also the non-cleartext IEs (sensitive information) are sent with null ciphering i.e. without any encrptions so the man in middle can get know the sensitive information of the UE. Also CT1 in last CT1 meeting#117 has agreed a CR (C1-193295) where it is mentioned that the UE will perform a Registration procedure for mobility and periodic registration update when a UE selects a new PLMN in the 5GMM-REGISTERED.PLMN.SEARCH state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that when the UE registered to a PLMN and 5G NAS security context is established with null ciphering and the UE selects another PLMN then the UE sends Initial NAS message with cleartext IE(s) only. When the new PLMN initiates NAS Security mode control procedure then the UE sends the initial NAS message with cleartext(s) and non-cleartext IE(s) in Security mode complete message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will sends viloates the local regulation and also the user sensitive information are sent without encryption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 6.4.6	Protection of initial NAS message

The initial NAS message is the first NAS message that is sent after the UE transitions from the idle state. The UE shall send a limited set of IEs (called the cleartext IEs) including those needed to establish security in the initial message when it has no NAS security context or has a NAS security context containing the EEA0 or NEA0 algorithm and the UE selects a PLMN other than Registered PLMN in 5GMM-IDLE state. When the UE has a NAS security context, the UE shall send a message that has the complete initial NAS message ciphered in a NAS Container along with the cleartext IEs with whole message integrity protected. The complete initial message is included in the NAS Security Mode Complete message in a NAS Container when needed (e.g. AMF cannot find the used security context) in the latter case and always in the former case as described below.

The protection of the initial NAS message proceeds as shown in Figure 6.4.6-1.

Figure 6.4.6-1: Protecting the initial NAS message

Step 1: The UE shall send the initial NAS message to the AMF. If the UE has no NAS security context or has a NAS security context containing the EEA0 and the UE selects a PLMN other than Registered PLMN in 5GMM-IDLE state, the initial NAS message shall only contain the cleartext IEs, i.e. subscription identifiers (e.g. SUCI or GUTIs), UE security capabilities, ngKSI,  indication that the UE is moving from EPC, Additional GUTI, and IE containing the TAU Request in the case idle mobility from LTE.

If the UE has a NAS security context, the message sent shall contain the information given above in cleartext and the complete initial NAS message ciphered in a NAS container which is ciphered. With a NAS security context, the sent message shall also be integrity protected. In the case that the initial NAS message was protected and  the AMF has the same security context, then steps 2 to 4 may be omitted In this case the AMF shall use the complete initial NAS message that is in the NAS container as the message to respond to..

Step 2: If the AMF is not able to find the security context locally or from last visited AMF, or if the integrity check fails, then the AMF shall initiate an authentication procedure with the UE. If the AMF fetches old security context from the last visited AMF, the AMF may decipher the NAS container with the same security context, and get the initial NAS message, then the step 2b to 4 may be omitted. If the AMF fetches new K AMF from the last visited AMF (receiving keyAmfChangeInd), the step 2b may be omitted.

Step 3: If the authentication of the UE is successful, the AMF shall send the NAS Security Mode Command message. If the initial NAS message was protected but did not pass the integrity check (due either to a MAC failure or the AMF not being able to find the used security context) or the AMF could not decrypt the complete initial NAS message in the NAS container (due to receiving "keyAmfChangeInd" from the last visited AMF), then the AMF shall include in the Security Mode Command message a flag requesting the UE to send the complete initial NAS message in the NAS Security Mode Complete message.

Step 4: The UE shall send the NAS Security Mode Complete message to the network in response to a NAS Security Mode Command message. The NAS Security Mode Complete message shall be ciphered and integrity protected. Furthermore the NAS Security Mode Complete message shall include the complete initial NAS message in a NAS Container if either requested by the AMF or the UE sent the initial NAS message unprotected. The AMF shall use the complete initial NAS message that is in the NAS container as the message to respond to.

Step 5: The AMF shall send its response to the Initial NAS message. This message shall be ciphered and integrity protected.