# 3GPP TSG SA WG3 (Security) Meeting #95-Bis	S3-192348

# Sapporo (Japan), 24-28 June 2019	merge of S3-191914 and S3-192120

# Source:	Qualcomm Incorporated, Ericsson, Nokia, Huawei, HiSilicon

# Title: 	Conclusion on KI #1 for Study on the security for URLLC

# Document for:	Approval

Agenda Item:	8.13

# 1	Decision/action requested

This contribution proposes a conclusion for the KI#1.

# 2	References

[1]	TR 33.825 Study on the Security for 5G URLLC

# 3	Rationale

Based on the evaluations on solution #5 and solution #7, the solution #7 is recommended for the security of URLLC.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

***** START OF CHANGES *****

## 7.1	Key Issue #1: Security for redundant transmission

For Key Issue #1, it is recommended that the normative work proceed as follows:

- Solution #4 is used as the baseline for ensuring that the security protection in case of the redundant traffic is fulfilled by independent N3 tunnels.

- Solution #7 is used as the baseline to ensure security and cryptographic separation for the different radio bearers serving redundant transmissions based on Dual Connectivity architecture. It is agreed that the different UP integrity keys and UP encryption keys are used for the radio bearers serving those redundant transmissions.

***** END OF CHANGES *****