# 3GPP TSG-SA WG3 Meeting #95bis 	S3-192349

# Sapporo (Japan), 24-28 June 2019	revision S3-192122

# Source:	Huawei, HiSilicon, Qualcomm Incorporated, Ericsson, Nokia

# Title:	Conclusion for KI #2

# Document for:	Approval

Agenda Item:	8.13

1	Decision/action requested

This contribution proposes a pCR for the conclusion for KI #2 of TR 33.825.

2	References

[1]	3GPP TR 23.725 Study on enhancement of Ultra-Reliable Low-Latency Communication (URLLC) v0.4.0

3	Rationale

The contribution proposes to add the conclusion for KI #2.

For KI #2, the potential security requirements as follows are proposed:

- Confidentiality protection over the air shall be supported for UP data via redundant multiple transmission paths.

- Integrity protection over the air shall be supported for UP data via redundant multiple transmission paths.

The solution #7 and solution #5 proposes the means on how to generate the confidential protection and integrity protection keys which are used for cyphering and integrity protection. In order to limit the impacts comparing with the existing dual connectivity architecture, it’s proposed to take the solution #7 to be as the baseline for the normative phase work.

4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.825.

***	BEGIN CHANGES	***

7.2	Key Issue #2: Support of security for high reliability by redundant data transmission in user plane

For Key Issue #2, it is recommended that solution #7 is taken into account as the basis for normative work.

***	END OF CHANGES	***