3GPP TSG-SA WG3 Meeting #95Bis                                                 S3-191956

# Sapporo(Japan), 24-28 June 2019																		revision of S3-19xabc

# Source:	Intel

# Title:	Security solution for UE to avoid connecting to the false base station during a handover procedure

# Document for:	Approval

Agenda Item:	8.9

# 1	Decision/action requested

It is requested to approve the solution to avoid UE connection to the false base station during handover

# 2	References

# 3	Rationale

# This solution addresses the key issues #1 “Security of unprotected unicast messages” for RRC Reject message protection and Key issue #7 “Protection against Man-in-the-Middle false gNB attacks

# 4	Detailed proposal

********************* Start of Changes**********************

## 6.Y	Solution #X: Security solution for UE to avoid connecting to the false base station during a handover procedure

### 6.Y.1	Introduction

This solution addresses the security requirement in key issue #3 for preventing UE from connecting to false base station and key issue 7 Protection against Man-in-the-Middle false gNB attacks.

### 6.Y.2	Solution details

#### 6.Y.2.1	Background

In false base station scenario, it is possible that fake base station is mimicking the system information and reference signals of the real base station and monitoring messages sent to the UE or by the UE. During handover, UE may send measurement report of the fake base station while it may get the genuine handover command for the real base station. Also, it is possible that UE sends preamble or contention-free preamble to the real base station while a fake UE is listening to it and sends fake Msg2 in response to the preamble. This Msg2 may arrive earlier than the genuine Msg2, which could result in HO failure and more power consumption to UE, as shown in figure 6.Y.2.1-1 below. The solution proposes to fix

Figure 6.Y.2.1-1 HO procedure caused by false base station C

#### 6.Y.2.2	HO procedure

#### Proposed HO procedure is shown in figure XXXX. In this procedure, target gNB sends and allocates T-C-RNTI or a range of T-RNTI before sending the HO Request Ack. Target gNB also sends some of the information that it intends to send in MSG2. After receiving the handover command, UE initiates the synchronization to the target base station. It can send a preamble starting RACH. There are two options discussed in the solution when contention free preamble is used, and contention preamble is used.

#### Figure 6.Y.2.2-1 HO procedure with Temporary C-RNTI in HO Command.

#### Step 1-4.) Upon receiving the HO request command,

- In case of contention free preamble, the target gNB allocates a temporary C-RNTI beforehand the UE, for which handover command is prepared and sends preamble in the handover command.  A contention-free preamble, which will be provided in the real Msg2, is also provided in the HO command beforehand by the target base station.

- When contention free preamble is not configured, the real gNB does identify if the Msg1 is sent by the intended UE performing handover or any other UE initiating random access. Therefore, in this case, the target gNB provides a range of temporary C-RNTI it plans to allocate over time. The range can be refreshed as a sliding window based on handover failure time T304 or a new timer. In this case, along with range or a unique temporary C-RNTI, scheduling information like offset value, multiplier value to the offset and the total number of RNTIs allocated during the time window defined by a new timer or existing timer T304 can be provided in the handover command. The range of RNTIs provided in handover command can be large enough such that during the time window, it will be sufficient to allocate unique temporary C-RNTI for any UE that sends preamble.

#### Step 5) UE sends msg1 using Preamble received in HO command.  The severity of the MITM threat is reduced as it assured that the M1 message is from genuine UE.

#### Step 6) When UE receives the Msg2, it matches (or within range)the temporary C-RNTI provided in Msg2 with the one provided in HO command. If it matches, then it received the real Msg2. If it does not match, the UE ignores the Msg2 and keep monitoring the Msg2 following legacy procedure.

### 6.Y.3	Evaluation

TBD

********************* End of Changes**********************