# 3GPP TSG SA WG3 (Security) Meeting #96 bis	S3-193787

# 14-18 October 2019, Chongqing (China) 	revision of S3-193640

# Source:	China Mobile

# Title:	Adding Generic assets and threats of GVNP for type 1 into clause 5.2.3.b

# Document for:	Approval

Agenda Item:	5.9

1	Decision/action requested

It is proposed to add Generic assets and threats of GVNP for type 1 into clause 5.2.3.b

2	References

[1]  3GPP TR 33.916: "Security Assurance Methodology (SCAS) for 3GPP network products"

[2]  3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes"

[3]  3GPP TR 33.117: "Catalogue of general security assurance requirements"

[4]  3GPP TR 21.905: " Vocabulary for 3GPP Specifications"

[5] 3GPP TS 28.500: "Management concept, architecture and requirements for mobile networks that include virtualized network functions"

[6] ETSI GS NFV-SEC 001: "Network Functions Virtualisation (NFV); NFV Security; Problem Statement"

[7] GSMA Network Equipment Security Assurance Scheme – Vendor Development and Product Lifecycle Requirements and Accreditation Process

[8] ETSI GR NFV-SEC 007: "Functions Virtualisation (NFV); Trust; Report on Attestation Technologies and Practices for Secure Deployments"

[9]3GPP TR 33.848: "Study on security impacts of virtualisation"

[10] 3GPP TR 33.805: "Study on security assurance methodology for 3GPP network products (Release 12) "

[11] ETSI GS NFV 002: " Network Functions Virtualisation (NFV); Architectural Framework"

3	Rationale

This contribution analyzes the SPD for 3GPP virtualized network products and adds Generic assets and threats of GVNP for type 1 into clause 5.2.3.Y.

4	Detailed proposal

****************** Start of the change ******************

5.2.3	Security Problem Definition (SPD) for 3GPP virtualized network products class

5.2.3.a 	Introduction

Clause 5.2.2 of TR 33.916[1] describes the steps to be accomplished for the SPD part of the SCAS writing phase, principles and structures for threats and security objectives. These are general guidelines and can also be applied to SPD analysis of 3GPP virtualized network products. In addition, clause 5 of TR 33.926[2] describes the generic assets and threats according to the structures described in TR 33.916[1]. The following subclauses describe the generic assets and threats in the course of developing 3GPP security assurance specifications for a particular virtualized network product class by referring to the generic assets and threats in TR 33.926[2].

5.2.3.b 	Generic assets and threats of GVNP for type 1

5.2.3.b.1 Generic assets of GVNP for type 1

The critical assets of GVNP for type 1 that need to be protected are:

-	User account data and credentials (e.g. passwords, private key);

-	Log data;

-	Configuration data, e.g. GVNP's IP address, ports, VPN ID, Management Objects (e.g. user group, command group) etc.

-	Guest Operating System, i.e. the files that make up the guest OS and its processes (code and data);

-	GVNP Application;

-	Sufficient processing capacity: that processing powers are not consumed close to limits;

-	The interfaces of GVNP to be protected and which are within SECAM scope: for example:

-	OAM interface, for remote access: interface between GVNP and OAM system

-	Interface between virtualized network function (VNF) and VNFM

-    Interface between VNF and virtualisation layer, for providing the execusion environment to run VNF

-	GVNP Software: binary code or executable code

-	VNF package;

-	VNF image;

5.2.3.b.2 Generic threats for GVNP of type 1

5.2.3.b.2.1 Introduction

In clause 5.3.1 of TR 33.926[2], the identified threats are grouped into seven categories, one covering threats relating to 3GPP-defined interfaces and the other six corresponding to the categories proposed by STRIDE. Since these seven categories are for generic 3GPP network products, they are also applicable to GVNP of type 1. In addition, GVNP of type 1 also needs to consider the threats related to ETSI-defined interfaces. As a result, there are eight categories of threats for GVPN of type 1. The following subclauses describe the threats according to these security categories and use the template of threat description in clause 5.3.1 of TR 33.926[2]. For threats descriptions of current seven categories, this present document will focus on the differences between GVNP threats and GNP threats which are described in TR 33.926[2].

5.2.3.b.2.2 Threats relating to 3GPP-defined interfaces

For GVNP of type1 and GNP in TR33.926[2], the threats related to 3GPP-defined interfaces are the same. So, all texts in clause 5.3.2 of TR 33.926[2] apply to GVNP of type 1. It means that there is no need repeat the threats relating to 3GPP-defined interfaces which are covered in 3GPP security specifications. If threats relating to 3GPP-defined interfaces are found not sufficiently covered in existing 3GPP security specifications, they need to be addressed in the SCAS for virtualised network products.

5.2.3.b.2.3 Threats relating to ETSI-defined interfaces

Two of the interfaces defined in ETSI NFV specification [11] are identified as the critical assets of GVNP type 1, i.e. interface between VNF and VNFM, interface between 3GPP VNF and virtualisation layer. The threats on these interfaces are as follows.

-	Threats on interface between 3GPP VNF and VNFM: an attacker can compromise a VNFM to attack a 3GPP VNF. For example, the attacker illegally terminates a 3GPP VNF or tampers with VNFD of a 3GPP VNF without authorization, resulting in DoS attack or information leak against the 3GPP VNF.

-     Threats on interface between 3GPP VNF and virtualisation layer: an attacker can attack a VNF through a compromised virtualisation layer. For example, cryptographic keys or other security critical data of a VNF could be stolen by an attacker with access to the virtualisation layer.

Editor’s note: More threats described in 3GPP TR 33.848[9] or/and ETSI specification etc are to be added if identified as related to the above two interfaces.

5.2.3.b.2.4 Spoofing identity

5.2.3.b.2.4.1 Default Accounts

The threat in clause 5.3.3.1 of TR 33.926[2] is generic, so it also applies to GVNP of type 1. The difference is that VNF is accessed through VNC (Virtual Network Console) rather than through the physical console interface, an attacker can use a default account to access a VNF via VNC.

5.2.3.b.2.4.2 Weak Password Policies

The threat in clause 5.3.3.2 of TR 33.926[2] is generic, so it also applies to GVNP. However, the attacker using the weak password accesses GVNP through VNC (Virtual Network Console) rather than through the physical console interface.

5.2.3.b.2.4.3 Password peek

The threat in clause 5.3.3.3 of TR 33.926[2] is generic, so it also applies to GVNP. However, the attacker using the peeked password accesses GVNP through VNC (Virtual Network Console) rather than through the physical console interface.

5.2.3.b.2.4.4 Direct Root Access

The threat in clause 5.3.3.4 of TR 33.926[2] is generic, so it also applies to GVNP of type 1. There are no differences between direct root accesses for GVNP and GNP described in TR 33.926[2].

5.2.3.b.2.4.5 IP Spoofing

The threat in clause 5.3.3.5 of TR 33.926[2] is generic, so it also applies to GVNP of type 1. However, the objective of unauthorized access is a VNF, not a computer.

5.2.3.b.2.4.6 Malware

The threat in clause 5.3.3.6 of TR 33.926[2] is generic, so it also applies to GVNP of type 1.

5.2.3.b.2.4.7 Eavesdropping

The threat in clause 5.3.3.7 of TR 33.926[2] is generic, so it also applies to GVNP of type 1.

5.2.3.b.2.5 Tampering

5.2.3.b.2.5.1 Software Tampering

The threat in clause 5.3.4.1 of TR 33.926[2] is generic, so it also applies to GVNP of type 1.

5.2.3.b.2.5.2 Ownership File Misuse

The threat in clause 5.3.4.2 of TR 33.926[2] is generic, so it also applies to GVNP of type 1.

5.2.3.b.2.5.3 Boot tampering for GVNP of type 1

For GVNP of type 1, there is no hardware. This is different from external device boot of GNP described in clause 5.3.4.3 of TR 33.926[2]. The threat is described as follows:

-	Threat name: GVNP of type 1 boot tampering

-	Threat Category: Tampering

-	Threat Description: the GVNP bootloader may be maliciously tamperred by an attacker, e.g. the attacker tampers the bootloader of GVNP through a malicious virtulisation layer.

-	Threatened Asset: guest operating system

5.2.3.b.2.5.4 Log Tampering

The threat in clause 5.3.4.4 of TR 33.926[2] is generic, so it also applies to GVNP of type 1.

5.2.3.b.2.5.5 OAM traffic Tampering

The threat in clause 5.3.4.5 of TR 33.926[2] is generic, so it also applies to GVNP of type 1.

5.2.3.b.2.5.6 File Write Permissions Abuse

The threat in clause 5.3.4.6 of TR 33.926[2] is generic, so it also applies to GVNP of type 1.

5.2.3.b.2.5.7 User Session Tampering

The threat in clause 5.3.4.7 of TR 33.926[2] is generic, so it also applies to GVNP of type 1.

5.2.3.b.2.6 Repudiation

5.2.3.b.2.6.1 Lack of User Activity Trace

The threat in clause 5.3.5.1 of TR 33.926[2] is generic, so it also applies to GVNP of type 1.

5.2.3.b.2.7 Information disclosure

The threat in all subclauses of clause 5.3.6 for TR 33.926[2] is generic, so they apply to GVNP of type 1.

5.2.3.b.2.8 Denial of Service

The threats in all subclauses of clause 5.3.7 for TR 33.926[2] are generic, so they also apply to GVNP of type 1. In addition, there is DoS attack due to changing virtualisation resource that is used by GVNP. The detailed threat description is as follows:

-	Threat name: changing virtualisation resource without authorization

-	Threat Category: DoS

-	Threat Description: There are several ways to cause a DoS attack for the GVNP: a compromised virtualisation layer can change the virtualisation resource used by a GVNP without authorization, or malicious VMs deployed on the same host as that for a VNF can illegally occupy the host's resources to result in resource limitation of the VNF, or a compromised VNFM can scale in the virtualisation resource used by a GVNP without authorization.

-	Threatened Asset: GVNP applications, sufficient processing capacity

5.2.3.b.2.9 Elevation of privilege

The threats in all subclauses of clause 5.3.8 for TR 33.926[2] are generic, so they also apply to GVNP of type 1.

5.2.3.b.2.10 Summary of threats for GVNP of type 1

The threats for GVNP of type 1 can be compared to TR33.926[2] and summarized as following:

| Threat Category | Detailed threat | Comparison to TR33.926[2] |
| --- | --- | --- |
| Threats relating to 3GPP-defined interfaces | - | All threats can be applied. |
| Threats relating to ETSI-defined interfaces | - | New threats: - The threats on interface between 3GPP VNF and VNFM - The threats on interface between 3GPP VNF and virtualisation layer |
| Spoofing identity | Default Accounts | Threats can be applied with difference that access through VNC instead of physical console interface. |
|  | Weak Password Policies | Same as above. |
|  | Password peek | Same as above. |
|  | Direct Root Access | Threats can be applied. |
|  | IP Spoofing | Threats can be applied with difference that objective is VNF instead of computer. |
|  | Malware | Threats can be applied. |
|  | Eavesdropping | Threats can be applied. |
| Tampering | Software Tampering | Threats can be applied. |
|  | Ownership File Misuse | Threats can be applied. |
|  | Boot tampering for GVNP of type 1 | Different threats. See detail in clause 5.2.3.b.2.5.3. |
|  | Log Tampering | Threats can be applied. |
|  | OAM traffic Tampering | Threats can be applied. |
|  | File Write Permissions Abuse | Threats can be applied. |
|  | User Session Tampering | Threats can be applied. |
| Repudiation | Lack of User Activity Trace | Threats can be applied. |
| Information disclosure | - | All threats can be applied. |
| Denial of Service | - | Different threats. See detail in clause 5.2.3.b.2.8. |
| Elevation of privilege | - | All threats can be applied. |

****************** End of the change ******************