3GPP TSG-SA WG3 Meeting #96 	S3-192883

# Wroclaw (Poland), 26-30 August 2019				 	(revision of S3-19xxxx)

# Source:	Ericsson, Vodafone

# Title:	New WID on integrating GBA to 5GC

# Document for:	Approval

Agenda Item:	7.12

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Work item on integrating GBA to 5GC

## Acronym: GBA_5G

## Unique identifier:

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| No |  |  | X |  |  |
| Yes |  |  |  | X |  |
| Don't know | X | X |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

N/A

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 800036 | Study on authentication and key management for applications based on 3GPP credential in 5G | SA3 SID |

## 3	Justification

SA3 has studied authentication and key management for applications based on 3GPP credential in 5G in the FS_AKMA study. The study is covering also the evolution of Generic Bootstrapping Architecture (GBA) as it is stated in the SID: “The study will consider new solutions as well as evolve existing 3GPP mechanisms such as GBA and BEST.”

GBA was developed to be access agnostic in the sense that it needs only IP connectivity from the UE to Bootstrapping Server Function (BSF) and to application servers (NAFs). Currently GBA uses Diameter-based interfaces Zh between the BSF and HSS, Zn between the BSF and NAF and Zpn between the BSF and push-NAF. This has allowed GBA to be used in 3G and also in 4G core networks since the HSS in 3G and 4G supported Diameter-based interfaces. This has changed in 5G, since 5GC has introduced service-based interfaces. In order to make GBA and GBA Push compatible with 5GC and thereby enable the use of GBA and GBA Push also in 5G, there is a need to specify service-based versions of those interfaces. SA2 has already introduced service-based interfaces for the HSS for IMS and the GBA Zh is the only remaining interface to the HSS which is not yet service-based.

GBA and GBA Push were designed to support 3GPP defined services as well as applications specified outside of 3GPP. To support the former in 5G, service-based versions Zn and Zpn interfaces need to be specified.

GBA and GBA Push are not specific to IMS subscribers but were designed to support subscribers with ISIM and subscribers with USIM. To enable this also in 5GC, service-based Zh may need to be defined for both UDM and HSS.

## 4	Objective

The objectives of this work item are to:

- Specify service-based versions of Zh, Zn and Zpn interfaces which are currently Diameter-based interfaces to allow the use of GBA and GBA Push in 5GC for both USIM and ISIM subscribers.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| - | - | - | - | - |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 33.220 | CRs to accommodate the needed changes to integrate GBA to 5GC. | TSG #86 (Dec. 2019) |  |
| TS 33.222 | CRs to accommodate the needed changes to integrate GBA and GBA Push to 5GC. | TSG #86 (Dec. 2019) |  |
| TS 33.223 | CRs to accommodate the needed changes to integrate GBA Push to 5GC. | TSG #86 (Dec. 2019) |  |
| TS 33.501 | CRs to accommodate the needed changes to integrate GBA and GBA Push to 5GC. | TSG #86 (Dec. 2019) |  |

## 6	Work item Rapporteur(s)

Vlasios Tsiatsis, Ericsson, vlasios.tsiatsis@ericsson.com

## 7	Work item leadership

SA3

## 8	Aspects that involve other WGs

Stage-3 work is considered by CT1, CT3 and CT4 WGs.

## 9	Supporting Individual Members

Supporting IM name

Ericsson

Vodafone