# 3GPP TSG SA WG3 (Security) Meeting #96	S3-192948

# 26 – 30 Aug 2019, Wroclaw (PL)

# Source:	NIST, ATT, Sprint, CableLabs, Cisco

# Title:	New Solution for botnet threats caused from improper CIOT device usage

# Document for:	Approval

# Agenda Item:	7.8

# 1	Decision/action requested

Approve this contribution for a new Solution in TR 33.861.

# 2	References

[1]  https://tools.ietf.org/html/rfc8520

[2] https://www.nccoe.nist.gov/sites/default/files/library/sp1800/iot-ddos-nist-sp1800-15-preliminary-draft.pdf

[3]  Enhancing the Resilience of the Internet and Communications Ecosystem Against Botnets and Other Automated, Distributed Threats. May 22, 2018. Available at https://csrc.nist.gov/CSRC/media/Publications/white-paper/2018/05/30/enhancing-resilience-against-botnets--report-to-the-president/final/documents/eo_13800_botnet_report_-_finalv2.pdf.

[4] United States Computer Emergency Readiness Team, Alert (TA16-288A): Heightened DDoS Threat Posed by Mirai and Other Botnets, https://www.us-cert.gov/ncas/alerts/TA16-288A (last revised Oct. 17, 2017).

# 3	Rationale

Automated and distributed attacks, such as botnet-based distributed denial of service (DDoS) attacks, have been a concern since the early days of the Internet. Traditional DDoS mitigation techniques relied on excess network capacity to absorb the effects of botnets. With new botnets that capitalize on the sheer number of “Internet of Things” (IoT) devices, DDoS attacks have outstripped excess capacity, resulting in significant outages. [3]

IoT devices may be designed with limited security functionality to accommodate constraints in power and computation and minimize development cost. For example, many of the devices compromised by the Mirai botnet were shipped with hardcoded administrative passwords. [4] While there have been advances in IoT security, new techniques that allow edge networks to effectively protect locally connected IoT devices re urgently needed. By communicating the communications requirements required by a locally connected device to support intended operations, networks can detect or even block attempts to compromise devices or limit compromised devices’ participation in a botnet attack. The best source for this information is generally the manufacturer of the device.

# 4	Detailed proposal

************************************* Start of Changes *****************************************

### 6.Y	Solution #Y: Security solution for preventing Botnet Attacks from Improper CIOT Device Use

### 6.Y.1	Introduction

This solution addresses key issue #x: preventing Botnet Attacks from Improper CIOT Device Usage. This solution is proposing enabling the serving network to be aware of CIOT devices expected behaviour. This solution provides the serving network this information by utilizing IETF RFC 8520 which specifies the Manufacturers Usage Description (MUD) protocol. MUD provides a scalable and efficient solution for communicating detailed networking requirements to local network devices. This information is sufficient for network devices to create per-device white and black lists. However, the IETF did not address cellular IOT devices. This document identifies three potential options for communicating networking requirements to cellular networks using MUD. The MUD protocol is designed to enable a CIOT device to signal to the network the types of access and functionality it requires to properly operate. This information is provided from the device in the form of a MUD URL as specified in IETF RFC8520. The ability for networks to be aware and constrain the communication abilities of exploited IoT devices reduces the potential for the devices to be used in attacks—both DDoS attacks that could be launched across the internet and attacks on the IoT device’s local network that could have security consequences.

The IETF MUD RFC provides a mechanism for signalling a standardized way for a CIOT device to signal this information to the serving network. A simple example of this capability is as follows;

“A light bulb is intended to light a room. It may be remotely controlled through the network, and it may make use of a rendezvous service (which could be accessed by an application on a smart phone). What we can say about that light bulb, then, is that all other network access is unwanted. It will not contact a news service, nor speak to the refrigerator, and it has no need of a printer or other devices. It has no social networking friends. Therefore, applying an access list to it that states it will only connect to the single rendezvous service will not impede performing its function; at the same time, this will allow the network to provide the light bulb and other devices an additional layer of protection. [1]”

6.Y.2	Solution A details

The CIOT device sends the MUD URL specified in IETF RFC8520 as part of the UE network capability information element.

6.Y.2	Solution B details

The Home network delivers the MUD URL specified in IETF RFC8520 to the serving network upon successful attach of the CIOT device. The information regarding the CIOT device’s expected usage is available to the home network based on the IMSI and subscription information.

6.Y.2	Solution C details

The serving network retrieves the MUD URL specified in IETF RFC8520 based on the IMEI. The information is available to the serving network through the GSMA IMEI database. This solution requires collaboration with GSMA to include the MUD URL in the TAC table of the IMEI database.

Botnets have not been known to change the IMEI of compromised IoT devices because they only operate within the main processor and not the modem itself, therefore it makes sense to have an understanding of a devices expected network usage based on IMEI by the serving network.

6.Y.3	Evolution

Solutions utilizing the IETF RFC 8520 Manufacturers Usage Description (MUD) protocol meet the potential security requirement discussed in the key issue ‘Botnet threats caused from improper CIOT device usage’.

************************************* End of Changes ****************************************