3GPP TSG-SA3Meeting #96	S3-192617

# Wroclaw, Poland, 26th Aug 2019 - 30th Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0643 | rev | - | Current version: | 15.5.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security for registration with AMF re-allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  | Date: |  |  | 2019-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current registration procedure with AMF re-allocation is define in TS 23.502. In some cases security context can’t be successfully transferred to Target AMF, thus the procedure will fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initial AMF replaces the UE identity in the RR message to be rerouted with the new 5G-GUTI. Target AMFconsequently requests security context from Initial AMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This may lead to procedure failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************************************************start of change************************************************

6.4.X	Security for registration with AMF re-allocation

This sub-clause describes the security mechanisms for registration with AMF re-allocation. The following figure illustrates the procedure with is based on 4.2.2.2 and 4.2.2.3 in TS 23.502 [8].

Figure 6.4.X-1: Security for registration with AMF re-allocation

15. The initial AMF allocates a new 5G-GUTI.

16. The initial AMF uses the new 5G-GUTI instead of the UE ID in the full Registration Request(RR) message.

17. The initial AMF invokes integrity protect the Registration Request using the 5G NAS security context.

20. Target AMF to initial AMF: Namf_Communication_UEContextTransfer (complete Registration Request).

21.	Initial AMF to target AMF: Response to Namf_Communication_UEContextTransfer(SUPI, UE Context in AMF (as per Table 5.2.2.2.2-1of 3GPP TS 23.502)).

24.	[Conditional] initial AMF to old AMF: Namf_Communication_RegistrationCompleteNotify ().

**************************************************end of change************************************************