3GPP TSG-SA3 Meeting #99e 	S3-201099

# e-meeting, 11 – 15 May 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0061 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Handling of ICMP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS |  |  |  |  |  | Date: |  |  | 2020-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Several ICPM messages are extensively used in the reality, such as the Echo request/reply, and time exceeded used to locate the network product. Hence, it is suggested to change their option “optional” into “permitted”. Furthermore, the Timestamp and Timestamp Reply message are usually used to analyse the quality of the network. It is suggested to change their option “not permitted” into “Optional”, which means that generally it is disabled by default As defined in the RFC 4861, A pair of Router Solicitation and Router Advertisement messages is often used for the neighbor discovery. Therefore, considering its usage in 5G network, respond to the Router Solicitation should be permitted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the “Optional” to “Permitted” for the echo message. Change the “Not permitted” to “Optional” for the timestamp message. Change the “Not permitted” to “Permitted” for the Router Solicitation response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possibly misleading and too strict requirement in specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 33.926 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***	BEGIN CHANGES	***

4.2.4.1.1.2	Handling of ICMP

Requirement Name: Processing of ICMPv4 and ICMPv6 packets

Requirement Description:

Processing of ICMPv4 and ICMPv6 packets which are not required for operation shall be disabled on the network product. In particular, there are certain types of ICMP4 and ICMPv6 that are not used in most networks, but represent a risk.

ICMP message types which on receipt lead to responses or to configuration changes are not mentioned in this requirement, but they may be necessary to support relevant and specified networking features. Those must be documented.

Certain ICMP types are generally permitted and do not need to be specifically documented. Those are marked as "Permitted" in below table.

The network product shall not send certain ICMP types by default, but it may support the option to enable utilization of these types (e.g. for debugging). This is marked as "Optional" in below table.

| Type (IPv4) | Type (IPv6) | Description | Send | Respond to |
| --- | --- | --- | --- | --- |
| 0 | 128 | Echo Reply | Permitted (i.e. as automatic reply to "Echo Request") | N/A |
| 3 | 1 | Destination Unreachable | Permitted | N/A |
| 8 | 129 | Echo Request | Permitted | Permitted |
| 11 | 3 | Time Exceeded | Permitted | N/A |
| 12 | 4 | Parameter Problem | Permitted | N/A |
| 13 | N/A | Timestamp | N/A | Optional |
| 14 | N/A | Timestamp Reply | Optional (i.e. as automatic reply to "Timestamp") | N/A |
| N/A | 2 | Packet Too Big | Permitted | N/A |
| N/A | 135 | Neigbor Solicitation | Permitted | Permitted |
| N/A | 136 | Neighbor Advertisement | Permitted | N/A |

The network product shall not respond to, or process (i.e. do changes to configuration), under any circumstances certain ICMP message types as marked in below table.

| Type (IPv4) | Type (IPv6) | Description | Send | Respond to | Process (i.e. do changes to configuration) |
| --- | --- | --- | --- | --- | --- |
| 5 | 137 | Redirect | N/A | N/A | Not Permitted |
| N/A | 133 | Router Solicitation | N/A | Permitted | Not Permitted |
| N/A | 134 | Router Advertisement | N/A | N/A | Not Permitted |

Test Case:

The test for this requirement can be carried out using a suitable tool or manually by performing the steps described below. If a tool is used then the tester needs to provide evidence, e.g. by referring to the documentation of the tool, that the tool actually provides functionality equivalent to the steps described below.

Test Name: TC_HANDLING_OF_ICMP

Purpose:

To verify that the network product does not process the following ICMPv4 and ICMPv6 types:

-	"Redirect (5)"

-	Router Solicitation

-	Router Advertisement

Procedure and execution steps:

Pre-Conditions:

-	The tester knows whether the network product supports IPv4 and/or IPv6:

-	If applicable, the tester has the needed system privileges for confirming that the ICMP messages with types "Not Permitted" to process are indeed not leading to configuration changes..

-	If applicable, the tester has the needed system privileges for confirming that certain ICMP message types are dropped by the network product on receipt.

-	A tester machine is available and equipped with a suitable ICMP packets generator tool.

Execution Steps

The following needs to be done for all IP protocol versions (IPv4 and/or IPv6) supported by the network element.

For verifying that the network product does not reply to ICMP messages with types where this is not permitted: The tester sends samples of the applicable ICMP messages from the tester machine to the network product and verifies by appropriate means that

-	the messages are dropped on receipt by the network product (e.g. by means of appropriate firewall rules),

-	or no response is sent out towards the test machine,

-	or there are other means ensuring that the ICMP messages cannot trigger a response.

For verifying that the network product does not change its configuration due to receiving ICMP messages with types where this is not permitted: The tester sends samples of the applicable ICMP messages from the tester machine to the network product and verifies by appropriate means that

-	the messages are dropped on receipt by the network product (e.g. by means of appropriate firewall rules),

-	or the network product's applicable system configuration remains unchanged upon receipt of the messages,

-	or there are other means ensuring that the ICMP messages cannot lead to configuration changes.

The tester utilizes appropriate means to verify consistency between the documentation in regard to ICMP and the network product.

Expected Results:

The ICMP messages which are "Not Permitted" to generate a response from the network product do not generate a response.

The ICMP messages which are "Not Permitted" to change the configuration of the network element do not change the configuration.

ICMP message types which lead to responses or to configuration changes on receipt, if neither mentioned in the requirement nor in the documentation, are not enabled.

Expected format of evidence:

The following information needs to be retained and included into the report as appropriate:

-	Tools used and their configuration

-	Tool output

-	Test result (Passed or not)

**	END OF CHANGES	***