3GPP TSG-SA3 Meeting #99e 	S3-201227

# e-meeting, 11 -15 May 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.517 | CR | 0004 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test Case on Confidentiality of the Attributes not in Data-Type Encryption Policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G |  |  |  |  |  | Date: |  |  | 01-05-2020 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the updated threat analysis in TR 33.926 G.2.4.2 proposed in S3-201228, a new test case is proposed for Confidentiality of the Attributes not in Data-Type Encryption Policy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new clause for the proposed test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of test case to fully address the threat in TR 33.926 clause G.2.4.2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 4.2.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 33.926 CR 0035 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the Change ****************

##### 4.2.2.x	Confidentiality of the Attributes not in Data-Type Encryption Policy

Requirement Name: Confidentiality of the Attributes not in Data-Type Encryption Policy

Requirement Reference: TS 33.501 [3], clause 5.9.3.3.

Requirement Description:

"Confidentiality protection shall be applied to all attributes specified in SEPP’s Data-type Encryption Policy (clause 13.2.3.2). The following attributes shall be confidentiality protected when being sent over the N32 interface, irrespective of the Data-type Encryption Policy:

-	Authentication Vectors

-	Cryptographic material

-	Location data, e.g. Cell ID and Physical Cell ID

The following attributes should additionally be confidentiality protected when being sent over the N32 interface:

-	SUPI".

as specified in TS 33.501 [3], clause 5.9.3.3.

Threat References: TR 33.926 [4], clause G.2.4.2, Exposure of confidential IEs in N32-f message

Test Case:

Test Name: TC_ENC_ATTRIBUTE_NO_POLICY

Purpose:

Verify that the SEPP under test encrypts the attributes whose confidentiality is required even if the attributes are not defined in the Data-type Encryption Policy.

with the value "NULL" and creates JSON patches containing the respective encrypted values in the dataToIntegrityProtectAndCipher object.

Procedure and execution steps:

Pre-Conditions:

-	System documentation of the SEPP under test, which details how raw public keys/certificates of peer SEPPs are to be configured and how internal log files can be accessed.

-	A second SEPP instance for N32 communication with the SEPP under test, which allows for the creation of custom N32-f messages. This system may be simulated.

-	Both SEPPs are to be configured with a raw public key/certificate of their communication peer to be able to establish a N32-c connection.

-	A Data-type encryption policy which excludes at least one information element defined in TS 33.501 [3] clause 5.9.3.3 requiring encryption on N32-f, e.g. location data of the UE. The tester configured this policy on the SEPP under test for sending.

Execution Steps

1.	Both SEPPs establish a mutual N32-c connection.

3.	Via the PLMN-internal interface, the tester provides the SEPP under test with a message to be forwarded to the peer SEPP on N32. This message needs to contain at least one information element requiring encryption that is not included in the Data-type Encryption Policy, e.g. location data of the UE.

4.	The tester captures the related N32-f message after transformation by the SEPP under test.

Expected Results:

Information elements in the original message that require encryption (e.g. location data of the UE) are replaced with the value "encBlockIdx" and appropriate JSON patches containing the respective encrypted values (e.g. location data of the UE) are created in the dataToIntegrityProtectAndCipher object.

Expected format of evidence:

Evidence suitable for the interface, e.g. text representation of the captured N32-f message.

*************** End of the Change ****************