3GPP TSG-SA WG3 Meeting #99e 	S3-200988

# E-meeting, 11-15 May 2020

Title:	Draft reply LS to RAN2 on reply LS to SA3 on FBS detection(R2-1914224/S3-200944)

Release:                     R16

Work Item:	FS_5GFBS

Source:	SA3

To:	RAN2

Cc:	RAN3

Contact Person:

#### Name:	Ivy Guo

E-mail Address:	ivy_guo@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA3 would like to thank RAN2 for the LS received in S3-200944 (R2-1914224). SA3’s answers to the questions are as below:

Q1: How frequently the network will request UE to report the hashes of system information which will bring impact on UE power consumption?

A1: The frequency that network request UE to report the hashed of SIB is based on the provision. This solution is not mandating this detail implementation. Please see the agreed companion agreed updated solution in S3-194688.

Q2: For the hash of MIB/SIBs, does SA3 assume this is only for MIB and SIB1, or also for other SIBs? Does SA3 assume this mechanism applies for both idle/inactive mode and connected mode?

A2: The MIB and SIB1 from both camped and neighbouring cells are the minimum set that need to be protected. It is better to protect other SIBs (for example the SIB3/4/5 for mobility IE protection and SIB9 for time information protection) from security perspective.

The basic idea of this solution is to enhance the current MR procedure, thus SA3 assumes this mechanism is supported for all RRC states as the same as the current MR procedure. SA3 is open to RAN2’s recommendations from the network optimisation perspective, ie., if RAN2 reckons that UE measuring in all RRC states would cause a lot network resource, then this solution could be adjusted.

Q3: For the additional information to be included in the logged MDT results, what additional information is required by SA3, compared with the existing parameters defined in NR measurement reporting (including SON/MDT)?

A3: At least the following info should be included in the logged MDT results, as the clause 6.4.2.1 in TR33.809 indicates:

- Reject info: NAS reject messages UE received in the last visited cell, e.g. registration/service reject;

- Signal info: ”: a) signal power (ex. Pilot/RS); b) pilot power; c) power for pilot/RS + Signal

SA3 is open to RAN2’s recommendations on whether it is feasible to add those information in the logged MDT results.

2. Actions:

ACTION: SA3 kindly asks RAN2 to take the SA3’s above reply into consideration and give the feedback on whether it is feasible from the RAN perspective.

3. Date of Next TSG-SA WG3 Meetings:

SA3#100e		   06 - 10 July 2020				e-meeting

SA3#100bis                      17 – 21 August 2020                                           US